3GPP TSG-RAN WG4 Meeting #114bis	R4-2503737

Wuhan, China, 7th – 11st April 2025

# Source: 	ZTE Corporation

# Title: 	Discussion on power domain enhancements for single carrier

# Agenda item:	7.1.1.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on power domain enhancements for single carrier.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

  - 2.1 Applicable requirements

A-MPR

- Issue 1-1-1: Boundary to apply SE - Options - Option 1: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW - Option 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz - WF - option 1, to meet the ITU recommendation - Issue 1-1-2: Integral region basis for OOBE - Options - Option 1: The OOBE requirements are based on UE CBW - Option 2: The OOBE requirements are based on extended UE CBW OP 1. OP 2. - WF - option 1 - Issue 1-1-3: A-MPR handling - Proposals - Option 1: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 2: - For a given additional emissions requirement and corresponding A-MPR table, if it can be confirmed that A-MPR is specified for all RB allocations X for which A−MPR(X)≥minX∈SMPR(X) where S is the set of all possible RB allocations for the given carrier, then the UE performs max(MPRenh(X), A-MPR(X)) - Option 3: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation. - Others - WF - FFS, considering that additional emission in most cases are applied on one side of the UE CBW

The above is the agreement and way forward we reached in last meeting [2]. For A-MPR handling, option 3 is more preferred. A-MPR is defined to meet the additional emission requirements, which is for a particular cell to comply with a specific regulatory or deployment scenario. Consequently, these additional emission requirements are generally stringent and not easily relaxed. Moreover, A-MPR is often applied to the RB allocations on the side of UE CBW, where additional emission requirement is more critical. Therefore, power domain enhancement scheme can be effectively implemented on RB allocations that are not constrained by A-MPR.

Proposal 1: It is proposed to only apply the power domain enhancement scheme to channels which are not subject to A-MPR.

In-band emissions

- Issue 1-1-4: How to derive the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified with ΔRB =1 or ΔRB =0. FFS the specific offset value. - IBE is not used in the GB - Option 2: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions (Moderator: NRB,V and LCRB,V could be replaced by other terms, but the concept is similar) max−25−10⋅log10(NRB,V/LCRB,V), 20⋅log10EVM−3−5⋅(ΔRB−1)/LCRB,V, −57dBm+10log10SCS/15kHz−PRB - Option 3: Others - WF - FFS Extended IBE can be used in the original GB of UE CBW - FFS the extended IBE in the GB of extended CBW - Issue 1-1-5: Modulation order for the extended IBE - Proposals - For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. Table. Comparison of emission level @ the first 1MHz adjacent to the UE CBW UE CBW (MHz). 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. SEM (dBm/30kHz); -18.2; -20.0; -21.2; -22.2; -23.0; -23.6; -24.2; -24.8; -24.0. IBE QPSK 26dBm (dBm/30kHz); -16.7; -18.7; -20.0; -21.0; -21.8; -22.5; -23.2; -23.7; -24.1; -25.0; -25.7; -26.3; -26.8; -27.2. IBE 16QAM 26dBm (dBm/30kHz); -19.6; -21.6; -22.9; -24.0; -24.8; -25.5; -26.1; -26.6; -27.1; -28.0; -28.6; -29.2; -29.7; -30.2. IBE BPSK 26dBm (dBm/30kHz); -12.0; -14.0; -15.3; -16.4; -17.2; -17.8; -18.5; -19.0; -19.5; -20.3; -21.0; -21.6; -22.1; -22.6. - WF - FFS whether the extended IBE with high order EVM could have impact on MPR for new inner RB allocation

In terms of how to derive the extended IBE, in last meeting, the observation in [3] shows IBE requirements for full RB allocation are more stringent than SEM when modulation order is QPSK and 16QAM. If we use option 1 to derive the extended IBE, the extended IBE requirement between the edge of the configured UE CBW and the edge of the virtual CBW will be more stringent than the original SEM requirement for QPSK or 16QAM when UE CBW is full RB allocation. Consequently, we can not achieve the goal of converting full RB allocation in UE configured CBW to inner RB allocation with an extended UE CBW. Compare with fixing the EVM parameter to the one for BPSK in general part of IBE requirement in option 1, option 2 is more preferred. Another advantage of option 2 is facilitating to test, and the extended IBE can be tested using the same method as the original.

- Proposal 2: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions

max−25−10⋅log10(NRB,V/LCRB,V),
20⋅log10EVM−3−5⋅(ΔRB−1)/LCRB,V,
−57dBm+10log10SCS/15kHz−PRB

  - 2.2 Approaches to enable MPR reduction for both scenario 1 and scenario2

- Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed 0.5* UE CBW is considered - Symmetrical extension is 0.5 * UE CBW on each side. - Asymmetric extension is 0.5 * UE CBW on one side. - Option 2: ≤ 0.5* UE CBW, namely flexible size at each side pending on deployment scenario, and offset of the edges between UE CBW and BS CBW, etc. - Option 3: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion. - Option 4: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should always result into a total number of RBs (“original UE channel RBs” + “RB extension”) being an element of Table 5.3.2-1 from TS 38.101-1. - Option 5: Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth - WF - FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible - Issue 1-2-2: How to get the width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed value, i.e., 0.5* UE CBW is stipulated in the spec - Option 2: Flexible value signalled by the NW, the extended CBW at each side ≤ 0.5* UE CBW but not necessarily the one in Table 5.3.2-1 in TS 38.101-1 - Option 2a: Flexible value signalled by the NW, the extended CBW at each side should be the CBW specified in Table 5.3.2-1 in TS 38.101-1 - Option 3: Based on SIB1 cell specific CBW - WF - FFS in next meeting - Issue 1-2-3: Guard band for extended CBW - Options - Option 1: if extended CBW at each side is from CBW in Table 5.3.2-1 in TS 38.101-1, the RB number is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, GB is determined accordingly - Option 2: guard band for extended CBW is derived from the total RBs including the extended ones, which is larger than the larger guard band between: - The guard band of the CBW which is immediately lower than the bandwidth corresponding to the total number of RBs (UE RBs + extended RB). - The guard band of the CBW which is immediately higher than the bandwidth corresponding to the total number of RB (UE RBs + extended RB) - Option 3: For fixed 0.5* UE CBW case, The GB of the original CBW should be retained, and the ‘GB’ of the extension part should only be retained on the side that is far away from the original CBW. - ‘GB’ of the extension part = [(0.5* BWChannel x 1000 (kHz) – ceil(NRB/2) x SCS x 12) -SCS/2], BWChannel and NRB correspond to the original UE CBW. - Others - WF - FFS in next meeting

Regarding the width/RB numbers of extended CBW at each side of UE CBW, it is important to consider that the rationale behind the extended CBW based approach is to convert full RB allocation in UE configured CBW to inner RB allocation with an extended UE CBW. Moreover, based on the definition of inner RB allocation in TS 38.101-1, the maximum size of the extension at each side of UE CBW could be up to 0.5* UE CBW. The extended CBW could be flexible to achieve the MPR reduction as much as possible. Furthermore, the extended CBW should remain within the range of BS CBW for DL single carrier operation. In the case of DL-only intra-band contiguous CA, the extended CBW should not exceed the aggregated BS CBW.

Proposal 3: The maximum size of the extension at each side of UE CBW could be up to 0.5* UE CBW, and the extended CBW could be flexible.

Proposal 4: The extended CBW should remain within the range of BS CBW for DL single carrier operation. In the case of DL-only intra-band contiguous CA, the extended CBW should not exceed the aggregated BS CBW.

In terms of the width/RB numbers of the entire extended CBW, we propose that the extended CBW should be CBW specified in Table 5.3.2-1 in TS 38.101-1. This approach ensures the guard band at each side of the extended CBW corresponds to the values specified in Table 5.3.3-1 in TS 38.101-1.

Proposal 5: The extended CBW should be CBW specified in Table 5.3.2-1 in TS 38.101-1 and the guard band at each side of the extended CBW corresponds to the values specified in Table 5.3.3-1 in TS 38.101-1.

- Issue 1-2-4: Approaches of deriving the new inner RB region with RBs in extended CBW - Options - Option 1: Fixed value case with extended CBW - If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region. If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart_Low ≤ RBStart ≤ NRB – LCRB, and LCRB ≤ ceil(2/3 *NRB) If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart ≤ RBStart,High, and LCRB ≤ ceil(2/3 *NRB) - Option 2: Flexible value case with extended CBW at each side - Variety expressions from companies, but essentially are based on the new NRB to derive the new inner region, e.g. - N'RB=Nextended_low+NRB+Nextended_ℎigℎ - Option 3: Using cell specific CBW to derive the new inner region - WF - FFS in next meeting

For the new inner RB allocation in UE CBW, we can define it based on the extended UE CBW. From the following picture, we can observe that new inner RB allocation (yellow within the red line) in UE CBW is the intersection of inner RB allocation in the extended UE CBW and UE RB allocation, in which UE RB allocation corresponds to the configured UE CBW.

Figure 2.1-3: New inner RB allocation in UE CBW based on the extended UE CBW

For the extended UE CBW, its inner RB allocation can be expressed by the following equation:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x, NRB,extended is the maximum transmission bandwidth configuration of the extended channel bandwidth, RBStart,Low, RBStart, RBStart,High is relative to the extended UE CBW.

For the new inner RB allocation in UE CBW, RBStart should meet:

- RBshift  ≤  RBStart  ≤  NRB + RBshift

- where RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended UE CBW.

- In addition, the new inner RB allocation should be under the hypotenuse of the right triangle of UE RB allocation, which can be expressed by the following equation:

- RBStart − RBshift + LCRB ≤ NRB

- where NRB is the maximum transmission bandwidth configuration of the configured UE channel bandwidth.

- The above can be deduced to

- RBStart ≤ NRB + RBshift − LCRB

- Based on the above analysis, the new inner RB allocation in UE CBW can be defined based on the extended UE CBW as follows:

NRB,extended is the maximum transmission bandwidth configuration of the extended channel bandwidth.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x.

- Proposal 6: The new inner RB allocation in UE CBW can be defined based on the extended UE CBW as follows:

NRB,extended and NRB are the maximum transmission bandwidth configuration of the extended channel bandwidth and configured UE channel bandwidth respectively.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

- where RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended UE CBW and ceil(x) is the smallest integer greater than or equal to x.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: It is proposed to only apply the power domain enhancement scheme to channels which are not subject to A-MPR.

- Proposal 2: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions

max−25−10⋅log10(NRB,V/LCRB,V),
20⋅log10EVM−3−5⋅(ΔRB−1)/LCRB,V,
−57dBm+10log10SCS/15kHz−PRB

Proposal 3: The maximum size of the extension at each side of UE CBW could be up to 0.5* UE CBW, and the extended CBW could be flexible.

Proposal 4: The extended CBW should remain within the range of BS CBW for DL single carrier operation. In the case of DL-only intra-band contiguous CA, the extended CBW should not exceed the aggregated BS CBW.

Proposal 5: The extended CBW should be CBW specified in Table 5.3.2-1 in TS 38.101-1 and the guard band at each side of the extended CBW corresponds to the values specified in Table 5.3.3-1 in TS 38.101-1.

- Proposal 6: The new inner RB allocation in UE CBW can be defined based on the extended UE CBW as follows:

NRB,extended and NRB are the maximum transmission bandwidth configuration of the extended channel bandwidth and configured UE channel bandwidth respectively.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

- where RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended UE CBW and ceil(x) is the smallest integer greater than or equal to x.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- R4-2503013, WF on Power domain enhancement, Huawei, HiSilicon

- R4-2501832, On power domain enhancements for single carrier, Huawei, HiSilicon