# 3GPP TSG-RAN WG4 Meeting # 114bis		  R4-2503749

# Wuhan, Hubei, China, 07th – 11th April, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on SAN RF requirements for supporting new SDO band for LTE-based 5G Broadcast

# Agenda Item:	7.9.4

# Document for:	Approval

- Introduction

During RAN#107 meeting, a new Work Item [1] has been approved on specifying a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO. In this contribution, related SAN RF core requirements for 10MHz PMCH are discussed. The objective of this WI is shown as below:

Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO:

  - Specify related UE RF core requirements for 10MHz PMCH

    - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as  baseline (reuse assumption of 10 MHz UE Rx filter)

  - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements:

    - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline

    - Requirements based on Resolution 761 in WRC-19

    - Only consider QPSK and 16QAM modulation

Note: UE only needs to support 5G broadcast for this band

Note: Release independence applicability will be further discussed

- Discussion

For a SDO band SAN RF, only transmitter characteristic needs to be considered. The RF requirements are analyzed based on the system parameters in another contribution[6].

The spec impact on TS36.108 and our views on the SAN RF requirements are listed below in Table 1:

Table 1 SAN RF requirements for the new SDO band (TS 36.108)

| Rx requirement | Proposals for new SDO band |
| --- | --- |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Spec impact: To define 16.9 dB total power dynamic range for 10 MHz. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Spec impact: Add ACLR requirement for 10MHz PMCH channel bandwidth for GEO class. |
| 6.6.4 Out-of-band emissions | To check whether regional regulations should be considered for LTE-based 5G Broadcast. |
| 6.6.5 Transmitter spurious emissions | To check whether regional regulations should be considered for for LTE-based 5G Broadcast. |
| 6.7 Transmitter intermodulation | No specification impact, not applicable to SAN. |
| E.5.1 Window length | Spec impact: To reuse EVM window length for 10MHz defined in 36.104. |

Proposal1: To use the proposals in Table 1 for SAN RF requirements for the new SDO band for LTE-based 5G Broadcast operating.

- Conclusion

In this contribution, some initial views on SAN RF requirements are provided and the proposals ares made as following:

Proposal1: To use the proposals in Table 1 for SAN RF requirements for the new SDO band for LTE-based 5G Broadcast operating.

- Reference

“RP-250788, New band for 5G Broadcast Utilizing Geosynchronous Satellite.”

3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access"

3GPP TS 36.108: “Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception”.

3GPP TS 38.108: “NR; Satellite Access Node radio transmission and reception”.

R4-2503747 Discussion on band definition and system parameters for supporting new SDO band for LTE-based 5G Broadcast, ZTE, RAN4#114bis, Wuhan, China, April 2025.