# 3GPP TSG-RAN WG4 Meeting #114bis                             R4-2503766

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.17.4

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements of FR2-1 SSB based L3

measurement delay reduction for connected mode

Document for:	Discussion

- Introduction

In RAN #103 meeting, the WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- Objective of core part - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Objective of performance part - Specify RRM performance requirements and test cases for the Core part enhancements.

This is the first meeting to discuss RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode. This contribution provides initial discussion on this topic.

- Discussion

## 2.1 Rx beam sweeping factor optimization

L3 measurement delay reduction by optimizing Rx BSF is to reduce measurement delay by enabling multi-Rx simultaneous reception. It is agreed that serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) is the triggering condition to activate/deactivate multi-Rx for L3 measurement. If the serving cell’s signal quality is below the threshold, multi-Rx for L3 measurement is activated. If the serving cell’s signal quality is above the threshold, multi-Rx for L3 measurement is deactivated.

According to the core part discussion, scenario 1/2/3/4/5/7 are considered as first priority. For scenario 1/2/3/4, (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. For scenario 5/7, for UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch) and RRC re-establishment.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

The SSB processing delay/time for processing multiple beams received in a SMTC is different for different scenarios. The details are as below.

- For scenario 1/2/3/4

  - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

    - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

    - where TSSB_processing = 2 ms

- For scenario 7

  - to consider an additional processing delay (2ms) after the last SMTC periodicity

- For scenario 5

  - No need to add additional processing time in the delay requirement

It can be seen that the applied trigger condition and SSB processing delay/time are different for different scenarios. To guarantee the test coverage, it is proposed to define test cases for scenario 1/2/3/4/5/7.

Proposal 1: for Rx beam sweeping factor optimization, it is proposed to define test cases for scenario 1/2/3/4/5/7.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

It is agreed that L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), which means that test cases need to cover both non-DRX and DRX.

Proposal 2: for Rx beam sweeping factor optimization, it is proposed to define test cases for both non-DRX and DRX.

## 2.2 Enhancement on CSSF outside gap in CA/DC

According to the core part discussion on optimizing CSSF outside gap in CA/DC, there are three candidate solutions. Solution 1 is to measure one serving CC per band. Solution 2 is to change the searcher occupancy ratio of measurement among the measured CCs. Solution 3 is to support 3 searchers for CSSF. For now, it is agreed not to support solution 2 in Rel-19. And solution 1 and solution 2 are supported.

For solution 1: measure one serving CC per band, the whole solution is as below. It can be seen that new network indication is introduced. One network indication is to enable/disable this CSSF enhancement. Another network indication is to indicate which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. It is necessary to define test cases to verify whether UE follow network indication to perform corresponding measurement.

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

Proposal 3: it is proposed to define test cases for solution 1 (measure one serving CC per band).

Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

In last meeting, it is agreed to support solution 3 (perform CSSF enhancement based on 3 searchers). The detailed design of solution 3 is duplicated as below. For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing.

For legacy 2 searcher case, one searcher is dedicated for PCC measurement. For FR1, all the SCCs share the 2nd searcher. For FR2, 50% of the 2nd searcher is for FR2 SCC where neighbour cell measurement is required, and the other 50% of the 2nd searcher is for FR2 SCC where neighbour cell measurement is not required, inter-frequency MO with no measurement gap and E-UTRA inter-RAT MO with no measurement gap.

It was agreed that the 3rd searcher resource sharing is same as the second searcher sharing. With this assumption, for FR1, all the SCCs share the 2nd and 3rd searcher. For FR2, 50% of the 2nd searcher and 50% of the 3rd searcher are for FR2 SCC where neighbour cell measurement is required, and the other 50% of the 2nd searcher and the other 50% of the 3rd searcher are for FR2 SCC where neighbour cell measurement is not required, inter-frequency MO with no measurement gap and E-UTRA inter-RAT MO with no measurement gap. Based on above consideration, it is necesary to define test cases to guarantee the performance for the case of three searchers.

Proposal 4: for solution 3 (perform CSSF enhancement based on 3 searchers), it is proposed to define test cases for FR1 scenario and FR1 + FR2 scenario (PCell is FR1).

- Conclusion

This contribution provides discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode. The proposals are:

Proposal 1: for Rx beam sweeping factor optimization, it is proposed to define test cases for scenario 1/2/3/4/5/7.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

Proposal 2: for Rx beam sweeping factor optimization, it is proposed to define test cases for both non-DRX and DRX.

Proposal 3: it is proposed to define test cases for solution 1 (measure one serving CC per band).

Proposal 4: for solution 3 (perform CSSF enhancement based on 3 searchers), it is proposed to define test cases for FR1 scenario and FR1 + FR2 scenario (PCell is FR1).

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements