3GPP TSG-RAN WG4 Meeting #114bis	R4-2503782

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	China Telecom

Title:	TP to 37.887: DL interruption clarification for CA_n3-n5-n78 for 3 bands dynamic Tx switching

Agenda Item:	6.10.2

Document for:	Approval

- Background

This contribution provides a TP for TR 37.887 to finish the DL interruption clarification for CA_n3-n5-n78 for 3 bands dynamic Tx switching

- Text Proposal

<Start of Text Proposal for TR 37.887>

5.1.1	CA_n3-n5-n78

5.1.1.1	Configurations

Table 5.1.1.1-1: CA configuration for mandating no DL interruption

| Configuration | Uplink band |
| --- | --- |
| CA_n3-n5-n78 | n3, n5, n78 |

5.1.1.2	Technical analysis

For CA_n3-n78 and CA_n5-n78, no DL interruption allowed has been specified in Table 5.2A.2.1-1in 38101-1[2], which means switching from n3/n5 to n78 has no impact to n3/n5 downlink. Thus for CA_n3-n5-n78 configured by 2 or 3 UL bands, there is no DL interruption when dynamitic Tx switching is triggered for one or more of the band pairs including n3-n78 and n5-n78

5.1.1.3	Conclusion

For CA_n3-n5-n78 configured by 2 or 3 UL bands, no DL interruption is mandated when dynamitic Tx switching is triggered for one or more of the band pairs including n3-n78 and n5-n78

Table 5.1.1.3-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band | DL interruption allowed (Note 1) |
| --- | --- | --- |
| CA_n3-n5-n78 | n3, n5, n78 | No for n3-n78, n5-n78 |
| NOTE 1: Applicable when dynamic Tx switching is triggered across any 2 UL bands among the configured 2 or 3 UL bands. The DL interruption requirement is specified in clause 8.2.2.2 of 38.133 [3]. The nX-nY in the requirement of “No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching. |  |  |

<end of TP >