# 3GPP TSG-RAN WG4 Meeting #114bis                                                          R4-2503795

# Wuhan, China, 7th – 11st  April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM core requirements for less than 5MHz in NTN enhancement

Agenda item:	7.10.3.4

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN evolution [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. In the last meeting we provided our initial views on potential impacts on RRM requirements and in this meeting we will continue to study the concrete RRM requirements.

Discussion

In the last meeting , we still have one issue on NTN less than 5MHz and the wayforward can be seen as below:

Issue 1-1-1: Measurement(SSB index detection requirements) Agreements: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by [3 or 4] and [3] for intra-frequency and inter-frequency cases respectively. Notes: the final value can be confirmed in the next meeting and the corresponding value will be updated in the draftCR. Issue 1-1-2: CHO Agreements: - For CHO with L3 measurement, when Tmeasure is extended with [3] for intra-frequency case, T∆ in Tinterrupt can be extended to [3] Trs.

We have discussed several meetings on above two issues which have strong connection. Some companies deem that different UEs have different capabilities, if the second side condition is introduced, the UE with better capability would not spend too much time on measurement under a side condition which is better than legacy. For example, the legacy side condition for intra-frequency measurement is SNR=-6dB, if the improved side condition is SNR=-4dB like side condition for inter-frequency measurement, the measurement delay shall be reduced, that is, the number of SSB samples shall be reduced from 4 to 3. However, RAN4 has not defined two sets of requirements under different side conditions before, it is a little bit weird for us. The opponents mentioned that side condition is a generic condition, not only for intra-frequency measurement in this scenario, but for intra-frequency measurement in all scenarios. If the side condition is changed, that means all requirements shall be modified. This is obviously not reasonable. So we deem that the legacy side condition and legacy the number of SSB for intra-frequency and inter-frequency measurement shall be maintained.

However, we partially agree with that the measurement delay could be reduced. Some companies deem that TΔ shall be maintained as legacy which is 3Trs in CHO scenario. Actually, we find that there are no any relevant CHO requirements for TN with less than 5MHz in the current spec, that is, if considering the requirement of HO with less than 5MHz, the situation is distinct from CHO since Tmeasure is not contained for HO. Thus, we shall have a comprehensive understanding. The TΔ shall be reduced to 2*Trs due to new parameter Tmeasure for CHO.

Based on above analysis, we do think this is a compromise result for us.

Proposal 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by 4 and 3 for intra-frequency and inter-frequency cases respectively.

Proposal 2: TΔ shall be extended to 2*Trs.

Conclusion

In this paper we provided our views on RRM requirements on R18 NR NTN enhancement.

Proposal 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by 4 and 3 for intra-frequency and inter-frequency cases respectively.

Proposal 2: TΔ shall be extended to 2*Trs.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133

- R4-2502589, “WF on RRM requirements for NR_IoT_NTN_req_test_enh ”, Xiaomi.