3GPP TSG-RAN WG4 Meeting #114bis                                         R4-2503806

Wuhan, P.R.China, 07th – 11th April, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS

Agenda Item:	7.26.2.3

Document for:	Approval

Introduction

Based on last meeting discussion for Rel-19 LP-WUS, the issues to be further studied on Rx requirements other than REFSNES, ACS and ASCS are captured in WF [1].

This contribution provides further analysis on other Rx requirements including IBB/OBB, intermodulation, spurious emissions and spurious response. Based on the discussion in [2], two different LP-WUR types should be considered. This contribution primarily focuses on LP-WUR type 1, which targets ultra-low power consumption. Draft requirements for LP-WUR type 1 and type 2 UE are also attached in the Annex for further discussion. We made some update for the proposed requirements in the Annex based on contribution in [4].

Discussion

IBB, OBB and Narrow Band Blocking

Regarding IBB and OBB, the issues to be further studied in the WF including:

Issue 3-1-1: IBB and OBB assumption

Agreement:

  - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR.

    - With assumption that Rx dynamic range for LR may not be as good as MR

Issue 3-1-2: IBB and OBB requirements

Agreement:

  - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level.

    - FFS IBB and OBB requirements

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. Noted that spurious response is a measure to avoid failure for blocking test due to instinct character of some UE architectures design, e.g. IF architecture. In that sense, it is reasonable to keep the spurious response requirement for LP-WUR as well.

In the specification, the absolute interferer levels are adopted for both in-band blocking (IBB) and out-of-band blocking (OBB). IBB refers to interference that occurs within the second adjacent and next adjacent channels, while OBB occurs outside a frequency range 15 MHz below or above the UE receive band. The interference of IBB and OBB are higher than ACS. The interferers relevant to the wanted signal are illustrated in the figure below:

Figure 1: Interference of ACS, IBB and OBB

The main challenges for receiver design in presence of strong interferer are the Rx chain dynamic range as well as the filter suppression capability, which could be the channel filters to alleviate interfering by ACS or IBB, or could be the band filter/duplexer to suppress OBB.

Regarding the dynamic range, the main determining components are ADC and AGC. The more ADC bits, the larger dynamic range for the receiver. However, the ADC bits cannot be increased without limitation. The relationship between ADC bit depth and power consumption is typically inversely proportional. Higher bit-depth ADCs generally offer greater precision and resolution but tend to consume more power due to the increased complexity of circuitry required for finer quantization and digitalization. Conversely, lower bit-depth ADCs consume less power but sacrifice resolution and dynamic range.

In the context of 5G smartphones, ADCs with 12 bits or higher are commonly employed. These higher precision ADCs are favoured to accommodate the demanding requirements of 5G communication, such as higher data rates and broader spectrum ranges. While some 5G smartphones may opt for even higher bit-depth ADCs like 14 or 16 bits for enhanced performance, the general trend revolves around 12 bits.

However, there's a trade-off to consider. LP-WUS which aims to minimize power consumption in smartphones, often utilize lower bit-depth ADCs to reduce power consumption. While this approach yields power-saving benefits, it may compromise the dynamic range and overall performance of the ADC. During the SI evaluation, 4 bits, 6 bits and 8 bits are considered for LLS simulation. Apparently, if low power consumption is the main target for LP-WUR design, it may not be possible that a LP-WUR could adopt the same ADC as MR, which means the dynamic range for LP-WUR could be sacrificed to some extent.

Apart from ADC, the dynamic range of AGC is another main factor which could have impact on the possibly large interferer handled by LR. As we discussed in [2], the AGC dynamic range could be much lower compared to that of MR for sake of power consumption design.

On the other hand, due to the limited suppression capability of the LP-WUR filters, it may not guarantee the same level of resistance against IBB and OBB interference as the main receiver (MR).

Observation 1: LP-WUR may have less dynamic range and capability to resist against strong blocking interference as MR could do owing to the tradeoff for the implementation of low power consumption at least for the LP-WUR type which targets ultra-low power consumption.

From requirements perspective, if the dynamic range and filter suppression capability for LR is not as good as MR, which end should be compromised for specifying the requirement? We think that better REFSENS to guarantee the coverage should have higher priority, as the NF is already expected much worse than MR. In this sense, probably the interference level should be relaxed to make it feasible for LR implementation. It is noted that the in-band blocking is from the non-coordinated NW and out-of-band blocking may come from other systems, if MR experiences such strong interference, LR will also be affected by these strong interferences. In such scenario, whether LR can work well in presence of strong interference may need further consideration.

Proposal 1: In order to guarantee the coverage of LP-WUR in blocking scenarios, the interference levels for IBB could be relaxed compared to the values defined for MR at least for the LP-WUR type which targets ultra-low power consumption. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

Similar to ACS, different cases are considered for IBB, typically Case 1 and Case 2 for NR operating bands. The power difference between Case 2 and Case 1 is 12 dB, reflecting filtering effects with further frequency offsets and the dynamic range of the receiver. However, due to phase noise impact, even with additional filtering, Case 2's interferer level cannot maintain a 12 dB difference compared to MR. Therefore, either there may be no need to specify the Case 2 IBB requirement, or a relaxed interferer level should be defined for LR IBB.

Table 1: In-band blocking parameters

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 6 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 2: In-band blocking or LP-WUR type 1 UE

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | [-63] | [-57] |
| Finterferer (offset) | MHz | – FIoffset, case 1 and + FIoffset, case 1 | – FIoffset, case 2 and + FIoffset, case 2 |

Alternatively, if keeping -56dBm and -44dBm interferer level unchanged as MR for case 1 and case 2 respectively, we could accept REFSENS + 18 dB for case 1 and REFSENS + 12 dB for case 2.

In case the LR encounters larger in-band blocking interferer which could not handle, MR should take over the monitoring of the PO or PDCCH no matter in in IDLE/INACTIVE mode or CONNECTED mode. It is worth noting that blocking is just a statistical event, the LR may not always experience the blocking scenarios, which is just a best effort target for LP-WUR design comparted to the low power consumption purpose.

Regarding OBB, with suppression of the out-of-band interferer by band filter and the analog filter before ADC, the blocking could be suppressed to relatively low level even with less dynamic range of LR, unless band filter is not considered for better coverage with less insertion loss for LR design. According to our evaluation, at least 6dB relaxation should be considered for OBB, either to reduce the interferer level or to increase the wanted signal level. Alternatively, keeping the same Pinterferer as MR, the REFSENS should be relaxed as REFSENS + 18dB and REFSENS + 12dB for type 1 and type 2 respectively.

In addition to the REFSENS degradation level and the interferer level, the allowed exceptions for spurious responses should also be taken into account for LP-WUR. Due to the constraints of low-cost implementation, the ability to restrict harmonics may be inferior compared to MR. Consequently, the number of exceptions may need to be increased to accommodate this limitation.

Proposal 2: 6dB relaxation of the interferer level is proposed for ultra-low power consumption LP-WUR. Additionally, the number of exceptions for spurious response may need to be increased to account for limitations arising from low-cost implementation constraints. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

As for Narrow Band Blocking, it was initially defined to address coexistence scenarios with GSM at a 200 kHz frequency offset, inherited from LTE for NR. We also apply this requirement to LP-WUS. Given the close offset to the desired signal, NBB can significantly degrade REFSENS, particularly if the phase noise of the interferer is poor. With a 16 dB degradation in REFSENS, the power level should align with that of IBB case 1. Therefore, we propose the following for NBB.

Table 3: Narrow Band Blocking or LP-WUR type 1 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 16 |
| Puw (CW) | dBm | [-62] |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

If keeping the same Puw (CW) level as MR, we could also accept relaxed sensitivity level as REFSENS + 34 dB and REFSENS + 22 dB for type 1 and type 2 UE respectively.

Proposal 3: Due to proximity of the NBB to LP-WUS signal and worse phase noise for LR, relaxation of the interferer level should be considered at least for the LP-WUR type aimed at ultra-low power consumption. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

Intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

This requirement is designed to verify the linearity of the Rx chain. Similar to the analysis of blocking requirement, the implementation of LR, specifically the linearity of the receiver may not as good as that for MR, thus, if the interferer level of blocking is relaxed, similar relaxation should be considered for intermodulation as well.

Proposal 4: If the blocking interferer is relaxed to accommodate the low power design for LP-WUR type which targets ultra-low power consumption, intermodulation requirement could also be relaxed for LP-WUR. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

Table 4: Wide band intermodulation parameters for LP-WUR type 1 UE

| Rx parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | [-59] |
| PInterferer 2 (Modulated) | dBm | [-59] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier with channel bandwidth ≥ 5 MHz. |  |  |

If keeping the same PInterferer levels as MR, we could also accept relaxed sensitivity level as REFSENS + 24 dB and REFSENS + 12 dB for type 1 and type 2 UE respectively.

Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. In current specification, the power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table below:

Table 4: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1 GHz | 100 kHz | -57 dBm |  |
| 1 GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 2 |
| 12.75 GHz – 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 2.69 GHz. NOTE 3: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. |  |  |  |

For receiver, the emissions at the antenna connector usually come from the reverse LO leakage. Since the emissions could be considered as kind of regulatory requirements, the same levels for MR should also be defined for LR.

Proposal 5: The same level of receiver spurious emissions for MR shall be defined for LR as well regardless of the LP-WUR types.

Spurious response

The source of generating spurious response includes mirror frequency interference, intermediate frequency interference, high-order mixing products of mixers, and other Nyquist zone aliasing of ADCs that happen to fall within the receiving band. These interferences cannot be processed by the receiver, thus causing a decrease in sensitivity. The only way to address this is by selecting appropriate intermediate frequency points, ADC sampling frequencies, and mixers with high-order suppression to reduce the number of spurious response points.

Spurious response reflects situations where the UE receiver architecture generates response points that cannot be adequately processed by the receiver. Blocking requirements cannot pass the test on these scattered points, therefore, spurious response can serve as a remedial measure for blocking tests.

When encountering spurious response, the level of the interfering signal is uniformly relaxed to -44dBm, while the wanted signal remains the same as that for out-of-band blocking.

Proposal 6: spurious response as a remedial measure for blocking tests needs to be considered for LP-WUR, and it should be relaxed at least for the LP-WUR type which targets ultra-low power consumption.

Similar to that of IBB, if the interferer level is relaxed several dBs, the spurious response should be relaxed as well.

Table 5: Spurious response parameters for LP-WUR

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | PREFSENS + 6dB |

Table 6: Spurious response for LP-WUR type 1 UE

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [-57] |
| FInterferer | MHz | Spurious response frequencies |

Conclusion

This contribution provides further consideration on IBB/OBB, intermodulation, spurious emissions and spurious response requirements for LP-WUR.

Observation 1: LP-WUR may have less dynamic range and capability to resist against strong blocking interference as MR could do owing to the tradeoff for the implementation of low power consumption at least for the LP-WUR type which targets ultra-low power consumption.

Proposal 1: In order to guarantee the coverage of LP-WUR in blocking scenarios, the interference levels for IBB could be relaxed compared to the values defined for MR at least for the LP-WUR type which targets ultra-low power consumption. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

Proposal 2: 6dB relaxation of the interferer level is proposed for ultra-low power consumption LP-WUR. Additionally, the number of exceptions for spurious response may need to be increased to account for limitations arising from low-cost implementation constraints. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

Proposal 3: Due to proximity of the NBB to LP-WUS signal and worse phase noise for LR, relaxation of the interferer level should be considered at least for the LP-WUR type aimed at ultra-low power consumption. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

Proposal 4: If the blocking interferer is relaxed to accommodate the low power design for LP-WUR type which targets ultra-low power consumption, intermodulation requirement could also be relaxed for LP-WUR. Alternatively, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR.

Proposal 5: The same level of receiver spurious emissions for MR shall be defined for LR as well regardless of the LP-WUR types.

Proposal 6: spurious response as a remedial measure for blocking tests needs to be considered for LP-WUR, and it should be relaxed at least for the LP-WUR type which targets ultra-low power consumption.

To facilitate the discussion, draft requirements for IBB/NBB/OBB, spurious response, intermodulation and spurious emissions for LP-WUR are presented in Annex for reference. The requirements are separated for two LP-WUR types and the definitions and discussion of LP-WUR types can be found in [3]. The suffix 'M' is tentatively used here for LP-WUR and may be revised if conflicts arise with the introduction of other features.

- References

[1] R4-2503028, WF on UE RF requirements for LP-WUS, vivo

[2] R4-2417886, Further consideration on UE RF REFSENS, ACS, ASCS requirements for Rel-19 LP-WUS, Huawei, HiSilicon

[3] R4-2417885, Further consideration on general aspects of LP-WUS, Huawei, HiSilicon

[4] R4-2417887, Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS, Huawei, HiSilicon

- Annex (draft requirements for IBB/NBB/OBB and others)

Editor’s note: the draft requirements are for information, which are subjected to further revisions according to the standard progress.

- 7.6M	Blocking characteristics for LP-WUR

- 7.6M.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

- 7.6M.2	In-band blocking for LP-WUR

The UE shall fulfil the minimum requirements specified in Table7.6M.2-1 for LP-WUR. The test parameters are chosen as in Table 7.6M.2-2 and Table 7.6M.2-3 for verification of the requirements specified in Table 7.6M.2-1 for LP-WUR type 1 UE and LP-WUR type 2 UE respectively. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x.

Table 7.6M.2-1: In-band blocking parameters for LP-WUR UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 6 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.6M.2-2: In-band blocking for LP-WUR type 1 UE

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | [-63] | [-57] |
| Finterferer (offset) | MHz | – FIoffset, case 1 and + FIoffset, case 1 | – FIoffset, case 2 and + FIoffset, case 2 |

Table 7.6M.2-3: In-band blocking for LP-WUR type 2 UE

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | -[56] | [-44] |
| Finterferer (offset) | MHz | – FIoffset, case 1 and + FIoffset, case 1 | – FIoffset, case 2 and + FIoffset, case 2 |

- 7.6M.3	Out-of-band blocking for LP-WUR

The out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters specified in Table 7.6M.3-1 and Table 7.6M.3-2.

Table 7.6M.3-1: Out-of-band blocking parameters LP-WUR

| LP-WUS bandwidth (RBs) | Power in LP-WUS bandwidth [dBm] |
| --- | --- |
| 11 | REFSENS + 6.0 dB |

Table 7.6M.3-2: Out of-band blocking for LP-WUR type 1 UE

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -50 | -36 | -21 |
| Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

Table 7.6M.3-3: Out of-band blocking for LP-WUR type 2 UE

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -44 | -30 | -15 |
| Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

[For interferer frequencies across ranges 1, 2 and 3 in Table 7.6M.3-2, a maximum of

max24,6⋅n⋅NRB/6minn⋅NRB/10,5

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of min(BWchannel/2,5) MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration of NR signal, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2 for SCS = 15, 30 kHz, respectively. For these exceptions, the requirements in clause 7.7M apply.]

- 7.6M.4	Narrow band blocking for LP-WUR

This requirement is measure of a receiver's ability to receive a LP-WUS signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters specified in Table 7.6M.4-1 and Table 7.6M.4-2 for LP-WUR type 1 and type 2 UE respectively.

Table 7.6M.4-1: Narrow Band Blocking for LP-WUR Type 1 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 22 |
| Puw (CW) | dBm | [-62] |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.6M.4-2: Narrow Band Blocking for LP-WUR Type 2 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + [22] |
| Puw (CW) | dBm | -55 |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

- 7.7M	Spurious response for LP-WUR

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.6M.3 is not met.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters specified in Table 7.7M-2 and Table 7.7M-3 for LP-WUR type 1 and type 2 UE respectively.

Table 7.7M-1: Spurious response parameters for LP-WUR

| RX parameter | Units | LP-WUS channel bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS channel bandwidth | dBm | PREFSENS + 6dB |

Table 7.7M-2: Spurious response for LP-WUR type 1 UE

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [-57] |
| FInterferer | MHz | Spurious response frequencies |

Table 7.7M-3: Spurious response for LP-WUR type 2 UE

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [-44] |
| FInterferer | MHz | Spurious response frequencies |

- 7.8M	Intermodulation characteristics for LP-WUR

- 7.8M.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

- 7.8M.2	Wide band Intermodulation for LP-WUR

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters specified in Table 7.8M.2-1 and Table 7.8M.2-2 for LP-WUR type 1 and type 2 UE respectively.

Table 7.8M.2-1: Wide band intermodulation parameters for LP-WUR type 1 UE

| Rx parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | [-59] |
| PInterferer 2 (Modulated) | dBm | [-59] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier with channel bandwidth ≥ 5 MHz. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

Table 7.8M.2-2: Wide band intermodulation parameters for LP-WUR type 2 UE

| Rx parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + [12] dB |
| PInterferer 1 (CW) | dBm | [-46] |
| PInterferer 2 (Modulated) | dBm | [-46] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier with channel bandwidth ≥ 5 MHz. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

- 7.9M	Spurious emissions for LP-WUR

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9M-1

Table 7.9M-1: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1 GHz | 100 kHz | -57 dBm |  |
| 1 GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 2 |
| 12.75 GHz – 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 2.69 GHz. NOTE 3: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. |  |  |  |