3GPP TSG-RAN WG4 Meeting # 114bis 	R4-2503813

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion on the system parameter of AIoT

Agenda Item:	7.24.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on the system parameter design for AIoT.

- Discussion

  - System parameter for R2D

- R2D Channel bandwidth

In [1], 1/2/3/4 RB is agreed as starting point for R2D transmission bandwidth.

- Define R2D transmission bandwidth configuration:

  - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS

  - FFS on other RB numbers

- Further discuss the R2D channel bandwidth considering additional guard band.

- If RAN1 has the agreement different than above, RAN4 will revisit the agreements

For R2D, Bocc,R2D is not specified in RAN1.

•	This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed.

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

•	For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

•	For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

•	Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

In RAN1#120, the following agreement is achieved:

It seems RAN1 will not make further decision on R2D channel bandwidth and leave the details to RAN4. For the maximum PRB for R2D transmission bandwidth, from RAN4 perspective it is also not really necessary to set an upper bound of AIoT system bandwidth, we prefer skip this discussion in RAN4 also.

The guard band need to be defined. One possible reference is NB-IoT and the transmission bandwidth is 180kHz while the channel bandwidth is 200kHz, which means the 90% spectrum utilization is feasible for OFDM based signal and for simplicity we can consider same SU is applied to all RB number above.

Proposal 1: Consider same spectrum utilization of NB-IoT is applied to R2D, i.e., the channel bandwidth for 1/2/3/4RB could be 200/400/600/800 kHz.

- R2D Channel raster

In band n8, the NR channel raster is 100kHz and recall that in previous release, the enhanced channel raster with 10kHz granularity was introduced due to the issue illustrated in Figure I:

Figure I Motivation of enhanced channel raster

Since the in-band operation also need to be supported, and similar issue may also happen if the RB number of BS CBW is even/odd but the AIoT RB number is odd/even.

Observation 1: Similar problem in enhance channel raster WI may happen for AIoT in-band operation if only 100kHz channel raster is defined for R2D in band n8.

Proposal 2: The enhanced channel raster is used for R2D, i.e., 10kHz.

2.2 System parameter for D2R

- D2R Channel bandwidth

- D2R channel bandwidth for A-IoT device

  - Define D2R channel bandwidth for A-IoT device considering the following aspects:

    - [1SB or] 2SB

    - D2R maximum data rate (depending on RAN1 conclusion)

    - SFO

    - FDMA, i.e., considering the different device 1 has different frequency shift

    - Reference Filter performance

    - Other aspects are not precluded

In [1], we agreed that:

In RAN1#120, RAN1 confirm that only 2SB transmission is supported. We understand that from definition perspective, both 1SB and 2SB are feasible, but the 1SB definition needs more clarification in the spec since such definition does not directly match with the real D2R spectrum, which may lead to unnecessary confusion in the spec.

Observation 2: It is feasible to define D2R with 1SB transmission way but may lead to confusion in the spec since the definition is not align with the real D2R spectrum.

Proposal 3: Confirm that the D2R channel bandwidth is defined based on 2SB transmission

- Conclusion

Observation 1: Similar problem in enhance channel raster WI may happen for AIoT in-band operation if only 100kHz channel raster is defined for R2D in band n8.

Proposal 2: The enhanced channel raster is used for R2D, i.e., 10kHz.

Observation 2: It is feasible to define D2R with 1SB transmission way but may lead to confusion in the spec since the definition is not align with the real D2R spectrum.

Proposal 3: Confirm that the D2R channel bandwidth is defined based on 2SB transmission

- References

- R4-2502859 WF on requirements for A-IoT BS and CW, Huawei, RAN4#114