3GPP TSG-RAN WG4 Meeting # 114bis 	R4-2503815

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement for device 1

Agenda Item:	7.24.2.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on the RF requirements for device1.

- Discussion

  - Tx Requirement

- Radiated power/Backscatter loss

In the last meeting, it is agreed that only the radiated requirement is specified for device, and the input CW power and other condition may need to be defined [1]:

- Only define radiated RF requirements for device 1 in Rel-19 WI. The details will be further discussed.

- Measurement configuration including input CW power and potential other conditions need to be defined

The radiated power will be changed with the incoming CW power and one possible problem is that whether the requirement need to be defined in different CW configuration. In [2], the formula of power transmission coefficient is provided:

ZA is the antenna impedance and the ZL is the load antenna. The formula shows the backscatter efficiency only depends on the mismatch between ZA and ZL. When ZA and ZL are the complex-conjugate with each other, the backscatter power can be maximized.

Observation 1: The backscatter loss depends on the mismatch between antenna impedance and load impedance which should be similar for different incoming CW power.

So, it should be enough to only define one CW power configuration and 0 dBm can be considered which is easy to know the backscatter loss after get the radiated power.

Proposal 1: Only one incoming CW power level configuration at device antenna is enough to verify the backscatter loss, and 0 dBm can be considered for simplicity.

Another issue here the metric for radiated power. In FR2, there are four types of transmission requirement: min peak EIRP, max EIRP, spherical coverage and max TRP. The max EIRP is from regulation and the max TRP from co-existence which are not necessary for device 1. For the radiated power level, the min peak EIRP can be a good choice, and the radiated power can be -6 dBm when CW is 0 dBm considering the backscatter loss assumed in SI.

Proposal 2: Use min peak EIRP to define the radiated power of device and the -6 dBm under 0 dBm CW power can be considered.

Most of the passive tag implemented with dipole antenna, which doesn’t have obvious directivity and the antenna gain is around only 2.15 dBi. It may not necessary to define a requirement for antenna pattern, e.g., spherical coverage or orientation tolerance.

Proposal 3: Whether the requirement for antenna patten is necessary or not (e.g., orientation tolerance, spherical coverage) needs more discussion.

- Transmit signal quality

There are several modulation quality requirements are under discussion, e.g., RF envelope, SFO, and one general issue is the direction for the requirement definition. In FR2, the signal quality is only defined and verified in beam peak direction, which can be also applied to device.

Proposal 4: The transmit signal quality (e.g., SFO, RF envelope) is only defined at peak EIRP direction

Since the D2R has different data rate which may depend on the small frequency shift, it is may be redundant to verify each of them and the D2R signal with maximum data rate can be considered as the worst case which require the highest MCU switch time.

Proposal 5: The transmit signal quality is verified under maximum D2R data rate.

- Unwanted emission

As we mentioned in the last meeting, In ITU-R SM328, the occupied bandwidth is applied to the device with amplifier, not for passive device

Classes of emission A1A and A1B with fluctuations

When large short-period variations of the received field are present, the specifications given below for single-channel, amplitude-modulated, continuous-wave telegraphy (Class A1A and A1B), represent the desirable performance obtainable from a transmitter with an adequate input filter and sufficiently linear amplifiers following the stage in which keying occurs

Observation 2: ITU-R has no clear occupied bandwidth limitation for passive device type.

Our concern on defining such requirement is that the OOK naturally has poor spectrum efficiency. For a periodic square wave, it can be expanded using the Fourier series

The harmonic has quite high power and it may need include 5th harmonic into D2R channel bandwidth to achieve 99% occupied bandwidth, which may lead to a wide D2R bandwidth and waste the spectrum resource.

Observation 3: To achieve 99% occupied bandwidth, at least 5th harmonic needs to be included in the D2R Channel bandwidth.

Companies may have concern on the interference to other radio system, but this issue can also be solved by defining as proper emission mask.

Proposal 6: The occupied bandwidth is not defined for AIoT device and the concern on the impact to other radio system can be addressed by emission mask.

For the emission mask, the Figure I show the spectrum for a 15kHz OOK signal with 160 kHz small frequency shift:

Figure I 15kHz OOK spectrum with 160kHz small frequency shift

Since the emission of D2R should be worst when the D2R close to the channel bandwidth, and the channel bandwidth depends on the maximum D2R data rate, so the emission mask only needs to be verified under maximum D2R data rate.

As we mentioned above, the harmonic naturally exists in OOK signal which should be considered when defining the emission mask.

Proposal 7: The emission mask of AIoT is verified under maximum D2R data rate and the hormonic need to be considered when defining the requirement.

As we mentioned in the last meeting, unwanted emission of ETSI RFID tag is only verified in the EIRP peak direction [5]which is more friendly to the low-cost device. We can also try to define the unwanted emission in a single direction.

Proposal 8: The unwanted emission is defined on the EIRP peak direction which is same as the radiated power.

  - Rx Requirement

Before dive into the specific requirement discussion, the performance metric for Rx requirement should be decided first. For LTE/NR, 5% throughput loss of reference channel is used which is not suitable for AIoT device. In LP-WUS discussion, the miss detection rate was agreed as metric for Rx requirement, which can be considered used for device.

Observation 4: For LP-WUR, the Rx requirement is defined based on miss detection rate which is also based on OOK waveform.

Proposal 9: It is suggested that the Rx requirement of device is defined based on 10% miss detection rate.

- REFSENS

Applicable maximum distance target value(s): The maximum distance targets are set separately for device 1, device 2a, device 2b, respectively. The distance target is compared to the evaluation results which assume the agreed mandatory evaluation assumptions.

For device 1 in D1T1: 15 m

For device 2a in D1T1: 25 m

For device 2b in D1T1: 50 m

For device 1 in D2T2: 10 m

For device 2a in D2T2: 15 m

For device 2b in D2T2: 40 m

The REFSNES requirement mainly intends to ensure the device can achieve sufficient coverage and the coverage design target is listed in TR38.769:

Observation 5: The REFSENS is mainly to ensure the device has sufficient coverage performance and the maximum coverage design target is 15m for device 1 in D1T1.

In [3], the sensitivity of real commercialized UHF RFID tag for sensing is listed in Table I

Table I RF sensitivity of UHF RFID with on-chip sensor

Many devices don’t have good sensitivity as we assumed in SI (-36 dBm) since they have different usage and we need to keep in mind that when we define a sensitivity requirement, actually we are drawing a bottom line of cost for all AIoT device which could be risky since the market now is not mature enough.

Observation 6: The usage of IoT device is various and the sensitivity requirement will set a bottom line of cost for all device which need to be carefully treated.

In our understanding, there are two possible ways to define this requirement. One is that 3GPP only define the performance metric (e.g., 10% MDR or BLER) and test method, the sensitivity can be based on manufacturer declaration. Another way is we only set a requirement to guarantee the coverage design target in RAN1, and the sensitivity can be calculated based on pathloss formula: R2D power (38 dBm) – 15m pathloss (51.8 dB) = -13.8 dBm.

Proposal 10: Consider following two options for REFSENS definition:

Option 1: RAN4 only define the test methods and performance metric, the REFSENS can be declared by manufacturer

Option 2: Define REFSENS for device 1 based on the 15m coverage design target in R19, e.g., -13.8 dBm.

- Conclusion

In this contribution, we further update views on AIoT device RF requirements.

Observation 1: The backscatter loss depends on the mismatch between antenna impedance and load impedance which should be similar for different incoming CW power.

Proposal 1: Only one incoming CW power level configuration at device antenna is enough to verify the backscatter loss, and 0 dBm can be considered for simplicity.

Proposal 2: Use min peak EIRP to define the radiated power of device and the -6 dBm under 0 dBm CW power can be considered.

Proposal 3: Whether the requirement for antenna patten is necessary or not (e.g., orientation tolerance, spherical coverage) needs more discussion.

Proposal 4: The transmit signal quality (e.g., SFO, RF envelope) is only defined at peak EIRP direction

Proposal 5: The transmit signal quality is verified under maximum D2R data rate.

Observation 2: ITU-R has no clear occupied bandwidth limitation for passive device type.

Observation 3: To achieve 99% occupied bandwidth, at least 5th harmonic needs to be included in the D2R Channel bandwidth.

Proposal 6: The occupied bandwidth is not defined for AIoT device and the concern on the impact to other radio system can be addressed by emission mask.

Proposal 7: The emission mask of AIoT is verified under maximum D2R data rate and the hormonic need to be considered when defining the requirement.

Proposal 8: The unwanted emission is defined on the EIRP peak direction which is same as the radiated power.

Observation 4: For LP-WUR, the Rx requirement is defined based on miss detection rate which is also based on OOK waveform.

Proposal 9: It is suggested that the Rx requirement of device is defined based on 10% miss detection rate.

Observation 5: The REFSENS is mainly to ensure the device has sufficient coverage performance and the maximum coverage design target is 15m for device 1 in D1T1.

Observation 6: The usage of IoT device is various and the sensitivity requirement will set a bottom line of cost for all device which need to be carefully treated.

Proposal 10: Consider following two options for REFSENS definition:

Option 1: RAN4 only define the test methods and performance metric, the REFSENS can be declared by manufacturer

Option 2: Define REFSENS for device 1 based on the 15m coverage design target in R19, e.g., -13.8 dBm.

- References

- R4-2502858, WF on requirements for system parameters and A-IOT device, CMCC, RAN4#114bis

- Griffin, J.D., & Durgin, G.D. (2009). Complete Link Budgets for Backscatter-Radio and RFID Systems. IEEE Antennas and Propagation Magazine, 51.

- Sharif, A., Ouyang, J., Arshad, K., Imran, M. A., & Abbasi, Q. H. (2021). Passive UHF RFID Tag Antennas‐Based Sensing for Internet of Things Paradigm. Backscattering and RF Sensing for Future Wireless Communication, 133–155.

- 3GPP TR 38.769

- ETSI EN 302 208

- ITU-R SM.328-11