3GPP TSG-RAN WG4 Meeting # 114bis 	R4-2503817

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion on the OTA test method for device 1

Agenda Item:	7.24.4

Document for:	Approval

- Introduction

In this contribution, we provide our views on the OTA test method.

- Discussion

Take following as a starting point and further discuss the details.

- The traditional test system of AC for handheld UE radiated testing can be considered as the starting point for A-IoT device testing.

- The following test setup parameters shall be defined for A-IoT device OTA testing:

  - DCW2D: distance between CW node and DUT

  - DMA2D: distance between measurement antenna and DUT

  - IsoCW2M: Isolation between CW node and measurement antenna

In [1], we agreed that:

The quiet zone is related to antenna size and for simplicity we can reuse current 30cm assumption. The Table in [2] give the minimum range length of the probe:

Table I minimum range length in [2]

| F [GHz] | Drad [m] | RQZ+2Drad²/λ | 3D = 6RQZ | RQZ+2λ | max(RQZ+2λ,3D,RQZ+2D²/λ) |
| --- | --- | --- | --- | --- | --- |
| 0.41 | 0.30 | 0.40 | 0.9 | 1.61 | 1.61 |
| 0.6 | 0.30 | 0.51 | 0.9 | 1.15 | 1.15 |
| 0.7 | 0.30 | 0.57 | 0.9 | 1.01 | 1.01 |
| 0.8 | 0.30 | 0.63 | 0.9 | 0.90 | 0.90 |
| 1 | 0.30 | 0.75 | 0.9 | 0.75 | 0.90 |
| 1.2 | 0.29 | 0.83 | 0.9 | 0.65 | 0.90 |

Considering the AIoT tag now is targeted working in band n8 which is around 900MHz, 1m could be a sufficient length between the center of quiet zone and measurement probe.

Observation 1: Based on the quiet zone with 30cm diameter, the minimum range length of probe for AIoT tag is 0.9m.

Proposal 1: Both the DCW2D and DMA2D are set to 1m

For the isolation between CW source and measurement antenna, 50 dB is used in RFID test [4], which can be considered reused for AIoT tag.

Proposal 2: The minimum ISOCW2M is 50 dB

Since the Tag need to be mounted on the test equipment, the on-object antenna gain penalty need to be considered. The object will change the impedance and reduce the antenna efficiency, and in the worst case, the tag cannot work at all.

Observation 2: The material behind the tag in the test system has significant impact on the antenna performance of tag.

Proposal 3: The device need to mount on the non-RF-reflective and non-RF-conductive material.

In [3], the direction of verification for tag is described as below:

This may relate to how the core requirement will be defined but if the core requirement is defined on the peak EIRP direction only, the default device orientation and corresponding peak direction can be declared by manufacturer which should be simpler.

Observation 3: ETSI RFID tag is only verified in the EIRP peak direction.

Proposal 4: The default device orientation and corresponding EIRP peak direction are declared by manufacturer if the core requirement is the core requirement is only defined at EIRP peak direction.

As we mentioned in the last meeting, one big different compared to legacy NR test is the CW node need to transmit the signal while measure the D2R requirement, and it is questionable whether the TE can extract the D2R POWER from CW correctly. If the TE can not easily do the cancellation, this issue need to be addressed during test procedure design.

Proposal 5: It is suggested TE vendor to feedback on whether the TE can extract D2R power when the CW source is transmitting.

One possible way to extract the D2R power is listed below:

- Measure the power P1 (D2R+ CW) when device exist,

- Remove the device and measure the power P2 (CW only)

- Calculate the D2R measurement results = P1 – P2

Proposal 6: if the TE cannot cancel the CW easily, the following procedure can be considered to extract the measurement results of D2R:

- Measure the power P1 (D2R+ CW) when device exist,

- Remove the device and measure the power P2 (CW only)

- Calculate the D2R measurement results = P1 – P2

Since the test setup for AIoT is based on 38.870 which is also recorded in WID:\

- Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

In the last meeting, many test setup from TR38.870 are confirmed to apply to AIoT. From spec organization perspective, it would be redundant to capture same content in different content

Observation 4: It would be redundant to capture exactly same content in the different TS and also make the TS become bloated.

It is better to only capture AIoT specific test setup in new TS and other duplicate content can be just reference in the corresponding Clause in the TR 38.870.

Proposal 7: Only the unique test setup for AIoT is captured in the new TS, and the duplicate content as TR38.870 is directly referenced in the corresponding Clause of the TR.

In addition, the detailed test procedure or method normally will be captured in TR, and we can create several new clauses in the TR 38.870.

Proposal 8: The outcome of detailed test method in this WI is capture in TR 38.870, and two new clause is added:

- Clause 10: testing methodologies for A-IoT

- B.6 MU assessment for A-IoT

- Conclusion

In this contribution, we provide our views on AIoT test method

Observation 1: Based on the quiet zone with 30cm diameter, the minimum range length of probe for AIoT tag is 0.9m.

Proposal 1: Both the DCW2D and DMA2D are set to 1m

Proposal 2: The minimum ISOCW2M is 50 dB

Observation 2: The material behind the tag in the test system has significant impact on the antenna performance of tag.

Proposal 3: The device need to mount on the non-RF-reflective and non-RF-conductive material.

Observation 3: ETSI RFID tag is only verified in the EIRP peak direction.

Proposal 4: The default device orientation and corresponding EIRP peak direction are declared by manufacturer if the core requirement is the core requirement is only defined at EIRP peak direction.

Proposal 5: It is suggested TE vendor to feedback on whether the TE can extract D2R power when the CW source is transmitting.

Proposal 6: if the TE cannot cancel the CW easily, the following procedure can be considered to extract the measurement results of D2R:

- Measure the power P1 (D2R+ CW) when device exist,

- Remove the device and measure the power P2 (CW only)

- Calculate the D2R measurement results = P1 – P2

Observation 4: It would be redundant to capture exactly same content in the different TS and also make the TS become bloated.

Proposal 7: Only the unique test setup for AIoT is captured in the new TS, and the duplicate content as TR38.870 is directly referenced in the corresponding Clause of the TR.

Proposal 8: The outcome of detailed test method in this WI is capture in TR 38.870, and two new clause is added:

- Clause 10: testing methodologies for A-IoT

- B.6 MU assessment for A-IoT

- References

- R4-2502294 Way Forward for [113][327] NR_FR2_OTA, Qualcomm, RAN4#114

- 3GPP TR 38.870

- ETSI EN 302 208

- GS1 EPC global, Tag Performance Parameters and Test Methods Version 1.1.3