3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503840

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	NR NTN HPUE Tx Requirements and feasibility analysis

Agenda Item:	7.10.2.2.1

Document for:	Approval

Introduction

In RAN4#114, the summary and WF in NR NTN HPUE requirements is in [1][2]. A comprehensive discussion paper for Tx is in [3]. The are some progresses, but still issues to be resolved.

In this paper, further discussion was made for NR-NTN. It is noted that the duty cycle related discussion is not included here but put to another paper for Iot-NTN Tx.

Requirements Applicability

Current condition:

In last meeting, considerable progress has been made on the requirements that would be defined, and almost all the cases have been set except a few cases. The following agreements have been copied below as following:

Issue 2-1: NR-NTN PC2

Agreement:

- Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible

  - 1Tx handheld

  - 2Tx handheld

  - 1Tx non-handheld

  - 2Tx non-handheld

- If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19

Issue 2-3: NR-NTN PC1.5

Agreement:

- Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19.

- FFS on other cases if time is allowed

Issue 2-3: NR-NTN PC1

Agreement:

- RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19.

With these agreements, the following updated requirements applicability was provided:

Table 1: Current requirements applicability in R19 (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [4Tx] | Non-handheld |
| PC1.5 | FFS if time allows |  |
|  | 2Tx | Non-handheld |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Observation 1: There are two remaining unconfirmed scenarios for requirements definition:

- cases other than 2Tx non-handheld in PC1.5

- 4Tx for PC1 non-handheld

For PC1.5, the current agreed scenario (2Tx non-handheld) was agreed after long discussion, can be regarded as a compromise considering many factors. Anything more than this scenario for PC1.5 is not that likely to be agreed in the current situation. I would propose not to consider other cases in Rel-19.

Proposal 1a: Do not consider defining requirements other than 2Tx non-handheld in Rel-19 for PC1.5.

For PC1 non-handheld, the 4Tx architecture was not discussed much before. According to [5], this implementation has the benefit of using current available 26dBm PA and related ecosystem. This argument is somewhat similar to what was described in [4] that using three separate PC2 RF chain (30.77dBm) and PC2 duplexers as another implementation. As explained before, current UE duplexer cannot handle too high power, and a single PC1 PA is far beyond what current UE duplexer can handle.

However, multiple Tx chain implementation has a lot of challenges in NTN scenario as discussed before. Even 2Tx was questioned before, and 4Tx may encounter more challenges in the potential performance gain.

Proposal 1b: 4Tx PC1 non-handheld requirements can be further discussed.

HPUE Impact for Tx Requirements

MOP

The MOP discussion is mainly focus on the tolerance. In RAN4#113, the following agreement has been reached for PC2:

Agreement:

Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE

However, no agreement has been reached for PC1.5 and PC1 yet. The following observation and proposal was re-submitted from [4]:

Theoretically, the nominal MOP for the same power class can be different for NR NTN and NR, since they are defined in separate specs. However, all the current discussion has assumed the same nominal MOP in NTN as TN, i.e. PC1/1.5/2 is 31/29/26 dBm, and there were not any different views on this point. This might be because no benefit foreseen to have different values, and reusing those basic parameters are also natural from the reusing the TN components.

Observation 2a: No need to consider different nominal MOP for the same power class in NR NTN with TN.

For the tolerance value, currently all the HPUE cases in NR NTN is +2/-3. Considering the target to reuse TN components as much as possible, this tolerance range is also proposed to be re-used.

Observation 2b: Reusing tolerance for different power class in TN for NTN is needed for basic reuse of the components.

All those requirements are not applicable NR-NTN redcap since HPUE is not yet supported in REl-19.

Based these conditions, the following proposal is provided:

Proposal 2: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1 and PC1.5 for NR-NTN Non-Redcap HPUE.

SEM

The following agreement has been reached in [2]:

Issue 2-5: SEM

Agreement:

- Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline.

- Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202

Now it is already agreed that SEM from NR-NTN PC3 would be reused for other PCs for NR-NTN as baseline. For other regulatory or regional requirements such as defined in FCC/ETSI, the current 3GPP procedure of treating in A-MPR has been work well for a long time. In addition, re-discuss the SEM in TN which had already be used for over a decade since LTE may involves considerable work.

Based on the situation, the following observation and proposal from [3] were re-submitted:

Observation 3: Reconsider NTN regulatory requirement in SEM may involve considerable work, and the current A-MPR solution is already effective and flexible to consider the various regional regulatory requirements.

Proposal 3: Re-use existing SEM from PC3 to other PCs for NR-NTN.

ACLR & MPR

In last meeting, the co-existence study was finally concluded.

Issue 1-1: Results handling with different isolation distances

Agreement:

- Agree to capture all submitted results in the TR, and to separate results with different isolation.

- Use 1.5km isolation results to draw ACLR conclusion from co-ex study perspective.

Issue 1-2: NR-NTN Co-ex study results discussions

Agreement:

- For NR-NTN PC2, it is observed that the required ACLR is less than 20 dB with 1.5km isolation distance assumed based on the simulation results.

Agreement:

- For NR-NTN PC1.5, it is observed that the required ACLR is less than 20.7 dB with 1.5km isolation distance assumed based on the simulation results.

Agreement:

- For NR-NTN PC1, it is observed that the required ACLR is less than 21.6 dB with 1.5km isolation distance assumed based on the simulation results.

From the co-existence study, it can be seen that ACLR needed from co-existence is well below the current TN ACLR requirements. Thus there was a debate one what ACLR should be applied, the agreements were as following:

Issue 2-4-1: PC2 ACLR

Agreement:

- PC2 ACLR is [30 or 31] dBc for NR-NTN UE

Issue 2-4-2: PC1.5 ACLR

Agreement:

- PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE

Issue 2-4-3: PC1 ACLR

Agreement:

- PC1 ACLR is [31 or 37] dBc for NR-NTN UE

- Further study averaging value, i.e. 34.

Based on the previous agreement, it can be seen that:

Observation 4a: Basically, there were two choices for ACLR:

- Option 1: Using lower ACLR requirements for HPUE power classes for NTN compared to TN

- Option 2: Reusing TN PC2/1.5/1 ACLR requirements for NTN PC2/1.5/1 respectively (31/31/37dBc)

In addition, the MPR agreement seems to be closely related to what ACLR assumption. And the latest agreement is:

Issue 2-6: MPR/A-MPR

Agreement:

- For PC 2 and PC 1.5 MPR:

- Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc.

- Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc.

- For PC 1 MPR:

- Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc:

  - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies

  - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec.

- Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc

- Aim for MPR/A-MPR analysis in #114bis for different power classes

- For PA model calibration:…

Here the detailed the assumptions of MPR evaluation were omitted.

It can be seen that current MPR was deemed closely related to ACLR assumption. Specifically, re-using TN ACLR for different power classes would reusing the TN MPR, while using PC3 ACLR(30dBc) for other power classes may introduce new MPR requirements. However, we have the following observation:

Observation 4b: Even using 30dBc ACLR for PC2/1.5/1 may not necessarily means new MPR requirements for NTN compared to TN.

Here are some more explanations:

For PC1.5/PC2, ACLR value of 31 and 30dBc are actually quite close, and even if the resulting MPR differs from simulation or test, the MPR difference would be marginal considering other margin and tolerance needed. For the sake of simplifying the standardization, PC2/PC1.5 may directly adopt the MPR from NR, meaning that the Pcmax_L can remain unchanged in NTN compared to TN, which is a safe option since the TN ACLR is slightly more strict and a bit more MPR is allowed than actually needed.

Even with same MPR, a bit relaxation of ACLR (from 31dBc to 30dBc) can still potentially beneficial for UE to use higher power. Even with the same component, different calibration and adjustment strategies can still be employed for TN and NTN, and slightly lowering ACLR allows for the use of higher transmission power in similar configurations (better than legacy MPR), providing more implementation freedom for the UE while still satisfy co-existence.

Observation 4c: Slightly lowering ACLR for PC2/PC1.5 may leave more room for power optimization in the future for UE implementation; MPR can continue to be reused from TN to reduce the workload of standardization.

Proposal 4-1: For PC2/PC1.5 NR NTN, using 30dBc ACLR for NR NTN, and can reusing TN MPR requirements as baseline.

For PC2, the current TN requirements do not differentiate handheld and non-handheld type for MPR for both 1Tx/2Tx. While for PC1.5, two sets of MPR requirements were defined with one assumed for large FWA form factor. Based on this condition, we can continue this way for NTN.

Proposal 4-2: For PC2 NR NTN, do not differentiate handheld/non-handheld, while using 1Tx/2Tx requirements from respective TN requirements. For PC1.5 NR NTN, reusing the requirements for large FWA formfactor for non-handheld in NTN as baseline.

For PC1, there are some special situations compared to PC2/1.5. In TN, PC1 devices never been considered for handheld UE type. The PA model / duplexer assumption is also quite different from what smartphone usually use ,and the MPR basically follows the MPR of PC3 UE. In addition, legacy TN PC1 has never been widely used and has not formed a massive ecosystem like what PC3/2 does.

In this situation, reusing current available requirements seems less meaningful from the point of reusing ecosystem and currently available components. It is also quite doubtful on whether it would be possible in the future in the TN to have widely use of this PC1. What is sure is that the 37dBc ACLR in TN PC1 is not necessary for NTN, and this strict requirements may bring unnecessary burden for future implementation.

Observation 4d: NR PC1 is currently not widely used in TN and also not that likely even in the future. Reusing requirements that are much higher than actually needed may bring unnecessary obstacles to potential implementation of PC1 for NR NTN.

For the MPR requirements, for simplicity concern, reusing PC3 requirements may still be considered as a starting point.

Based on these observations, the following proposal is provided:

Proposal 4-3: For PC1 NR-NTN, ACLR can be defined as 31dBc, and TN PC3 MPR requirements can be used as starting point.

Conclusion

In this paper, the following observations and proposals are provided:

Requirement applicability

Table 1: Current requirements applicability in R19 (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [4Tx] | Non-handheld |
| PC1.5 | FFS if time allows |  |
|  | 2Tx | Non-handheld |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Observation 1: There are two remaining unconfirmed scenarios for requirements definition:

- cases other than 2Tx non-handheld in PC1.5

- 4Tx for PC1 non-handheld

Proposal 1a: Do not consider defining requirements other than 2Tx non-handheld in Rel-19 for PC1.5.

Proposal 1b: 4Tx PC1 non-handheld requirements can be further discussed.

MOP tolerance

Observation 2a: No need to consider different nominal MOP for the same power class in NR NTN with TN.

Observation 2b: Reusing tolerance for different power class in TN for NTN is needed for basic reuse of the components.

Proposal 2: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1 and PC1.5 for NR-NTN Non-Redcap HPUE.

SEM

Observation 3: Reconsider NTN regulatory requirement in SEM may involve considerable work, and the current A-MPR solution is already effective and flexible to consider the various regional regulatory requirements.

Proposal 3: Re-use existing SEM from PC3 to other PCs for NR-NTN.

ACLR & MPR

Observation 4a: Basically, there were two choices for ACLR:

- Option 1: Using lower ACLR requirements for HPUE power classes for NTN compared to TN

- Option 2: Reusing TN PC2/1.5/1 ACLR requirements for NTN PC2/1.5/1 respectively (31/31/37dBc)

Observation 4b: Even using 30dBc ACLR for PC2/1.5/1 may not necessarily means new MPR requirements for NTN compared to TN.

Observation 4c: Slightly lowering ACLR for PC2/PC1.5 may leave more room for power optimization in the future for UE implementation; MPR can continue to be reused from TN to reduce the workload of standardization.

Proposal 4-1: For PC2/PC1.5 NR NTN, using 30dBc ACLR for NR NTN, and can reusing TN MPR requirements as baseline.

Proposal 4-2: For PC2 NR NTN, do not differentiate handheld/non-handheld, while using 1Tx/2Tx requirements from respective TN requirements. For PC1.5 NR NTN, reusing the requirements for large FWA formfactor for non-handheld in NTN as baseline.

Observation 4d: NR PC1 is currently not widely used in TN and also not that likely even in the future. Reusing requirements that are much higher than actually needed may bring unnecessary obstacles to potential implementation of PC1 for NR NTN.

Proposal 4-3: For PC1 NR-NTN, ACLR can be defined as 31dBc, and TN PC3 MPR requirements can be used as starting point.

- References

[1] R4-2500680, Topic summary for [113][128] NR_IoT_NTN_HPUE_part1, Moderator (Samsung), RAN4#114

[2] R4-2502954, WF on HPUE requirements for NR-NTN, Samsung, RAN4#114

[3] R4-2500762, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114

[4] R4-2500742, Discussion on co-existence of NTN HPUE, vivo, RAN4#114

[5]R4-2501458, Discussion on Tx requirements for NR NTN HPUE, Huawei, HiSilicon, RAN4#114