3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503845

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion of UE RF impact for LB-LB switching

Agenda Item:	7.7.2

Document for:	Approval

Introduction

In RAN#106, a new WID [1] was approved on Low band carrier aggregation via switching. A motivation paper including some pictures was provided in [2].

In RAN4#114, most discussions were made focus on switching time for low NR band carrier aggregation via switching. Some other issues were listed but not discussed. The main results are WF [4] and LS [5].

In this paper, further discussions were made.

Discussion

UE RF

Basic scenario

For the basic switching behavior studied, the following figure from [2] was also partly copied here:

It should be noted the two cases for switching can be interpreted as two states:

- State1: The UE operates in both Tx and Rx on the FDD band.

- State2: The UE operates only in Rx mode on the SDL band..

Switching period

In the WF, the following agreements reached:

-	Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations

-	35us, 140us

-	RAN4 to further discuss 13us

-	LS will capture the currently confirmed values

-	It is not precluded to send another LS with more values in future meetings

Whether 13us should be a defined as an option of switching period has raised a lot of discussion. Our understanding is that in case of typical implementation, this is a rather challenging value, even under some satisfactory condition such as minimised sharing components.

In addition, the current option of 35us already corresponds about 1 symbol duration. As a switching period, a shortened time compared to his value may also not that meaningful since anyway symbol is the shortest possible duration that could be considered in scheduling.

Based on this situation, the following proposal is made:

Proposal 1: Do not consider 13us as an option for switching period definition.

Rx testing

As discussed in [6], this “switching” is different from the legacy “Tx Switching” studied in previous releases, in that legacy “Tx Switching” is a switching between Tx chains without any involvement of the Rx functionality. In addition, the time mask requirements defined for “Tx Switching” are solely Tx operations.

In another word, the switching period currently defined as ON/OFF mask is for Tx only. For case 2 which is only Rx exist, a testability issue would rise, since the case 2 (SDL only) do not have any Tx and no ON power could be measured. Currently we do not have mature established methods to check whether Rx chain is on or not. The usual way to test receiver by means of demodulation may face challenges to do verification in the order of Micro seconds.

Although testability issue is strictly speaking may not belong to core requirements, this still need to be considered.

Proposal 2: The testability of timing of receiver ON period may need further discussion.

Capability

The definition of UE capabilities are also included in the WID scope as following:

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

Apart from the basic capability to support this feature in a per BC basis, there are also possible capabilities on different switching performance, and also others. For example, the switching period location can also be RRC configurable.

Proposal 3: The initial thinking is the capabilities can be on a per BC basis. Some basic parameters may include:

- Basic support of switching capability

- Switching delay

- Switching period location

Conclusion

In this paper, a preliminary analysis has been done for Low band carrier aggregation via switching. The following observations and proposals have been provided:

Proposal 1: Do not consider 13us as an option for switching period definition.

Proposal 2: The testability of timing of receiver ON period may need further discussion.

Proposal 3: The initial thinking is the capabilities can be on a per BC basis. Some basic parameters may include:

- Basic support of switching capability

- Switching delay

- Switching period location

- References

[1] RP-243317, New WID on low band carrier aggregation via switching, Apple, RAN#106

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, RAN#106

[3] R4-2502853, Topic summary for [114][126] NR_LBCA_Sw, Moderator(Apple), RAN4#114

[4] R4-2503016, WF on low-low band switching, Apple, RAN4#114

[5] R4-2502877, LS on switching periods for low-low band switching, AT&T, RAN4#114

[6] R4-2500768, Discussion of UE RF impact for LB-LB switching, vivo, RAN4#114