3GPP TSG-RAN WG4 Meeting # 114bis                                R4-2503849

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.17.5

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

Introduction

In the RAN #103 meeting, the WID on several enhancements for NR and MR-DC RRM requirements was approval. There are two objectives need to discuss in Rel-19, one is FR2-1 SSB based L3 measurement delay reduction for connected mode, and the other is FR2-1 SSB based L3 measurement delay reduction for connected mode. In this meeting, we should discuss performance part on fast SCell activation for UE supporting R18 EMR from this meeting. according to schedule [1].

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this contribution, we would like to provide our views on performance part.

Discussion

According the WI, R18 eEMR is considered in the existing known/unknown SCell condition during SCell configuration/activation in R19, which could reduce delay requirement for the target SCell activation.

During the discussion on core part, RAN4 has agreed that both single direct activation and MAC CE based single SCell activation are supported at SCell addition, meanwhile, MAC CE based PICCH SCell activation is included. Therefore, RAN4 should discuss the test cases for the requirements we achieved on core part [2].

RAN4#112bis meeting Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - On top of Single Direct SCell Activation at SCell addition, MAC CE based Single SCell Activation is also supported. - Note: MAC CE based PUCCH SCell activation is included.

In the design of legacy test cases, known SCell and unknown SCell cases were considered for SCell Activation and deactivation in A.6.5.3.1, A.6.5.3.2, A.6.5.3.3, and only the validation of known SCell case was considered for Direct SCell activation at SCell addition in A.6.5.3.4 and PUCCH SCell Activation and deactivation in A.6.5.3.6. Similarly, for known SCell case, R19 did not modify the previous requirements, but only modified the requirements for the case where the target SCell is unknown and meets the conditions and the requirenebts refer to 8.3.2A. Therefore, we think that only one scenario needs to be considered in the design of the test case, and the requirements of unknown SCell case are the same as the former and do not require repeated validation. According to the current spec, we tend to validate the SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode.

Proposal 1: Define test cases to verify the SCell Activation Delay Requirement for unknown SCell based on measurement in IDLE/INACTIVE mode.

In the previous discussion, we discussed the issue on applicability of fast SCell activation delay requirements and reached a conclusion blew [3][4].

RAN4#112 meeting Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command. RAN4#113 meeting Issue 1-1-1: Applicability of fast SCell activation delay requirements Agreement: - For R19 fast SCell activation for UE supporting R18 EMR, when neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured: - Fast SCell activation delay requirements (same with conclusion of the case where either measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured) apply under the following conditions: - For EMR carriers: - Consider the case when the T331 timer hasn’t expired at the moment of initiation of RRC state transition to Connected mode. i.e., measIdleDuration-r16 is configured, and T331 is running. Otherwise, follow the legacy unknown SCell activation delay requirements

According to the agreements in core part, we should consider three cases that:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

R18 EMR is used for the first bullet to reduce the activation delay. Meanwhile, R18 cell reselection measurement results and R16 EMR are used for bullet2 and bullet3 separately. In our understanding, the reporting for these cases has been verified in previous release and the validitity of case1 and case2 has been checked in release 18. So, it can be used directly. In release 19, we need to consider test cases to verify that the SCell activation time are within the requirements stated in clause 8.3.2A, and R16 EMR and R18 eEMR including valid EMR and valid cell reselection measurement should be considered when designing the test case.

Proposal 2: R16 EMR and R18 eEMR including valid EMR and valid cell reselection measurement should be considered when designing the test case.

Conclusions

In this paper, we have some discussion on fast SCell activation for UE supporting Rel-18 EMR. And we have the following proposals:

Proposal 1: Define test cases to verify the SCell Activation Delay Requirement for unknown SCell based on measurement in IDLE/INACTIVE mode.

Proposal 2: R16 EMR and R18 eEMR including valid EMR and valid cell reselection measurement should be considered when designing the test case.

References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT

- R4-2420094, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT