3GPP TSG-RAN WG4 Meeting # 114bis                                R4-2503855

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.27.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on CSI-RS based L1-RSRP measurement

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for CSI-RS based L1-RSRP measurement.

Discussion

In previous meeting, we have discussed the RRM scope of CSI-RS based L1-RSRP measurement and reached consensus that RAN4 should discuss at least measurement delay, measurement reporting requirements, measurement restrictions and scheduling availability for CSI-RS based L1-RSRP measurement on LTM candidate cells, which is considered in high level and same as the procedure of study on SSB based L1-RSRP measurement on candidate cells.

Similar as SSB based L1-RSRP measurement on neighbor cells, RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells. And we discussed the known cell condition in CSI-RS based L1 measurement in previous meetings.

- Issue 4-1: Known cell definition in CSI-RS based L1 measurement Issue 4-1-1: Whether to use detectable or measurable? Tentative agreement: - Use measurable and specify the side condition for RS being measurable. Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell Candidate options: - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

It can be observed from WF that there is no doubt to use measurable other than detectable on the description of CSI-RS from the target cell configured for L1 measurement. In the current specification, detectable is used to describe SSB. According to information provided by other companies, it is considered inaccurate to use detectable here for CSI-RS because there are clear identification requirements for SSB in clause 9.2, and it is possible to perform cell search and detect SSB without prior knowledge. If no condition is specified for CSI-RS, maybe use detectable is inaccurate. To avoid confusion with SSB, we agree with this view. And we support to use measurable on the description of CSI-RS from the target cell configured for L1 measurement.

As to the side condition, there are two options mentioned by companies on the value of SINR. One is -6dB, which is same as L3 measurement, and the other is -3dB, which is same as L1 measurement on serving cell and SSB based L1-RSRP measurement in R18. In our understanding, CSI-RS based L1-RSRP measurement defined in R19 is a supplement to R18 LTM and it is reasonable to align the side condition. Therefore, we perfer to use the same SINR condition for SSB and CSI-RS based L1-RSRP measurement in LTM.

Proposal 1: Reuse the legacy side condition of SINR=-3dB for CSI-RS based L1 measurement for neighbor cell.

- Issue 4-2: Measurement procedure - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] The following is concluded in RAN4#113 meeting. Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

For this issue, we have reached consensus on step1 and step3 that consider SSB based L3 measurement and report on the same candidate cell which will excute CSI-RS based L1 measurement. The controversial aspect of this issue lies in step 2, whether SSB-based L1 measurements and reports on the same candidate cell are needed to be performed before executing CSI-RS based L1 measurements. In our understanding, for LTM, CSI-RS based L1 measurement and SSB based L1 measurement are seperated features and each can work seperately., Therfore, SSB-based L1 measurements do not need to be conducted before CSI-RS based L1 measurements, and they should be able to be conducted independently. Furthermore, if step 2 is completed first, there seems to be no need for the existence of CSI-RS L1 measurement. If step 2 is not executed and step 3 is directly executed, the Rx beam used for CSI-RS measurement may be different from the Rx beam measured for SSB measurement. Correspondingly, the best Rx beam corresponding to the Tx beam will be selected, and the received and transmitted beam pair will be more matched. So we prefer to skip step 2. However, as mentioned by other company, there maybe a long measurement delay if no SSB-based L1 measurement considered. Considering the importance of measurement delay in mobility, we can compromise to option A and introduce a UE capability to determine whether step 2 can be skipped.

Proposal 2: Step 2 about SSB-based L1 measurements and reports on the same candidate cell for FR2-1 can be skipped based on UE capability.

In the last meeting, we have discussed the definiation of intra-f and inter-f measurement for CSI-RS based L1 RSRP measurement. And there are two options can be chosen.

- Issue 4-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

For option1, it is samilar as the definition of intra-frequency/inter-frequency CSI-RS based L3 measurement, which is aglin with the previous discussion for measurement. Of course, there are some differences with CSI-RS based L3 measurement. The main reason is the different configuration methods of CSI-RS, one is to follow CSI-RS configuration for MO (MeasObjectNR), the other one is to follow NZP-CSI-RS-Resource structure. In order to address the issue that different CSI-RS resources on the serving cell may have different SCS and CP types, active DL BWP is used instead of "serving cell" in legacy L3 measurement. Meanwhile, regarding center frequency, any of the CSI-RS resources of the serving cell indicated for L1 measurement is used for judgment. In our opinion, such modifications can solve the problem of L1 and L3 mismatch based on the definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement.

For option2, the different from option1 is focus on bullet3. Regardless of whether the center frequency is aligned, only focus on whether the CSI-RS resources are included in the active DL BWP. In our understanding, it is because the difference between intra-f measurement and inter-f measurement mainly lies in whether a gap is needed during measurement. If the measurement signal is fully included in the active DL BWP, gap is not required for measurement. Conversely, if it is not included, the use of gap needs to be considered.

As to how to define CSI-RS resource is included within the active DL BWP, option2 mentions that at least 48 RBs of the CSI-RS resource need to be confined within the active DL BWP. Based on the previous agreement, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs for CSI-RS based L1-RSRP measurement on neighbor cell. Therefore, the 48RB CSI-RS is the minimum value we define for requirements and require at least 48 PRBs to be included in the active BWP. Based on the analysis above, we can compromise to option2 which not consider center frequency but within active BWP.

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- It is applied for SCS = 60KHz

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Meanwhile, for CSI-RS based L1-RSRP measurement, we discussed the measurement type, mainly study whether to cover with and without gap. In the last meeting, RAN4 made a package to discuss whether to consider measurement with gap and RTD larger than CP in R19 LTM, and we can observe the progress blew.

- ssue 4-4: Measurement type - Issue 4-7: RTD assumption Candidate options: - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP. Agreement: - Conclude this issue in the next meeting.

In our views, both with and without gap should be covered due to this is for neighbor cell measurement and measurement with gap is a typical scenario. Some companies proposed the concern. For serving cell, there is no specific restriction in time domain on configurations of CSI-RS resources for L1-RSRP measurement. So, it’s difficult to ensure all the CSI-RS of the same frequency layer are within a single gap window.

For this issue, we think it can be solved by using the same principle for CSI-RS based L3 measurement that all CSI-RS resources in the same MO are confined in the same 5ms window. However, if we consider perform SSB based L3 measurement and CSI-RS based L1 measurement in FR2, it is difficult to use one gap to cover both SSB based L3 or L1 measurement and CSI-RS based L1 measurement, so concurrent gap shall be used.

In addition, as mentioned by other company, active BWP may be dynamically switched by DCI. While CSI-RS resource configuration is configured by RRC. So the same CSI-RS resource would be measured dynamically with measurement gap and without gap. If there is no requirement for measurement with gap, how to handle this case? So, we prefer to support CSI-RS based L1 measurement with gap.

As to RTD assumption, RTD>CP is a typical case for neighbor cell measurement. It’s reasonable to consider this case in CSI-RS based L1-RSRP measurement. We prefer to define requirements for both RTD<CP and RTD>CP. Meanwhile, the same method as SSB based L1-RSRP measurement can be used, i.e., introduce an optional UE capability for RTD>CP. However, as mentioned by UE vendor, it’s complicated to use different FFT for CSI-RS based measurement. Consider UE implementation and RTD>CP has been supported in SSB based measurement, it’s ok for us to deporitize the discussion on RTD>CP.

Proposal 4: For Measurement type and RTD assumption,

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- No RAN4 requirements for RTD > CP.

- Issue 4-6: Measurement on CSI-RS with repetition ON/OFF Candidate options: - Option 1: (CATT, CT, vivo) In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. In FR2, RAN4 will define the requirements for the following case: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: (Huawei) Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. - Option 3: (Nokia) Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration. - Option 4: (Ericsson, Apple, Nokia) RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

- For the measurement on CSI-RS with repetition ON/OFF, it should up to RAN1. From RAN4’s prespective, if measurement on CSI-RS with repetition ON, beam training on CSI-RS resources would be executed and finer beam will be find. If measurement on CSI-RS with repetition OFF, beam training on CSI-RS resources can be avoided and measurement delay will be shorten. Therefore, we prefer to wait RAN1 agreement on whether to support L1 measurement on CSI-RS with repletion on.

- Proposal 5: RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

- Issue 4-8: CSI-RS resource type Candidate options: - Option 1: (Apple, OPPO, Nokia, vivo) RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision. - Option 2: (CMCC, Ericsson) Semi-persistent CSI-RS resource for LTM candidate cells is supported. - Option 3: (CATT) RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

- According to the agreement from RAN1#120 meeting, RAN4 should consider CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS. Meanwhile, for AP CSI-RS, there is no consensus on the support of aperiodic CSI-RS resource for gNB scheduled reporting or event triggered reporting.

- Agreement in RAN1#120:

- Confirm the following working assumption made in RAN1#118bis

Working Assumption (Agreement (RAN1#118bis) - In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). - Support of semi-persistent CSI-RS is subject to UE capability.

- MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

- Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500.

- Proposal 6: RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

- Issue 4-14: CSI-RS accuracy requirements Candidate options: - Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (Apple, ZTE)

As to the accuracy and side condition for CSI-RS based L1-RSRP measurement, we prefer to reuse CSI-RS based L1-RSRP accuracy of the serving cell. In our understanding, one CSI-RS resource is used for both serving and neighboring cell measurements, e.g., CSI-RS resources for L1-RSRP measurement on neighbor cell for UE#1 are in fact the resources for L1 measurement of serving cell for UE#2. Meanwhile, measurement accuracy is performance requirement, we can discuss it in performance part.

Proposal 7: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: Reuse the legacy side condition of SINR=-3dB for CSI-RS based L1 measurement for neighbor cell.

Proposal 2: Step 2 about SSB-based L1 measurements and reports on the same candidate cell for FR2-1 can be skipped based on UE capability.

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- It is applied for SCS = 60KHz

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Proposal 4: For Measurement type and RTD assumption,

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- No RAN4 requirements for RTD > CP.

Proposal 5: RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

Proposal 6: RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

Proposal 7: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple