# 3GPP TSG-RAN WG4 Meeting # 114bis                                 R4-2503861

# Wuhan, Hubei, China, 07th – 11th April, 2025

# Agenda item:	8.2

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion and draft Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

# Document for:	Approval

Introduction

Based on the LS [1] sent by RAN2 regarding the conclusions of the previous meeting, RAN2 kindly asks RAN4 to discuss the blew understandings and provide clarification on which one is correct.

In this contribution, we provide our views on the definition of the source band of the band pair.

Discussion

1 Overall description The UE capability pdcch-RACH-AffectedBandsList-r18 (FG39-4) is defined in RAN4 with the granularity as “Per band pair per band combination (between the target band for RACH transmission and band under UE’s current band combo)” For each reported capability, there are two possible values: interruption or noInterruption. Regarding the band pair (one source band +one target band) of the BC, RAN2 understands that the target band refers to the target band towards which the UE performs PDCCH-ordered early RACH. However, the definition of the source band of the band pair remains unclear in RAN2. - Understanding 1: The source band of the band pair represents the source band (or PCell) on which the PDCCH order is sent to the UE to perform PDCCH-ordered early RACH to the target cell/band. Regarding the interruption band(s), there are two further understandings, denoted by 1a and 1b: - Understanding 1a: The value “interruption/noInterruption” means that interruption is needed or not for all serving bands of the current band combination. - Understanding 1b: The value “interruption/noInterruption” means that interruption is needed or not for the source band (the same band in which the PDCCH order is sent) of the current band combination. For other serving bands, noInterruption can be always assumed. - Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. Please note that in Understanding 1, the UE cannot indicate which specific source band of the BC has an interruption or not (either all bands in current BC in 1a or the same source band that receives the PDCCH order in 1b). In Understanding 2, the information about which source band receives the PDCCH order is not provided. 2 Actions To RAN4 ACTION: RAN2 kindly asks RAN4 to discuss the above understandings and provide clarification on which one is correct.

RAN4 has discussed the granularity of FG39-4 in RAN4#110 meeting and reached consusens that the type of FG39-4 is Per band pair (between the target band for RACH transmission and band under UE’s current band combo) per band combination [2].

Issue 4-4-1: Interruption on DL symbols due to PDCCH-ordered RACH < Agreement> - Regarding the capability for interruption on DL symbols due to PDCCH-ordered RACH, the reported granularity is Per band pair (between the target band for RACH transmission and band under UE’s current band combo) per band combination. Details of signalling design is up to RAN2.

In our understanding, whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission should be depends on the value “interruption” or “noInterruption”. For example, suppose there is a band combination that contains three bands, denoted as band1, band2 and band3. According to the design of per band pair per band combination, if we want to check whether the RACH on the target band (band4) causes interruption to the source band, it can be solved by looking up the list. For example, if it causes interruption to band1, the value of band pair {band1, band4} is “interription”. Similarly, to determine whether interruptions will occur on other bands, it depends on the value of band pair {bandX, band4}. therefore, we think understanding2 is correct.

Proposal 1: Prefer to follow understanding2 that the source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band.

Conclusions

In this contribution, we put forward the following proposal on the definition of the source band of the band pair.

Proposal 1: Prefer to follow understanding2 that the source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band.

References

- R2-2501392, LS to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18, RAN2

- R4-2403385, WF on NR Mobility Enhancements (Part 1), MediaTek

Annex

# 3GPP TSG-RAN WG4 Meeting # 114bis                                 R4-xxxxxxx

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Response to:	R2-2501392/R4-2503115

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Wu Yan

wu.yan7@zte.com.cn

Send any reply LS to:    3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1 Overall description

RAN4 would like to thank RAN2 for their LS on granularity definition of pdcch-RACH-AffectedBandsList-r18 in R2-2501392/R4-2503115. Based on the LS sent by RAN2, RAN4 understand that the source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band..

# 2 Actions

To RAN2

ACTION: 	RAN4 would like RAN2 to take into account the above RAN4 replies/feedback in their future work on granularity definition of pdcch-RACH-AffectedBandsList-r18.

# 3 Dates of next TSG-RAN WG RAN4 meetings

RAN4#115, 19th - 23rd May 2025, Malta.

RAN4#116, 25th-29th August 2025, India.