3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503870

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.28.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for Rel-19 XR

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 discussed the RRM impacts of the WI with agreements and issues captured in the WF [1]. In this contribution, we further provide our views on remaining issues.

# Discussion

## 2.1 Indication

In RAN4#113 meeting, there was very controversial discussion on DCI-based indication and RRC-based indication. RAN4 reply the LS in [2]. In the LS reply, RAN4 confirm the DCI-based solution, and at the same time, RAN4 is also discussing the need of semi-static based solution. As commented many times in previous meetings, there is clear benefits of semi-static solution compared with DCI-based solution.

Benefits of semi-static solution:

- It is less complicated to UE implementation that UE can have a stable long-term measurement pattern.

- Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay.

- A single RRC signal has less overhead than DCI which is per-occasion indication.

And it can also be observed from the discussion and also captured in chairman’s notes, all UE vendors support that semi-static solution can ease UE implementation complexity. One of the major concerns is that whether the RRC-based solution can support the XR service as it is not that flexible as DCI-based solution. It could be observed from the study phase that benefits come from that some XR frame will collides with MG inevitably due to non-integer frame interval. These patterns are not that frequently changed, and semi-static approach can cancel the MG which will collide with XR frame in following several occasions. And with that information, UE can re-arrange the measurement pattern using the available MG.

Observation 1: Compared with DCI-based solution, semi-static approach has following benefits:

- It is less complicated to UE implementation that UE can have a stable long-term measurement pattern.

- Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay.

- A single RRC signal has less overhead than DCI which is per-occasion indication.

However, though the discussion in RAN4 is controversial, we didn’t observe very concrete argument why the semi-static approach is not needed. The arguments are mainly about working procedure wise (e.g. other WG has made decision though we do not agree with the statement). The only technical concern is whether it will lead to UE only supporting semi-status approach if it is allowed.

Unfortunately, we don’t have chance to discussed the issue in last RAN4 meeting, the status are the same as the meeting before last meeting:

## Issue 6-1: Semi-static schemes <Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

As we all know, the semi-static solution is not a pure RAN4 discussion. If it is agreed to be supported, at least RAN2 needs to discuss the MAC-CE/RRC impacts. However, if this issue is keeping untouched in RAN4, other working group will have no time for further discussion even it is agreed to be introduced.

Thus, it is proposed that RAN4 to conclude whether to introduce semi-static indication by RAN4#114bis.

Proposal 1: RAN4 to conclude whether to introduce semi-static indication by RAN4#114bis.

Proposal 2: RAN4 to consider following approach to move forward:

- RAN4 to introduce semi-static indication to skip measurement gap.

- RAN4 discuss whether UE is allowed to only support semi-static indication.

## 2.2 Time offset requirement for measurement gap skipping

Regarding the processing time for measurement gap skipping, RAN4 achieved following agreements in last meeting.

## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

Regarding the FFS point, as we emphasized several times in last meeting, the way forward that we can compromised to a smaller value which is less than 5 ms is to limit to certain scenario. From UE implementation perspective, if UE is also need to perform measurement for LTE using MG, gNB has no idea which the targeting MG is for NR measurement or LTE measurement which is up to UE measurement pattern among different frequency layers/RAT. It may need more processing time to cancel the MG since the MG is already scheduled for LTE measurements. As commented by companies when discussing whether to include 3G measurement, it takes longer time than pure NR measurement and it is undesirable to modify the implementation of previous RAT.

Observation 2: When the MG is used to perform measurement for NR and LTE, gNB has no idea whether the following MG occasion is for NR or LTE, which is up to UE implementation.

Observation 3: Longer processing time may be needed to cancel a MG scheduling for LTE measurement, which involving cross-RAT processing.

Based on above conditions, 3 ms optional capability only apply when UE is only configured with NR measurement.

Proposal 3: 3 ms optional capability only apply when UE is only configured with NR measurement.

## 2.3 Requirements impacts

Regarding how to capture impact of measurement skipping, following options are considered:

## Issue 3-1: How to capture impact of measurement skipping ## Issue 3-1: Principles for measurement delay calculation <Agreement>: - Follow NR-U-like approach for designing the formulas for measurement delay extension. ## Issue 3-2: Formulas for measurement delay extension <Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options. - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases.

It is agreed to following NR-U approach to extend the measurement period requirements. Regarding the detailed formula, option 2 is preferred. For option 1, the extended measurement occasions are not scaled by CSSF, it somehow meanings UE shall prioritize the measurement on certain carriers. However, when a certain MG is skipped, we cannot determine on which carriers the MG is used for measurement, which is up to UE implementation.

Proposal 4: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended.

- Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

## 2.4 UE assistance information

RAN4 spend most online time to discuss whether to introduce UAI and the requirements impacts with following agreements:

## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. <Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

From our understanding at least following types of UAI shall be introduced which will help NW scheduling.

- UAI: number of occasions that can be skipped without measurement delay extension

Based on the discussion in last meeting, companies raise concerns that indication about unused MG may expose the UE implementations on detailed measurement pattern. Another concern is that the detailed unused MG pattern has to be worked with semi-static indication, and it may not valid if the cancelled MG beyond the UE UAI indication. We agree with the comments above. This is to achieve maximum resource efficiency trying to utilized all unused MG. In this meeting, we try to proposed a compromised solution with the balance between robustness and resource efficiency. Taking following Fig.1 as example, there are two carriers to be measurement using MG, and the SMTC for CC1 and CC2 is 20ms and 40ms, and the MGRP is 20 ms. The CSSF is calculated as 2.

Fig.1

Within the measurement period of 5*max (40,20) *2 (i.e. the green block), there is total 20 MG occasions. According to the minimum requirements defined in the spec, 10 MG is needed for CC1 and 5 MG is needed for CC2. Which means there are 5 MG unused. One conservative way is that when number of skipped MG is smaller (e.g. 2), no matter which 2 MGs are cancelled, UE will always be able to find other alternative MG occasions to use within the measurement period. Though, not all 5 unused MG can be utilized, the performance of RRM measurement will not be impacted when there are not too much MG to be canceled. It should be noticed that the example is based on the minimum requirements defined in the spec, and if UE can do betters, the value could be larger.

Proposal 5: Introduce UAI on number of MG occasions that can be skipped within measure period without measurement delay extension.

Note: It applies to both DCI-based solution and semi-static solution if introduced.

Another discussion related to UAI is about the condition for measurement skipping.

## Issue 3-3: Conditions for measurement skipping <Way forward>: - Option 1: conditions are needed - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133 - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

As commented during last meeting, the too much consecutive skipped MG, UE may lose the measurement timing and is not able to combined the new measurement results and results long time before, as shown in following fig.2. Besides, as discussed during the AH in last meeting, this value is hard to be predefined. It depends on UE implementation on maintaining the timing/AGC, and the MGRP configurations and number of carriers need to be measured. For instance, if there is multiple carriers to be measured using MG, the interval should be even smaller as the measurement occasions are shared by multiple carriers. Upon exceeding the maximum interval, UE may need to restart the measurement instead of simply extending the delay.

Fig.2

Observation 4: The interval between available measurement occasions shall be defined as conditions for delay extension. Upon exceeding the maximum interval, UE may need to restart the measurement instead of simply extending the delay.

Observation 5: The interval between available measurement occasions is hard to be predefined with constant value which depends on UE implementation on maintaining the timing/AGC, and the MGRP configurations and number of carriers need to be measured

Proposal 6: Introduce UAI maximum interval between available measurement occasions for delay extension. UE is allowed to restart measurement upon exceeding the interval instead of only extending the delay.

## 2.5 Applicable range of measurement gap skipping

Regarding the applicable CC for MG cancelling, one important aspect should be considered is the supporting of per-FR gap or per-UE gap. RAN1 achieved following agreements in last meeting,

Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

From UE implementation perspective, if UE is capable of per-FR gap, and if the per-FR gap is cancelled, UE shall be ready for data reception/transmission in the MG in the FR and has no impacts on UE behavior in the other FR. However, different from legacy RRM requirements which only relies on whether UE supporting per-FR gap, the situation for MG cancellation is a bit different. If gNB is not implementing the per-FR gap, and only per-UE gap is configured to a UE which is capable of per-FR gap, there could be misunderstanding between UE and gNB. UE may keep measurement in the other FR and gNB may think all CCs are ready for data scheduling. Thus, it should be relied on whether per-FR gap is actually configured by gNB instead of whether UE is supporting per-FR gap.

Proposal 7: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# Conclusions

Observation 1: Compared with DCI-based solution, semi-static approach has following benefits:

- It is less complicated to UE implementation that UE can have a stable long-term measurement pattern.

- Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay.

- A single RRC signal has less overhead than DCI which is per-occasion indication.

Proposal 1: RAN4 to conclude whether to introduce semi-static indication by RAN4#114bis.

Proposal 2: RAN4 to consider following approach to move forward:

- RAN4 to introduce semi-static indication to skip measurement gap.

- RAN4 discuss whether UE is allowed to only support semi-static indication.

Observation 2: When the MG is used to perform measurement for NR and LTE, gNB has no idea whether the following MG occasion is for NR or LTE, which is up to UE implementation.

Observation 3: Longer processing time may be needed to cancel a MG scheduling for LTE measurement, which involving cross-RAT processing.

Proposal 3: 3 ms optional capability only apply when UE is only configured with NR measurement.

Proposal 4: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended.

- Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of skipped MG occasions in the measurement period.

Proposal 5: Introduce UAI on number of MG occasions that can be skipped within measure period without measurement delay extension.

Note: It applies to both DCI-based solution and semi-static solution if introduced.

Observation 4: The interval between available measurement occasions shall be defined as conditions for delay extension. Upon exceeding the maximum interval, UE may need to restart the measurement instead of simply extending the delay.

Observation 5: The interval between available measurement occasions is hard to be predefined with constant value which depends on UE implementation on maintaining the timing/AGC, and the MGRP configurations and number of carriers need to be measured

Proposal 6: Introduce UAI maximum interval between available measurement occasions for delay extension. UE is allowed to restart measurement upon exceeding the interval instead of only extending the delay.

Proposal 7: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# References

[1] R4-2502700 WF on RRM requirements for XR_Ph3

[2] R4-2420198 Response LS on gaps/restrictions that are caused by RRM measurements