3GPP TSG-RAN WG4 Meeting #114bis	R4- 2503875

Wuhan, China, 07 - 11, April, 2025

Agenda item:	7.17.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part for RRM Phase 5

Document for:	Discussion

- Introduction

According to the TU schedule, the performance part for RRM phase 5 starts from the RAN4#114bis meeting. In this contribution, we provide our views on the performance impacts of the WI.

- Discussion

L3 measurement enhancement by optimizing Rx BSF

There are three separate objectives in this WI, and we provide our views on performance impact in case by case manner.

For FR2-1 enhancement based on multiple-Rx simultaneous reception. Based on progress so far in core requirements discussion, RAN4 mainly discussed the triggering conditions, and the scenario is limited to PCell only case without CA/DC. For performance part, it is premature to discuss the test cases list and detailed test configurations. In general, it is feasible to define test cases to verify the enhancement L3 measurement delay.

Proposal 1: Define test case(s) for L3 measurement enhancement by optimizing Rx BSF. The detailed test case list and test configuration are to be further discussed with more progress on the core part discussion.

L3 measurement delay by optimising CSSF outside gap in CA/DC

In the last meeting, we concluded the core requirement details of Solution 1 for the CSSF reduction. Specifically, a new network capability will enable or disable the UE to do the enhancement that measures one serving CC per band when multiple serving CCs are configured in the same band. Based on this design, this enhancement requires a test set to verify its performance. For current test cases, the test set is, for example, one PCell and one SCell to test the report delay, so RAN 4 needs to discuss whether or not to define a new test set for Solution 1 to verify whether or not UE achieve this enhancement.

Proposal 2: RAN 4 needs to discuss whether or not to define a new test set for Solution 1 to verify whether or not UE achieve this enhancement.

For CSSF reduction Solution 3, we need to define the performance requirement as well. The 3-searcher solution can allow UE to do the measurement of 3 MO simultaneously. In this case, two things are different from legacy 2-searcher solution performance requirements. Firstly, the test parameters about the Cell set should be like FR1 PCell (Cell 1), an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2), another FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4). Secondly, the measurement reporting delay accuracy needs to be considered because the 3-searcher solution may reduce the measurement delay when multiple MO are configured and further discusses the accuracy value.

To summarise, from our perspective, two things need to be considered in the performance requirements for Solution 3:

- Test parameters of Cell amount. (e.g. FR1 PCell (Cell 1), an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2), another FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4).

- Test requirements to define a new measurement reporting delay accuracy.

Proposal 3: The following points should be considered in the performance requirements for Solution 3 of the CSSF reduction:

1.	Test parameters of Cell amount, for instance at least 5. (e.g. an FR1 PCell (Cell 1); an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2); FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4).)

2.	Test requirements to define a new measurement reporting delay accuracy.

- Conclusions

Proposal 1: Define test case(s) for L3 measurement enhancement by optimizing Rx BSF. The detailed test case list and test configuration are to be further discussed with more progress on the core part discussion.

Proposal 2: RAN 4 needs to discuss whether or not to define a new test set for Solution 1 to verify whether or not UE achieve this enhancement.

Proposal 3: The following points should be considered in the performance requirements for Solution 3 of the CSSF reduction:

1.	Test parameters of Cell amount, for instance at least 5. (e.g. an FR1 PCell (Cell 1); an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2); FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4).)

2.	Test requirements to define a new measurement reporting delay accuracy.

References

[1] RP-240830 WID: NR Radio Resource Management (RRM) Phase 5