3GPP TSG-RAN WG4 #114bis	R4-2503878

Wuhan, China, 07 – 11 April, 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM core requirements for Rel-19 ATG

Agenda Item:	7.12.4

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The points and views of companies are detailed in the WF [1]. This paper presents our observations and views on RRM impacts on ATG CA.

RRM core requirement analysis

Compared with the requirements defined in Rel-18 ATG for single-Cell, we provide our analysis on the potential RRM impacts to support CA in case by case manner in the following part.

ATG UE capability

In the last meeting, we discussed the ATG UE that two bands share one panel and use a common beam. A new UE capability will be defined as shown below:

Agreement: - To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed - FFS for the details.

In this paper, we provided our perspective on the details of this new capability. Generally, UE will report the antenna type in an omnidirectional or antenna array. This capability is optional for ATG UE, the activate condition is ATG UE equipment with the antenna array with common beam. This new capability is used to let the network know that the ATG UE is using an antenna array with a common beam. For RRM requirements, this new capability only affects the following features:

      - SCell deactivated measurement with L1/L3 measurement factor.

      - Scheduling Availability/Restrictions.

      - SCell measurement enhancement.

      - FFS

Proposal 1: The capability is optional and will be reported by ATG UE if it is equipment with antenna array with a common beam. This new capability only affects the following RRM requirements:

1.	SCell deactivated measurement with L1/L3 measurement factor.

2.	Scheduling Availability/Restrictions.

3.	SCell measurement enhancement.

4.	FFS

Interruption requirement

In the last meeting, there were some FFS bullets which are shown below:

Issue 1-3-1-2: Whether Interruptions at NR SRS carrier based switching requirements need to be introduced FFS - Proposal 1: The NR SRS carrier based switching interruption requirement can be introduced.

Do the technical analysis, in the ATG scenario, this interruption requirement is still necessary. SRS carrier-based switching is primarily used in downlink CA-only scenarios, particularly when the SCell operates on a TDD band. This approach allows the UE to send SRS on both the PCell and SCell, enabling the network to assess the SRS weight of the SCC, which in turn enhances downlink transmission.

The current discussed ATG band combination is n3+n39 which is an FDD band and a TDD band. This band combination is matched to the SRS carrier based switching applicability.

Proposal 2: The NR SRS carrier based switching interruption requirement can be introduced and it is technically feasible.

In the last meeting, we discussed the spec structure. About the new chapter 8.2D interruption for ATG, we think the new chapter is needed and we just put every interruption feature that ATG UE needed in CA enhancement in this new chapter.

CR split 3.6.16 Applicability of requirements for ATG; CMCC. 4.2D Cell Re-selection for ATG; CMCC. 7.3D Timing advance for ATG; Intel. 7.6D(new) Maximum Receive Timing Difference for ATG; Intel. 8.1D Radio Link Monitoring for ATG; HW. FFS 8.2D(new) Interruption for ATG; Intel. 8.3D SCell Activation and Deactivation Delay for ATG; Apple. 8.5D Link Recovery Procedures for ATG; HW. 8.6D Active BWP switch delay for ATG; Apple. 8.19D Pre-configured measurement gap activation/deactivation delay for ATG; Ericsson. 9.1D General measurement requirement for ATG; CATT. 9.2D NR intra-frequency measurements for ATG; ZTE. 9.3D NR inter-frequency measurements for ATG; ZTE. 9.5D L1-RSRP measurements for Reporting for ATG; LGE. 9.8D L1-SINR measurements for Reporting for ATG; Ericsson. 9.10D CSI-RS based L3 measurements for ATG; CATT.

Proposal 3: RAN 4 define the new chapter 8.2D Interruption for ATG.

SCell Addition/Activation mechanism

In the last meeting, companies proposed to discuss the SCell addition/activation mechanism.

FFS - Proposal 1: Consider distance triggered SCell addition (CMCC, HW) - Proposal 1-1: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network.(CMCC) - Proposal 1-2: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type. (HW) - Proposal 2: RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion. (Ericsson) - Proposal 3: further optimization methods for SCell addition and activation can be considered (CATT) - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

In the discussion, the SCell activation or addition based on location information is feasible but we still need to consider the UE antenna type affection.

For instance, when the ATG UE equipment with the antenna array the location information matches the SCel activation condition the NW will activate the SCell. However, the beam of SCC may not aim at the direction of the NW SCell that is activated and needs to justify the beam direction from the original SCell to the new SCell direction.

On the other hand, if the UE equipment with the sharing panel antenna array this may be easy to achieve. The reason is that the PCC and SCC use the same panel and the beam direction of PCC and SCC is almost the same because of the co-located PCell and SCell. In this situation, the NW is able to activate SCell when the ATG UE is matched to the distance-triggered condition.

Proposal 4: The distance triggered SCell addition and/or activation is applicable to UE antenna Case 4 Exclusively.

Deactivated SCell measurement

The following is the conclusion in the WF[1]:

Agreement: - For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed - FFS: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

There are 5 types of UE antenna in ATG Rel-19 discussion. Due to variations in UE types, including omnidirectional and array antennas, the deactivated state does not have the L1 measurement, so we think the L1/L3 measurement sharing factor is not needed. For Case 4 when doing the SCell deactivation the L1/3 measurement sharing factor is needed.

Proposal 5: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed.

Measurement enhancement for ATG UE with common beam

In the last meeting, several measurement enhancements were discussed that could provide significant performance improvements. From our perspective, when the ATG UE uses a Case 4 antenna type, the SCell may not require neighbour measurements. This is because the PCC and SCC share the same beam, and SCC selection is based on the PCC selection. In other words, the SCC activated by the ATG UE is always co-located with the PCC. Therefore, in some scenarios, the neighbour cell measurement for the ATG UE's SCell is redundant.

Proposal 6: In some scenarios, the SCell neighbour cell measurement is redundant for ATG UE with Case 4 antenna type.

Conclusion

Proposal 1: The capability is optional and will be reported by ATG UE if it is equipment with antenna array with a common beam. This new capability only affects the following RRM requirements:

1.	SCell deactivated measurement with L1/L3 measurement factor.

2.	Scheduling Availability/Restrictions.

3.	SCell measurement enhancement.

4.	FFS

Proposal 2: The NR SRS carrier based switching interruption requirement can be introduced and it is technically feasible.

Proposal 3: RAN 4 define the new chapter 8.2D Interruption for ATG.

Proposal 4: The distance triggered SCell addition and/or activation is applicable to UE antenna Case 4 Exclusively.

Proposal 5: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed.

Proposal 6: In some scenarios, the SCell neighbour cell measurement is redundant for ATG UE with Case 4 antenna type.

References

[1] R4-2502591 WF on RRM requirements for NR_ATG_enh