3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503884

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on CSI-RS based L1 measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.27.2.2

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on CSI-RS based L1 measurement on neighbor cells.

Discussion

Know cell definition

It is agreed that RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells.

- Agreement: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells

The status of known cell definition for CSI-RS based L1 measurement for candidate cells is duplicated from [2] as below.

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition. - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

The first FFS is related with the CSI-RS L1 measurement procedure which will be discussed later. The second FFS is regarding with the term of “detectable and measurable”. In the existing L1-RSRP measurement for serving cell, “measurable” is used for SSB/CSI-RS resources for L1 RSRP measurement. The reason is that no cell detection is considered for L1-RSRP measurement. We think the similar description can be inherited to candidate cells.

- 9.5.2 Requirements applicability The requirements in clause 9.5 apply, provided: - The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable. An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met: - L1-RSRP related side conditions given in clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band, - SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band. A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met: - L1-RSRP related side conditions given in clauses 10.1.19.2 and 10.1.20.2 for FR1 and FR2, respectively, for a corresponding band, - CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.4.2 for a corresponding band. A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource.

Proposal 1: In known cell definition, the CSI-RS from the target cell configured for L1 measurement remains measurable.

In R18 LTM, the SINR of SSB based L1-RSRP is -3dB. In R19, CSI-RS based L1-RSRP measurement is introduced. We don’t observe the justification for CSI-RS based L1-RSRP to use a different SINR condition than SSB based L1-RSRP.

Proposal 2: SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB.

Measurement procedure

In RAN4#113 meeting, the basic structure of the measurement procedure for CSI-RS L1 measurement on candidate cells was settled [2]. RAN4 agreed to have three steps: UE performs SSB based L3 measurement and report (i.e., step1), and then UE performs SSB-based L1 measurement on the same candidate cell (i.e., step2), after that CSI-RS based L1 measurement would be performed on the same candidate cell (i.e., step3). There are still one FFS and one bracket.

- Issue 3-2: Measurement procedure <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

In our understanding the basic procedure of CSI-RS based L1 measurement on candidate neighbour cell shall be,

- Step 1 (gNB TX wide SSB beam, UE RX wide SSB beam): UE performs SSB based L3 measurement and acquires the SSB index information. UE reports the L3 SSB measurement results with SSB index to network.

- Step 2 (gNB TX wide SSB beam, UE RX fine SSB beam): UE performs SSB based L1-RSRP measurement on the candidate cell to refine the SSB based RX beam. Step 2 is what we specified for R18 LTM where UE sweeps RX beam based on SSB.

- Step 3 (gNB TX narrow CSI-RS beam, UE RX fine SSB beam): UE performs measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement in step 2. Herein UE performs L1 CSI-RS based measurement without RX beam sweeping. Step 3 is similar as R15 CSI-RS based L1-RSRP measurement, the difference is step3 is on candidate cell and R15 CSI-RS based L1-RSRP measurement is on serving cell.

If step 2 is removed, the subsequent measurement procedure is: CSI-RS resources within a resource set are transmitted with different downlink spatial domain transmission filter, and UE sweeps Rx beam with finer CSI-RS level beam. The schematic diagram is shown in figure 1.

Figure 1. CSI-RS L1 measurement on neighbour cell with skipping step2 (NOT preferred)

At first glance, skipping step 2 (shown in Figure 1) can get the refiner RX beam, however some outstanding drawbacks and implementation restriction are observed:

1. Due to RX beam sweeping on CSI-RS resource of neighbour cell, the CSI-RS measurement delay of skipping step2 would be N times (N is UE Rx beam number) than that of keeping of step 2. In other words, the timeliness of obtaining CSI-RS measurement result would be worse.

Moreover the measurement delay of neighbour cell would be quite longer (N times) than that of serving cell where typical L1 measurement periodicity is 20ms for serving cell. As we know, in FR2 environment, the channel changes quickly. If UE is moving to the cell edge, due to different measurement periodicity, the RSRP on serving cell would react channel degradation faster than that on neighbour cell. It is most likely that UE will switch to neighbour cell. However when “neighbour cell” becomes serving cell, RSRP fluctuation of the current serving cell will be intense, and UE may quickly switch back. In a word, different measurement periodicity between neighbour cell and serving cell may result in “ping-pong” handover.

Observation 1: The CSI-RS based L1-RSRP measurement delay of skipping step2 would be N times (N is UE Rx beam number) than that of keeping of step 2.

Observation 2: Longer L1-RSRP periodicity of neighbour cell and short L1-RSRP periodicity of serving cell would result in “ping-pong” handover.

2. In field network, CSI-RS Repetition ON is extremely rare to be configured due to the large overhead burden. So in serving cell it is typically assumed CSI-RS resources repetition set to OFF. It means that P-3 is typically NOT executed in beam management procedure where P-3 is that CSI-RS resources is transmitted repetition ON and UE refines the Rx beam to CSI-RS level. So according to R15 CSI-RS L1-RSRP measurement requirements for serving cell, UE uses SSB level RX beam to perform CSI-RS measurement (QCL-D in qcl-InfoPeriodicCSI-RS is SSB).

- For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

If L1 CSI-RS measurement on neighbour cell is executed with RX beam sweeping, the RX beam would be refined to CSI-RS level beam. Then the RX beam gain on serving cell and on neighbour cell would be different. Therefore the RSRP comparison between serving cell and neighbor cell will be unfair.

Observation 3: For CSI-RS L1-RSRP measurement, RX beam gain on serving cell and on neighbour cell would be different if skipping step 2.

3. As per existing R15 CSI-RS L1-RSRP measurement requirements, the CSI-RS resource for L1 measurement shall be QCL-D with SSB index for L1 measurement (rather than SSB for L3 measurement, see requirements in above table). The reason is that the interaction between L1 measurement module and L3 measurement module may be not supported in UE implementation. Therefore it may encounter UE capability obstacle to directly use the L3 SSB information for CSI-RS L1 measurement, if step 2 is skipped.

Observation 4: It may have UE capability obstacle to directly use the L3 SSB information for CSI-RS L1 measurement, if step 2 is skipped.

Based on the above analysis, we think step 2 shall be kept. In RAN4#114 meeting, some companies think keeping step 2 means that R18 SSB based neighbour cell measurement shall be supported as a prerequisite of the CSI-RS measurement procedure. We don’t think so. If a R19 UE declares to support neighbour cell L1 CSI-RS measurement, the UE shall implement the three steps as a whole package. It is no need for the UE to first declare to support R18 SSB based L1 measurement. Moreover from network configuration perspective, each CSI-RS resources shall be QCL’d with SSB, so the SSB resource configurations are not additional configuration. We also understand that the measurement procedure was discussed for multiple meeting cycles and no consensus was achieved. To proceed the progress, we can compromise to introduce a UE capability. But we think the requirements for both keeping step 2 and skipping step2 shall be specified.

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

Proposal 3: Regarding the measurement procedure for CSI-RS based LTM candidate cell measurements, whether step 2 can be skipped or not can be based on UE capability reporting. The requirements for both keeping step 2 and skipping step2 are to be specified.

- Issue 4-6: Measurement on CSI-RS with repetition ON/OFF Candidate options: - Option 1: (CATT, CT, vivo) In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. In FR2, RAN4 will define the requirements for the following case: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: (Huawei) Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. - Option 3: (Nokia) Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration. - Option 4: (Ericsson, Apple, Nokia) RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

If CSI-RS resources of neighbor cell are set to Repetition ON, the CSI-RS resources in the resource set use the same downlink spatial domain transmission filter (i.e, TX beam), and UE is supposed to sweep Rx beam. The intention of such kind of configuration is to train best RX beam for each TX beam. As discussed above, even for serving cell CSI-RS Repetition ON is extremely rare to be configured due the large overhead burden in practical network, we don’t think repetition ON configuration would be applied for CSI-RS L1 RSRP measurement on candidate cells.

Proposal 4: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Intra-frequency CSI-RS based L1 RSRP measurement definition

In last meeting, the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement was discussed and diverse options are supported by companies [3].

Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

We understand the motivation of both options. Option 1 follows the legacy way of intra-frequency/inter-frequency measurement definition. This kind of definition keeps the consistency of the definition L3 SSB measurement, L3 CSI-RS measurement and L1 SSB based measurement. This can facilitate the readability and comprehensibility of the specification.

Option 2 is a different way of the legacy intra-frequency/inter-frequency measurement definition. It cares about whether measurement gap is needed. The CSI-RS resources within active BWP are called as intra-frequency and measured without gap, even for CSI-RS resource in neighbour cell which has different center frequency as that of any resources in the serving cells.

If neighbour cell L1 CSI-RS RSRP measurement with gap is supported in R19 mobility, UE may still be able to simultaneously measure multiple CSI-RS resources whose bandwidth are within UE current measurement bandwidth when RF chain is switched outside active BWP. But the measurement bandwidth using gap is left to UE implementation, it is unknown to network how many CSI-RS resources can be measured in parallel. So to define measurement requirements, the “CSI-RS frequency layer” concept is to be introduced. And then the comparisons of center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources will be performed. If the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer, otherwise they are regarded as different frequency layers. To our understanding, the simple way of defining the measurement period of CSI-RS measurement with gap is to be scaled with CSI-RS frequency layers.

In summary if CSI-RS RSRP measurement with gap is not supported, either options of intra-frequency CSI-RS based L1 RSRP measurement definition have no big difference, as it is just a term. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. In essence the CSI-RS frequency layer is the comparison of center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources. Then the intra-frequency definition based on center frequency is more preferred.

Proposal 5: If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference.

Proposal 6: If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred.

Measurement with/without gap

One open issue is whether to support CSI-RS based L1 measurement with measurement gap.

- Issue 3-8: supported measurement type <Agreement> - CSI-RS based L1 measurement without gap is supported. - FFS on CSI-RS based L1 measurement with gap

On one hand, the CSI-RS based L1-RSRP measurement on candidate can be outside active BWP, as the active BWP may be changed dynamically.  If RAN4 only define requirements for measurement without gap case, it would restrict network configuration. Moreover gap based measurement is a typical scenario. Supporting completed mobility scenarios would be active for the prospect of feature application.

On the other hand, for R18 SSB based neighbour cell L1 measurement with gap, the measurement period is scaled by CSSFinter. For CSI-RS based L1 measurement, the CSI-RS resource configuration is more flexible than SSB, so the CSI-RS frequency layers would be larger than SSB frequency layers. Therefore from defining measurement requirement perspective, the measurement delay would be larger.

Therefore we think if RAN4 agrees to define requirements for L1 neighbor cell measurement with gap, the requirements shall be scaled with CSI-RS frequency layers and one capability is optional. Otherwise no requirements are specified in R19.

Proposal 7: If RAN4 agrees to define requirements for L1 neighbor cell measurement with gap, the requirements shall be scaled with CSI-RS frequency layers and one capability of supporting measurement with gap is optional. otherwise no requirements are specified in R19.

RTD assumption

In R18 LTM, the capability of SSB based measurement with RTD>CP is introduced to support the scenario when RTD is larger than CP. In R19 CSI-RS based neighbour cell measurement, companies have diverse views on whether supporting RTD>CP. We understand that the bandwidth CSI-RS resources are typically larger than SSB, additional FFT to accommodate CSI-RS resources with RTD>CP would take more effort than supporting SSB. Moreover as the bandwidth of FFT size for CSI-RS RSRP measurement is left to UE implementation, it is uncertainty how many CSI-RS resources are covered by one FFT. For R18 SSB based neighbour cell L1 measurement, the measurement period with RTD>CP is scaled by SSB frequency layers. For CSI-RS based L1 measurement, the CSI-RS resource configuration is more flexible than SSB, so the number of CSI-RS frequency layers would be larger than that of SSB frequency layers. Therefore from defining measurement requirement perspective, the measurement delay would be larger.

On the other hand, we understand if no requirements for RTD>CP, the application scenarios of the feature would be limited.

The situation of RTD assumption is very similar as inter-frequency with gap discussed in clause 2.4. If RAN4 agrees to define corresponding requirements, the requirements shall be scaled with CSI-RS frequency layers and one capability for support RTD>CP is optional. Otherwise no requirements are specified in R19.

Proposal 8: If RAN4 agrees to define requirements for RTD>CP case, the requirements shall be scaled with CSI-RS frequency layers and one capability for support RTD>CP is optional. Otherwise no requirements are specified in R19.

Conclusion

The contribution provides analysis on CSI-RS based L1 measurement. The following proposals are proposed:

Proposal 1: In known cell definition, the CSI-RS from the target cell configured for L1 measurement remains measurable.

Proposal 2: SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB.

Observation 1: The CSI-RS based L1-RSRP measurement delay of skipping step2 would be N times (N is UE Rx beam number) than that of keeping of step 2.

Observation 2: Longer L1-RSRP periodicity of neighbour cell and short L1-RSRP periodicity of serving cell would result in “ping-pong” handover.

Observation 3: For CSI-RS L1-RSRP measurement, RX beam gain on serving cell and on neighbour cell would be different if skipping step 2.

Observation 4: It may have UE capability obstacle to directly use the L3 SSB information for CSI-RS L1 measurement, if step 2 is skipped.

Proposal 3: Regarding the measurement procedure for CSI-RS based LTM candidate cell measurements, whether step 2 can be skipped or not can be based on UE capability reporting. The requirements for both keeping step 2 and skipping step2 are to be specified.

Proposal 4: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Proposal 5: If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference.

Proposal 6: If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred.

Proposal 7: If RAN4 agrees to define requirements for L1 neighbor cell measurement with gap, the requirements shall be scaled with CSI-RS frequency layers and one capability of supporting measurement with gap is optional. otherwise no requirements are specified in R19.

Proposal 8: If RAN4 agrees to define requirements for RTD>CP case, the requirements shall be scaled with CSI-RS frequency layers and one capability for support RTD>CP is optional. Otherwise no requirements are specified in R19.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple