3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503890

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	7.7.3

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides further analysis on low band carrier aggregation via switching from RRM impact perspective.

Discussion

Interruption requirements

Operators and interested companies have provided plenty information of the background, motivation and possible solutions of low band CA [2]. The agreed WI aims to specify requirements with the example band combination CA_n5A-n29A where n29 is SDL band, and n5 is FDD band.  The two carriers are collocated and synchronized in a single TAG.

Figure 1. Example scenario (referenced from [RP-242532])

If a single antenna is implemented to aggregate the two bands, it is challenged for simultaneous Tx/Rx between the PCell and the SCell. Therefore switching between two carriers is executed. A semi-static switching pattern is configured by RRC configuration. UE is allowed to switch between case 1 and case 2.

-	Case 1: Tx/Rx on FDD carrier and no Rx on SDL carrier

-	Case 2: Rx on SDL carrier and no Tx/Rx on FDD carrier

In RAN4#114 meeting, RRM session had some discussion on potential impacted RRM requirements [3]. Regarding interruption requirements, the alternatives are listed as below.

- Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion.

In RF, the switching period for low-low band switching is concluded as 35us, 140us and FFS on other additional value(s) [4].

- Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us Additional value(s) are under discussion. If any additional value(s) are confirmed, RAN4 will provide them in a future LS. RAN4 respectfully requests RAN1 to take this information into account in their work.

As we known, the interruption length in RRM considers not only switching period but also the time misalignment between aggressive band and victim band. So interruption requirement is different as switch period discussed in RF.

Observation 1: Interruption requirement is different as switching period discussed in RF.

Furthermore, RF session agreed that DL switching happens between two low bands, while the band combination can be more than two bands. It means that the interruption requirements can also be applicable to other combined SCell.

Issue 2-1: Switching delay Agreement: - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

Observation 2: The interruption requirements shall also be applicable to other combined SCell.

RAN1 will discuss the switching gap location in the upcoming April meeting. To our knowledge, there are multiple potential options, e.g., switching period is always within the switch-from carrier, or, switching period is contained within the switching gap configured by the network, etc. So whether interruption is present on PCell depends on RAN1 decision of the switch gap location. However anyway the interruption on other activated SCell is present.

Proposal 1: Interruption requirement for carrier switching between FDD CC and SDL CC are to be defined,

- Whether interruption is present on PCell depends on RAN1 decision on the location of switch gap.

- The interruption on other activated SCell is present.

SDL SCC related RRM requirements

In RAN4#114 meeting, for supporting low band CA via switching which assumption shall be based on is widely discussed, i.e., based on the assumption of UE supporting SSB-less operation, or based on the assumption of UE not supporting SSB-less operation [3]. In the following, we provide the analysis on the to-be-defined requirements for each assumption.

Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

SDL SCC related requirements for UE supporting SSB-less operation

As per TS38.104, for inter-band carrier aggregation, TAE shall not exceed 3µs. According to the deployment constraint in WID, the SCS for both carrier is 15kHz, then CP length is 4.67us. Furthermore, as the two switching carriers are co-located and synchronized, RTD between both carriers at UE side shall be less than CP. It means that the coarse timing on SDL SCell can refer to that of PCell.

The deployment scenario for low band carrier switching is that the carrier frequency for both bands is <1 GHz. Moreover the two carriers are co-located, and the transmit power configuration for each CC is similar, therefore the values of L3/L1 RSRP on SDL SCell and PCell shall be within a limited range, and the fluctuations of L3/L1 RSRP on SDL SCell and that on PCell are basically identical as well.

Therefore regarding the SSB based L1-RSRP and CSI-RSRP based L1-RSRP measurement, the above constraints of RTD, power difference can guarantee that the SDL SCell can fully leverage the L1-RSRP measurement results from PCell. Therefore the corresponding L1 measurements on SDL SCell can be skipped.

Proposal 2: For UE supporting SSB-less operation, UE can skip SSB/CSI-RS based L1-RSRP measurement on SDL SCell, and the L1 RSRP measurement on collocated PCell can be leveraged.

Regarding L3 measurement, based on co-located assumption, there is also no necessity to perform mobility measurement on SDL carrier (including serving cell and neighbor cells on SDL carrier). In other words, a SDL carrier is always co-located with a PCC when NW can determine the mobility based on the measurement on this Cell.

Proposal 3: For UE supporting SSB-less operation, UE can skip L3 RSRP measurement on SDL carrier, and the L3 RSRP measurement on collocated PCC can be leveraged.

In R18 inter-band SSB-less WI, the SSB-Less SCell activation delay is shorter than normal SCell activation procedure when the side conditions (duplicated as below) are satisfied. For low band CA via switching, as discussed above, the first RTD condition can be satisfied. In low band carrier switching scenario, as the frequency separation is very small, EPRE difference between the two switching carriers can be smaller than 12dB.

- The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The EPRE difference at the UE is smaller than or equal to [30] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference cell and A-TRS/P-TRS of SSB-less SCell, and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

Tactivation_time is - Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB - Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for SCell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB - Tfirst_TRS + 2*TTRS +5 ms, when the EPRE difference is larger than 12 dB but smaller than or equal to [30] dB

Therefore in general the SDL SCell activation delay can take R18 SSB-less SCell activation delay (where the EPRE difference is smaller than or equal to 12 dB) as a baseline. In other words, two (A)TRS resources are needed, i.e.., one for AGC, the other is for fining time tracking.

Proposal 4: For UE supporting SSB-less operation, two (A)TRS resources are needed for SDL SCell activation with the assumption that EPRE difference is smaller than or equal to 12 dB.

As we know, RAN1 will discuss the switch pattern in the upcoming April meeting. Although switching pattern is not clear to RAN4, one principle is worth to be discussed.

- Whether SCell activation procedure shall be completed within one SDL duration, or is allowed to be completed with multiple SDL durations?

If there are no sufficient TRS resources within one SDL duration (i.e., the TRS periodicity is long) to complete SCell activation procedure, the question is whether UE is allowed to utilize the subsequent TRS resource in the next SDL duration. If yes, the SSB-less SCell activation delay may need some updating. If not allowed, it means that UE will still stay on SDL carrier although it is time to switch to FDD carrier as per configured switching pattern. The issue would get worse when more RS resources are needed.

Proposal 5: RAN4 to discuss whether SCell activation procedure shall be completed within one SDL duration, or is allowed to be completed with multiple switch SDL switch durations.

SDL SCC related requirements for UE NOT supporting SSB-less operation

For UE not supporting SSB-less operation, UE will perform L1 and L3 measurement on SDL carrier. For L1/L3 measurement on activated SCC, besides SMTC/RS periodicity and DRX, the carrier switching pattern as another factor needs to be considered. The detailed requirements depend on the switching pattern specified in RAN1.

Proposal 6: For UE not supporting SSB-less operation, the switching pattern would be also considered on top of the existing DRX and SMTC/RS periodicity when specifying L1/L3 measurement requirements on activated SDL carrier. The detailed requirements depend on the switching pattern specified in RAN1.

The L3 measurement period requirements on deactivated SDL resembles the logic of that on activated SDL carrier. The slight difference is to consider the factors among DRX cycle, measCycleSCell, and switch pattern.

Proposal 7: For UE not supporting SSB-less operation, the switching pattern would be also considered on top of the existing DRX and measCycleSCell when specifying L3 measurement period on deactivated SDL carrier. The detailed requirements depend on the switching pattern in RAN1.

It shall be noted that the legacy interruption due to measurement on deactivated SCC requires has two cases:

-when measCycleSCell is smaller than 640ms, no interruption is allowed. It means that UE shall always open the SCC RF chain after SCell addition.

-when measCycleSCell is 640 ms or longer, up to 0.5% interruption is allowed. It means UE can close SCC RF chain after SCell addition, and open the RF chain again when measurement on deactivated SCC is performed.

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

However, in R19 low band switching scenario, UE is not able to receive from both FDD DL and SDL. In other words, UE shall be always allowed to swich to PCC after measurement on deactivated SDL carrier. Therefore the missed ACK/NACK percentage shall be allowed in low band switching scenario.

Proposal 8: For UE not supporting SSB-less operation, interruptions due to measurements on deactivated SDL are allowed with up to 0.5 % probability of missed ACK/NACK.

For SCell activation delay, UE not supporting SSB-less operation would follow the normal SCell activation procedure. As we know the normal SCell activation procedure requires multiple SSB samples for cell detection/ AGC adjustment/ L1-RSRP measurement/ fine timing tracking. So the issue mentioned in section 2.2.1 will become more outstanding. It is more challenging to complete the SCell activation procedure within one SDL switching duration. Or, SDL SCell activation procedure will take more SDL durations and further prolong the entire activation delay.

Proposal 9: For UE not supporting SSB-less operation, it is more challenging to complete the SCell activation procedure within one SDL switching duration. Or, SDL SCell activation procedure will take more SDL durations and further prolong the entire activation delay.

PCC related RRM requirements

For low band switching, UE periodically switches between PCC and SDL. Whether L1/L3measurement opportunities on PCC are colliding with the duration switching to SDL depends on switching pattern.

Proposal 10: For L1/L3 measurement on PCC, whether L1/L3measurement opportunities on PCC are colliding with the duration switching to SDL depends on switching pattern.

In last meeting, RAN4 identified UL transmit timing issue on PCell and sent LS to RAN1[5]. RAN4 can wait for the progress in RAN1.

Proposal 11: The impact on UL transmit timing requirements can wait for RAN1 progress.

Conclusions

This contribution provides further analysis on low band carrier aggregation via switching from RRM impact perspective. The following proposal is provided:

Observation 1: Interruption requirement is different as switching period discussed in RF.

Observation 2: The interruption requirements shall also be applicable to other combined SCell.

Proposal 1: Interruption requirement for carrier switching between FDD CC and SDL CC are to be defined,

- Whether interruption is present on PCell depends on RAN1 decision on the location of switch gap.

- The interruptions on other activated SCell are present.

Proposal 2: For UE supporting SSB-less operation, UE can skip SSB/CSI-RS based L1-RSRP measurement on SDL SCell, and the L1 RSRP measurement on collocated PCell can be leveraged.

Proposal 3: For UE supporting SSB-less operation, UE can skip L3 RSRP measurement on SDL carrier, and the L3 RSRP measurement on collocated PCC can be leveraged.

Proposal 4: For UE supporting SSB-less operation, two (A)TRS resources are needed for SDL SCell activation with the assumption that EPRE difference is smaller than or equal to 12 dB.

Proposal 5: RAN4 to discuss whether SCell activation procedure shall be completed within one SDL switch duration, or is allowed to be completed with multiple SDL switch durations.

Proposal 6: For UE not supporting SSB-less operation, the switching pattern would be also considered on top of the existing DRX and SMTC/RS periodicity when specifying L1/L3 measurement requirements on activated SDL carrier. The detailed requirements depend on the switching pattern specified in RAN1.

Proposal 7: For UE not supporting SSB-less operation, the switching pattern would be also considered on top of the existing DRX and measCycleSCell when specifying L3 measurement period on deactivated SDL carrier. The detailed requirements depend on the switching pattern in RAN1.

Proposal 8: For UE not supporting SSB-less operation, interruptions due to measurements on deactivated SDL are allowed with up to 0.5 % probability of missed ACK/NACK.

Proposal 9: For UE not supporting SSB-less operation, it is more challenging to complete the SCell activation procedure within one SDL switching duration. Or, SDL SCell activation procedure will take more SDL durations and further prolong the entire activation delay.

Proposal 10: For L1/L3 measurement on PCC, whether L1/L3measurement opportunities on PCC are colliding with the duration switching to SDL depends on switching pattern.

Proposal 11: The impact on UL transmit timing requirements can wait for RAN1 progress.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19

- R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2502877, LS on switching periods for low-low band switching, ATT

- R4-2502697, LS of RAN4 RRM agreements on LB CA via switching, Apple