3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503903

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on RRM impacts for SBFD operation

Source: 	Huawei, HiSilicon

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#114, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for RRM impacts for SBFD operation.

- CSI-RS based L1 measurement

- CSI-RS based L3 measurement

- Scheduling restriction

- RACH requirements

- MG and BWP switch

In this paper, we will provide our views on RRM impacts for SBFD operation.

Discussion

CSI-RS based L1 measurement

collision with UL

In [2], RAN1 provided the following reply related to collision between dynamic UL and L1 measurement.

The intention of the mentioned RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. The existing prioritization rules for serving-cell for collision Case 2 are as follows: - TS 38.213, Clause 11.1 “Slot configuration” “For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.” - TS 38.213, Clause 11.1.1 “UE procedure for determining slot format” “If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.”

Based on the RAN1 reply, dynamic UL is prioritized over semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. In this sense, the L1 measurement period requirements will be impacted due to collision with dynamic UL, i.e. the measurement occasion will be punctured, and the measurement period should be extended accordingly.

In previous RAN4 meetings, companies proposed two approaches to account for the puncture:

- Option 1: follow R17 con-MG approach, i.e. measurement period is scaled by a scaling factor which reflects the ratio of the punctured occasions over the total occasions.

- Option 2: follow R16 NR-U approach, i.e. measurement period is extended by the number of punctured occasions during the measurement period.

We suggest to go with option 2 because the puncturing is dynamic which is similar to LBT in NR-U, and it is difficult to define a window to count the number of total occasions. An example of the impact can be found in Table 9.5A.4.1-1 for SSB based L1-RSRP.

Table 9.5A.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB_CCA for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_CCA (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: L1=0 if higher layer parameter timeRestrictionForChannelMeasurement is configured. Otherwise, when DRX is not configured L1 is the number of SSBs not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, and when DRX is configured L1 is the number of DRX cycles in which at least one SSB is not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, where L1 ≤ L1max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(TReport, P * TSSB) if no DRX is used, Once per Max(TReport, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320ms, Once per P * TDRX if DRX cycle > 320ms. Note 3: L1max =7 for Max(TDRX,TSSB) ≤ 40ms assuming TDRX=0 for non-DRX, L1max =5 for 40ms < Max(TDRX, TSSB) ≤ 320ms, L1max =3 for TDRX > 320ms. |  |

Proposal 1: Measurement period for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamic UL following the R16 NR-U approach.

puncturing in frequency domain

In RAN4#114, following is agreed for the impact of CSI-RS puncturing in frequency domain.

- Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

First, we suggest to confirm existing measurement period for CSI-RS based L1 measurement apply for the case when CSI-RS resource is only within one DL subband, for both DU and DUD cases.

For DUD case with CSI-RS resource contained in two DL subbands, we support option 1.

First, the L1 measurement should be wideband, i.e. the measurement should reflect the channel status (RSRP or SINR) across 2 DL subbands on the OFDM symbol carrying the CSI-RS resource occasion. RAN1 has not defined any per DL subband reporting for L1 measurement. Besides, for aperiodic CSI-RS resource, UE anyway needs to get the wideband measurement result based on a single CSI-RS resource occasion according to the measurement and reporting timeline.

On option 2, if UE takes separate measurements for different DL subbands on different CSI-RS resource occasions, the combined measurement will not have a clear reference time point, and the usefulness of the measurement is questionable. Also, the processing efforts for punctured CSI-RS measurement is no larger than normal CSI-RS measurement. CSI-RS resource puncturing already occurs with SSB and CSI-RS on the same symbols, and no sequential measurement was assumed.

Proposal 2: Measurement period for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is not impacted by resource puncturing in frequency domain, with the assumption that UE is able to take wideband measurement across two DL subbands based on each CSI-RS resource occasion.

On the applicability of existing measurement accuracy requirements for DUD case with CSI-RS resource contained in two DL subbands,

- Case 1: it is straightforward that existing measurement accuracy requirements apply.

- Case 2: we suggest that existing measurement accuracy requirements apply because there are already enough resources for UE to meet the existing accuracy within one DL subband.

- Case 3: we understand there may be some degradation in the accuracy if the CSI-RS resource is not contiguous in frequency domain, and we may need to discuss L1-RSRP/L1-SINR and RLM/CBD/BFD separately due to different Es/Iot conditions and different minimum number of PRBs.

  - In our view, existing accuracy requirements can apply for L1-RSRP/L1-SINR if the number CSI-RS RBs in each DL subband is no less than 24.

  - In our view, for RLM/BFD/CBD, as the CSI-RS BW is limited (12 RBs) in each DL subband, and the measurement is supposed to work at Es/Iot as low as -15dB, we understand the accuracy degradation is not negligible especially with fading channel with large delay spread (in existing test cases the channel is TDL-C 300ns). We suggest to either relax the accuracy requirements (which is reflected by the SNR levels compared to Qin/Qout levels), or to improve the channel condition e.g. reducing the delay spread.

Proposal 3: For measurement accuracy for DUD case with CSI-RS resource in two DL subbands,

- For Case 1 and Case 2, existing accuracy requirements apply for L1-RSRP/L1-SINR and RLM/CBD/BFD,

- For Case 3, existing accuracy requirements apply for L1-RSRP/L1-SINR, provided that number CSI-RS RBs in each DL subband is no less than 24,

- For Case 3, relaxed accuracy requirements or propagation condition apply for RLM/CBD/BFD, provided that number CSI-RS RBs in each DL subband is no less than 12.

different symbol types

In [2], RAN1 provided the following reply related to collision between dynamic UL and L1 measurement.

- According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig..

Based on the RAN1 reply, CSI-RS based RLM/BFD/CBD measurement does not differ resource occasions on different symbol types, i.e. resource occasions on both SBFD and non-SBFD symbols will be used for the measurement, so the measurement period is not impacted by different symbol types.

Based on the RAN1 reply, CSI-RS based L1-RSRP/L1-SINR measurement will, however, be impacted. The valid symbol type will be configured in CSI-ReportConfig for each report configuration, and only CSI-RS resource occasions within the valid symbol types are used for CSI derivation.

Existing measurement period for L1-RSRP/L1-SINR is defined based on CSI-RS resource periodicity, and this means all resource occasions are counted for the measurement. Under SBFD operation, the measurement period should only count resource occasions on the valid symbol type, or in other words, the measurement period should be extended by excluding the resource occasions on the invalid symbol type. We suggest to follow the R16 NR-U approach to define the measurement period considering that 1) it is difficult to define the ratio of valid resource occasions over the total resource occasions since it depends on time domain SBFD configuration and periodicity and offset of the CSI-RS resource, and 2) we already use the NR-U approach to account for the impact of collision with dynamic UL.

Proposal 4a: Measurement period for CSI-RS based RLM/BFD/CBD is not impacted by resource occasions on different symbol types.

Proposal 4b: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

CSI-RS based L3 measurement

In RAN4#114, following is agreed for CSI-RS based L3 measurement.

- Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

We suggest to apply the RAN4#114 agreement also for DUD case. One clarification for DUD case is that there are two DL subbands, and the condition for the legacy requirements to apply should be that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols. If the CSI-RS resources are available in two DL subbands, it means some RBs in between the two DL subbands are punctured. As UE is not aware of SBFD operation in neighbour cell, UE will be measuring only noise and interference on those punctured RBs.

During the discussion in RAN4#114, the concern for DUD case was the required cell BW may be large for the existing requirements to apply, and in real world the condition may not be met for many cases or for typical cases. We understand this is not big issue.

In the following tables, we marked the cases where the transmission bandwidth configuration is enough for both CSI-RS and L1-SRS-RSRP requirements to apply, based on the assumption that 1) at least 48 RBs are required for one DL subband, and the BWs for the two DL subbands are almost same, 2) at least 24 RBs are required for the UL subband (for the L1-SRS-RSRP requirements to apply), and the BW can only be selected from the values in Table 5.3.2-1 and Table 5.3.2-2, and 3) some guard band are needed.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

Based on the analysis, both CSI-RS and L1-SRS-RSRP requirements can apply in many cases, and some are typical cases for SBFD operation, e.g. 30kHz SCS with 60/80/100MHz CBW. Of course, the conditions for both CSI-RS and L1-SRS-RSRP requirements to apply are not met for all applicable CBWs for SBFD, but this is normal in that most RAN4 requirements are defined based on certain side conditions, and they do not apply for all cases. We do not see strong reason to preclude L3 CSI-RS measurement requirements for DUD.

Another clarification on the RAN4#114 agreement is that the condition for the legacy requirements to apply should be that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols of the cell transmitting the CSI-RS. This is because whether a CSI-RS resource will be punctured in frequency domain depends on the SBFD configuration in the cell transmitting the CSI-RS, but not on the SBFD configuration in UE’s serving cell. We assume this was the common understanding when RAN4 made the agreement in RAN4#114, but it is better to make it clear to avoid possible confusion.

Technically, it may happen that SBFD configuration of neighbour cell is different from UE’s serving cell. For intra-frequency measurement, this would be a rare case, but anyway the existing scheduling restriction would apply because the measurement is same as in legacy from UE perspective. For inter-frequency measurement, measurement gap will be used, so symbols type of serving cell does not matter anyway.

Proposal 5: For L3 CSI-RS measurement, apply the RAN4#114 agreement also for DUD case and update the RAN4#114 agreement as follows.

UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS.

Scheduling restriction

One issue in scheduling restriction is whether to update the existing requirements to take DL/UL transition time into account. For example, RAN4 has defined scheduling restriction for TDD for L3 measurement but not for L1 measurement. The reason is that there is always guard period for the DL-to-UL transition, and UL transmission on UL symbols would never overlap with SSB reception for the serving cell in a reasonable configuration. However, there is no guard period between SBFD symbol and SSB symbol. UL transmission on SBFD symbol immediately after SSB symbol may collide with SSB reception due to TA.

Based on discussions in last meeting, we understand such a collision is already addressed by the following RAN1#116 agreement. Therefore, we suggest RAN4 not to update the existing scheduling restriction requirements to account for DL/UL transition time.

Agreement For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Taking L1 SSB measurement on SBFD symbols as example, not only the UL transmission on SSB symbols, but also the UL transmission on the symbol immediately after the SSB symbols would be dropped. Based on the note in the RAN1 agreement, we understand that this will be captured in RAN1 spec. In fact, dropping of additional UL transmission due to lack of sufficient transition time is not a specific issue for measurement, but it may also happen for other DL reception, e.g. UL transmission on the symbol immediately after an SBFD symbol where UE receives PDSCH will be dropped.

Considering that the impact of DL/UL transition will be addressed by RAN1 spec and it’s not limited to measurement, we suggest RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time. One exception case is the UE L1 CLI measurement as we discussed in our companion paper. As CLI measurement is not considered in RAN1 agreement for collision handling, RAN4#114 has agreed to define scheduling restriction to account for DL/UL transition.

Proposal 6: RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

Another issue raised up by some companies last meeting is the case when SSB for neighbor cell is not within DL subband of UE’s serving cell. In our understanding, such measurement must be inter-frequency because serving cell SSB is always within DL subband. Inter-frequency SSB may be measured outside gaps if UE supports R16 inter-frequency measurement without gap and the SSB is within UE active BWP. In this case, UE would measure SSB outside DL subband, and the question is whether UE can receive data within the DL subband at the same time.

In our view, this depends on whether UE Rx BW is assumed to only cover DL subband or the whole DL BWP on SBFD symbols. Technically, we understand UE may use a special RF filter on SBFD symbols whose Rx BW will only cover DL subband, so that the UE-to-UE CLI from the UL subband is suppressed. Although the benefit is there, we do not suggest to take this assumption for defining RAN4 requirements.

- It will increase the cost of the UE due to special RF filter.

- There will be interruptions due to RF retuning e.g. at transition between SBFD and non-SBFD symbols, or before and after CLI measurement in UL subband, or even at transition between UE’s Tx and Rx in consecutive SBFD symbols.

- It will incur large standardization efforts in RAN1 and RAN4 RF, as such RF retuning has not been considered in the transient period discussion in RAN1 and RAN4 RF.

Proposal 7: RAN4 not to define new scheduling restriction for the case when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots.

RACH requirements

In last meeting, some companies suggested that RACH requirements may be impacted by the RAN2 agreement about RO switching, i.e. UE is allowed to switch from SBFD RACH occasions to legacy RACH occasions after certain (configured) number of times of RACH attempt.

In our view, however, the RACH requirements in RAN4 are quite generic, e.g. the details of RO selection for RACH re-attempt are not specified in 38.133 but a reference to RAN2 spec is used. In this sense, we do not see there is any technical impact due to SBFD operation on the RACH requirements.

6.2.2.2.1.2 Correct behaviour when receiving Random Access Response The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble. ###### 6.2.2.2.1.3 Correct behaviour when not receiving Random Access Response The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [7].

So far, we have not identified any change needed in RAN4 requirements based on RAN1/2 agreements. Also, we understand RAN2 will not create a new clause for Random Access Resource selection in 38.321 for SBFD operation. RAN4 can further wait for RAN2 agreements/spec to decide whether editorial updates are needed to RACH requirements due to SBFD operation.

Proposal 8: RAN4 to wait for RAN2 agreements/spec to decide whether editorial updates are needed to RACH requirements due to SBFD operation.

MG and BWP switch

We do not see impacts of SBFD operation to BWP or CBW switch, as those procedures do not rely on measurement. However, some clarification may be needed because the requirements for DL (and UL) BWP switch are currently defined based on DL (and UL) transmission only on DL (and UL) slot. DL (and UL) transmission on SBFD symbols needs to be accounted.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where, …

Similarly, we do not see impacts of SBFD operation to the MG related requirements, but one clarification may be needed for the UL transmission immediately after MG. Currently it is only defined for UL slot, while with SBFD operation, the UL transmission can be on SBFD symbol.

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s), - when MGTA is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is other than 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the slot partially overlapped with measurement gap where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if for the UL transmission is less than the length of one slot; L=2 otherwise.

In RAN4#114, some companies mentioned that such clarification is not needed. Based on offline discussion, we understand the difference comes from terminology. There are two interpretations of term “UL slot”:

- Option 1: it includes only legacy UL slot, i.e. slot indicated as UL by tdd-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated. With this option, some clarification is needed in RAN4 spec.

- Option 2: it includes both legacy UL slot and SBFD slot, i.e. an SBFD slot can be an UL slot or a DL slot based on the determined link direction. With this option, no clarification is needed in RAN4 spec.

Our understanding is that option 1 will be used for RAN1 spec, so we have same proposal as RAN4#114.

Proposal 9: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Conclusions

In this paper we provided RRM impacts for SBFD operation.

Proposal 1: Measurement period for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamic UL following the R16 NR-U approach.

Proposal 2: Measurement period for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is not impacted by resource puncturing in frequency domain, with the assumption that UE is able to take wideband measurement across two DL subbands based on each CSI-RS resource occasion.

Proposal 3: For measurement accuracy for DUD case with CSI-RS resource in two DL subbands,

- For Case 1 and Case 2, existing accuracy requirements apply for L1-RSRP/L1-SINR and RLM/CBD/BFD,

- For Case 3, existing accuracy requirements apply for L1-RSRP/L1-SINR, provided that number CSI-RS RBs in each DL subband is no less than 24,

- For Case 3, relaxed accuracy requirements or propagation condition apply for RLM/CBD/BFD, provided that number CSI-RS RBs in each DL subband is no less than 12.

Proposal 4a: Measurement period for CSI-RS based RLM/BFD/CBD is not impacted by resource occasions on different symbol types.

Proposal 4b: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

Proposal 5: For L3 CSI-RS measurement, apply the RAN4#114 agreement also for DUD case.

UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS.

Proposal 6: RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

Proposal 7: RAN4 not to define new scheduling restriction for the case when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots.

Proposal 8: RAN4 to wait for RAN2 agreements/spec to decide whether editorial updates are needed to RACH requirements due to SBFD operation.

Proposal 9: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Reference

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple

- R1-2501560, Reply LS on CSI-RS measurement with SBFD operation, RAN1