3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503906

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on RRM requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	7.11.6

Document for:	Discussion

Introduction

RRM requirements for NTN in Ku band are discussed in RAN4#114, and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Scope of the RRM work

- RRM requirements

In this paper, we will provide our views on RRM requirements for NTN in Ku band.

Discussion

Scope of the RRM work

In [1] the following is agreed related to the scope of the RRM work.

Issue 1-1: RRM scope for NR NTN on Ku bands Agreement: - For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session. - Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE: - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology

One issue discussed in RAN4#114 without conclusion is whether fixed VSAT is in the scope of RRM work for Ku band. Some companies mentioned that fixed VSAT is a special case or subset of mobile VSAT, so it is naturally in the scope. However, based on R18 feature list, a VSAT is either fixed or mobile but cannot be both, so it is not reasonable to consider fixed VSAT is a special case or subset of mobile VSAT.

ntn-VSAT-MobilityType-r18 Indicates whether a VSAT UE is a mobile or fixed VSAT. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17.

We suggest RAN4 to clarify whether fixed VSAT is in the scope of RRM work for Ku band. This will impact at least the UL timing requirements which are currently specified separately for fixed and mobile VSAT, so it is a question whether requirements for fixed VSAT is applicable in Ku band or not.

Proposal 1: RAN4 to clarify whether fixed VSAT is in the scope of RRM work for Ku band.

Another issue is the support of 15kHz SCS. In the revised WID [2] from RAN#107, we have the following updates related to 15kHz SCS.

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE The objective of the performance part is to update as appropriate for Ku band: - RRM performance requirements and test cases - UE demodulation and CSI reporting requirements - For 15 kHz subcarrier spacing, the requirements are only applicable for GSO and low mobility scenarios. - Satellite access node demodulation requirements - Satellite access node conformance tests

It is clear that RRM work should include 15kHz SCS. The main impact in our view is the applicability of Table 7.1C.2-1 of 38.133. Requirements for both 15kHz and 30kHz should be applicable, and for 15kHz the requirements are only applicable for mobile VSAT UE served by GSO.

Proposal 2: RAN4 to confirm 15kHz SCS is in the scope of RRM work for Ku band.

RRM requirements

One issue raised up by companies in RAN4#114 is which RRM requirements defined in R18 for Ka band can be re-used and which need to be updated for Ku band, and whether new requirements are needed. In our view, except for UL timing requirements, all R18 requirements defined for Ka band can be re-used for Ku band as baseline. The reason is that

- The mobility and measurement requirements are agnostic to SCS, so consideration of FR1 SCS does not require updates to these requirements.

- The mobility and measurement requirements have already accounted for fixed VSAT served by NGSO, and they can be re-used for mobile VSAT served by NGSO considering the dynamic nature of NGSO.

Of course, we are open to discuss if any update is needed based on the difference between VSAT for Ku and Ka band. For example, some companies mentioned that the Rx beam steering time for inter-satellite HO may be different for Ka and Ku bands. We are open to further discussion if such updates are justified by RF.

Proposal 3: Except for UL timing, R18 requirements for Ka band are re-used for Ku band as baseline.

For UL timing requirements, following is agreed in [1].

Issue 1-3: UL transmit timing requirement Agreement: - RAN4 to define timing accuracy requirements for case 1, 2: - For FR1-NTN numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - RAN4 to define timing accuracy requirements for case 3: - For FR2-NTN numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - The endorsed CR with 7.5 Ts under the same side conditions as Rel-18 are considered. - RAN4 to define timing accuracy requirements for case 4: - For FR2-NTN numerology, FFS existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - The endorsed CR with 7.5 Ts is considered. FFS whether the same or different side condition from Rel-18 can be used.

The open issue is for case 4 (mobile VSAT UE served by NGSO in FR2-NTN numerology). In our view, existing requirements in Table 7.1C.2-3 can be re-used for case 4. To define the timing requirements for mobile VSAT served by NGSO, we need to determine the assumed GNSS error and satellite location error.

- Based on the latest agreement for R18 NTN from RAN4#114, GNSS error is assumed to be same for fixed and mobile UEs, but there would be additional side conditions related to GNSS for the mobile UE to achieve this accuracy.

- Satellite location error may be different for GSO and NGSO. In our understanding, satellite location error should be same as that for GSO, provided that the ephemeris information from the satellite control system is accurate and timely updated (which has also been the assumption so far).

Some companies mentioned that additional side condition related to the availability and accuracy of the ephemeris info may be needed. This has in fact been discussed in R18, and it was concluded no additional side condition is needed because the ephemeris info needs to be updated at most every 10.24s.

Based on above analysis, we suggest to re-use the timing requirements of mobile VSAT is served by GSO for mobile VSAT served by NGSO.

Proposal 4: For UL timing, existing requirements in Table 7.1C.2-3 are re-used for case 4.

As the requirements for mobile VSAT served by NGSO are introduced as part of Ku bands WI, they should apply to Ku bands only. Besides, there is note in the WID that “RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only”. We understand it means a UE is only required to meet one of the requirements for FR1 and FR2 numerology. Which requirement UE should meet depends on which of the FR1-NTN operation and FR2-NTN operation UE is supporting.

Proposal 5: The timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. A UE is required to meet requirements for FR1-NTN operation only, or for FR2-NTN operation only.

Conclusions

In this paper we provided our views on RRM requirements for NTN in Ku band.

Proposal 1: RAN4 to clarify whether fixed VSAT is in the scope of RRM work for Ku band.

Proposal 2: RAN4 to confirm 15kHz SCS is in the scope of RRM work for Ku band.

Proposal 3: Except for UL timing, R18 requirements for Ka band are re-used for Ku band as baseline.

Proposal 4: For UL timing, existing requirements in Table 7.1C.2-3 are re-used for case 4.

Proposal 5: The timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. A UE is required to meet requirements for FR1-NTN operation only, or for FR2-NTN operation only.

Reference

- R4-2502590, WF on RRM requirements for NR_NTN_Ku_bands, Apple