# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503915

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.23.3.1

Source:	China Telecom

Title:	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

General issues

Status in the summary [2]:

- Issue 1-1-2: Rx beam - Note: P2 of ZTE R4-2500911 is not listed for open issue discussion as it is more like an observation. - Proposals - Proposal 1a (LGE): - RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement of RAN1 agreements can be considered. - Proposal 1b (ZTE, CATT): - directly refer to RAN1 spec instead of repeating all details in RAN spec. - Proposal 2 (Nokia): - The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period. - Recommended WF - Agree on P2: The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period. - Further discuss P1a and P1b in next meetings.

For Rx beam, it’s preferred to directly refer to RAN1 spec instead of repeating all details in RAN spec.

Proposal 1: Directly refer to RAN1 spec instead of repeating all details in RAN spec.

Status in the summary [2]:

- Issue 1-1-6: Measurement restriction - Note: P11 of ZTE R4-2500911 is not listed for open issue discussion as it was already agreed in RAN4#113 (Issue 1-1-9 in WF R4-2420097) - Proposals for collision between L1 CLI measurements and L1/L3 RRM measurements - Proposal 1 (Nokia, E///, MTK): - Legacy measurement restriction applies, i.e. do not consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel - Proposal 1a (HW): - For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements are re-used as baseline. - Proposal 2a (CATT): - Consider the exception cases where UE can receive CLI resources and other resources at the same time - Proposal 2b (HW): - For L1-CLI-RSSI Method #1, measurement restriction applies only when UE cannot perform CLI measurement and other measurement at same time. - Proposals for collision between different CLI measurements - Proposal 1a (Apple): - FFS if it should be clarified in the RAN4 specification that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. - Proposal 1b (Nokia): - RAN4 shall specify that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. - Proposal 2a (Nokia): - L1-CLI-RSSI measurement requirements do not apply when the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with configured SRS-RSRP for method #2 - Proposal 2b (QC): - RAN4 to discuss how to handle the collisions of L1-CLI-RSSI resources with L1-SRS-RSRP resources configured for L1 CLI measurements, i.e., which one of the two should the UE prioritize? - Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements - For L1-SRS-RSRP, measurement requirements for L1-SRS-RSRP do not apply if L1-SRS-RSRP measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - For L1-CLI-RSSI Method #1, measurement requirements for L1-CLI-RSSI do not apply if L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, FFS whether to consider exception case - Option 1: do not consider exception case, i.e. measurement requirements for L1-CLI-RSSI do not apply - Option 2: consider exception case, i.e. measurement requirements for L1-CLI-RSSI apply when UE can perform L1-CLI-RSSI measurement and other measurement at same time. - For L1-CLI-RSSI Method #3, - If the measurement timing is same as L1-SRS-RSRP, as baseline, measurement restriction requirements are same as L1-SRS-RSRP. - If the measurement timing is same as L1-CLI-RSSI Method #1, as baseline, measurement restriction requirements are same as L1-CLI-RSSI Method #1. - FFS whether any adaptation is needed on top of baseline. - Recommended WF for collision between different CLI measurements - FFS whether to capture in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. - FFS how to handle collision between L1-SRS-RSRP and L1-CLI-RSSI measurement resources - Option 1: prioritize L1-SRS-RSRP measurement - Option 2: prioritize L1-CLI-RSSI measurement - Option 3: other

For collision between L1 CLI measurements and L1/L3 RRM measurements, we prefer legacy measurement restriction applies, i.e. do not consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

Proposal 2: For collision between L1 CLI measurements and L1/L3 RRM measurements, it’s preferred to apply legacy measurement restriction, i.e., do not consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

For collision between different CLI measurements, since RAN1 agreement is that the UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol, we propose to capture in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Proposal 3: For collision between different CLI measurements, it’s proposed to capture in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Status in the summary [2]:

- Issue 1-1-7: Measurement capability - Proposals - Proposal 1 (ZTE): - Per NW configuration, M wideband results could be reported by UE and each of them corresponding to a CLI measurement resource, the upper bound of M is 4. - Proposal 2 (MTK): - RAN4 not to discuss measurement capability requirements unless requested by other WGs. - Recommended WF - RAN4 not to discuss measurement capability requirements unless requested by other WGs.

In our views, the measurement capability is designed by RAN1, so RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Proposal 4: RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Status in the summary [2]:

- Issue 1-1-8: Report mapping - Proposals - Proposal 1 (Nokia): - Re-use the existing Rel-16 SRS-RSRP report mapping for L1-SRS-RSRP absolute power reporting. - Introduce a new report mapping table for L1-SRS-RSRP differential power reporting. Consider the same report mapping table as for SS-RSRP and CSI-RSRP differential reporting. - Re-use the existing Rel-16 CLI-RSSI report mapping for L1-CLI-RSSI absolute power reporting. The same report mapping table is applicable to both Method #1 and Method #3. - Introduce a new report mapping table for L1-CLI-RSSI differential power reporting. Consider the same report mapping table as for SS-RSRP and CSI-RSRP differential reporting. - Proposal 2 (HW, MTK): - RAN4 not to discuss report mapping for L1 CLI measurement unless triggered by other WG. - Recommended WF - RAN4 not to discuss measurement report mapping unless requested by other WGs.

For report mapping, we prefer RAN4 not to discuss measurement report mapping unless requested by other WGs.

Proposal 5: RAN4 not to discuss measurement report mapping unless requested by other WGs.

Status in the summary [2]:

- Issue 1-1-9: Requirement applicability - Proposals - Proposal 1 (LGE): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - SRS-RSRP measurement resource is fully confined within BW of UL sub-band - CLI-RSSI measurement resource is configured within DL sub-band - Proposal 2 (Nokia, CATT, HW, MTK, QC): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP - Proposal 3 (ZTE): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED: - For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP. - For CLI-RSSI resource - For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP. - For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration. - No gap is needed for UE-to-UE CLI measurement - Recommended WF - The proposals are close to each other, further discuss the following issues and come up with combined the wording - Whether we define L1 CLI measurement resources to be confined within UL or DL subband or just DL active BWP - Whether we define L1 CLI measurement as intra-frequency

For requirement applicability, we think R16 applicability can be reused, the L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

- when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

- when CLI-RSSI measurement resource is configured within active BWP.

Proposal 6: The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

  - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

  - when CLI-RSSI measurement resource is configured within active BWP.

L1-SRS-RSRP

Status in the summary [2]:

- Issue 1-2-3: Measurement accuracy - Proposals - Proposal 1 (Apple, CTC, MTK): - RAN4 need to define the measurement accuracy requirements for L1-SRS-RSRP based on the simulation results at agreed side condition and measurement sample. - RAN4 not to reuse legacy UE-to-UE CLI accuracy requirements directly. - Proposal 2a (CMCC): - Existing L3-SRS-RSRP accuracy requirements can be applied to L1-SRS-RSRP (based on timing offset Case 2). - Proposal 2b (vivo): - the accuracy of L1-SRS-RSRP should be kept (and the corresponding side conditions of L1-SRS-RSRP could be relaxed if necessary). - Proposal 3 (CMCC): - Same accuracy requirements are applied for both the case with 1 sample and the case with 3 sample. - Recommended WF - Further discuss the accuracy requirements after measurement period and side conditions are settled.

For L1 UE-to-UE CLI measurement accuracy requirements, it’s proposed to define measurement accuracy requirements for L1-SRS-RSRP. Since Rel-16 CLI measurement requirements are based on L3 framework while Rel-19 CLI measurement requirements are based on L1 framework, it’s proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly. The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

Proposal 7: The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

L1-CLI-RSSI

Status in the summary [2]:

- Issue 1-3-3: Measurement period - Proposals - Proposal 1a (ZTE): - For Method#1, for the case of two DL subbands configured, no matter whether puncturing happens, no impact on UE processing since all non punctured CSI-RS resource are within active DL BWP and half-duplex operation at UE side. - Proposal 1b (HW): - For Method#1, measurement delay requirements for L1-CLI-RSSI are agnostic to measurement resource types (type#1 or type#2) - Proposal 2 (CATT, E///, HW): - Measurement period for Method #3 is same as Method #1, i.e. based on single sample. - Recommended WF - Agree on P1a and P1b: For Method #1, measurement period is agnostic to measurement resource types, i.e. 1 sample for both type#1 and type#2. - Agree on P2: measurement period for Method #3 is same as Method #1, i.e. 1 sample.

For L1-CLI-RSSI measurement period, support to define measurement period requirements based on 1 sample.

Proposal 8: For L1-CLI-RSSI measurement period, support to define measurement period requirements based on 1 sample.

- Conclusion

The contribution presents our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation, with the following proposals:

Proposal 1: Directly refer to RAN1 spec instead of repeating all details in RAN spec.

Proposal 2: For collision between L1 CLI measurements and L1/L3 RRM measurements, it’s preferred to apply legacy measurement restriction, i.e., do not consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

Proposal 3: For collision between different CLI measurements, it’s proposed to capture in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Proposal 4: RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Proposal 5: RAN4 not to discuss measurement report mapping unless requested by other WGs.

Proposal 6: The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

  - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

  - when CLI-RSSI measurement resource is configured within active BWP.

Proposal 7: The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

Proposal 8: For L1-CLI-RSSI measurement period, support to define measurement period requirements based on 1 sample.

- References

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114, 17 – 21 February, 2025.

- R4-2500536, Topic summary for [114][222] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114, 17 – 21 February, 2025.