3GPP TSG-RAN WG4 Meeting #114bis	R4-2503923

Wuhan, China, Apr 7th – Apr 11th, 2025

Source:	Nokia

Title:	Discussion on UE CA Demodulation performance requirements for LessThan 5MHz

Agenda item:	7.3.4

Document for:	Discussion

Introduction

In addition to already agreed requirements for <5MHz in Rel-18, a Rel-19 WID [1] has been agreed for Rel-19 which has the following objective:

### 4.1 Objective of SI or Core part WI or Testing part WI - Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In addition, the objective of the performance part WI indicates for RAN4 to specify necessary performance requirements [1]:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify necessary performance requirements (RAN4)

RAN#107 has decided, that RAN4 shall also consider, whether additional CA performance requirements are to be added to TS38.101-4, since CA was not considered in the Rel-18 work.

Discussion

Introduction of PDSCH CA performance requirements

In Rel-18 it was decided to not introduce PDSCH CA requirements for <5MHz, as RAN4 RF did not consider CA. For the Rel-19 WID, CA is specifically considered. Hence there is a need for RAN4 to consider, whether requirements can be introduced for lessthan 5MHz with CA. 	
Specifically, it is highlighted in the WID that one example for a band combination would be 3MHz in band n100 and 5 or 10MHz in band n101.

As the bands allocated with 3MHz are only with FDD, we can exclude defining PDSCH CA demodulation requirements for 3MHz with TDD from the outset.

All bands supporting 3MHz only have 3MHz defined for SCS of 15kHz

CA is now in focus for Lessthan5MHz, hence PDSCH CA performance requirements for 3MHz CBW needs to be considered.

For 3MHz CBW only FDD and SCS=15MHz is relevant for requirement definition.

Introduce PDSCH CA requirements for 3MHz CBW with FDD and SCS=15kHz.

The existing PDSCH CA requirements in 38.101-4 [2] section 5.2A are contained in tables defining single carrier performance for different configurations. To enable tests including 3MHz, tables related to the potentially agreed 3MHz configurations will need to be updated.

There are only two meetings for finalizing the work, hence there will be no time for additional simulation work. However, since the difference in reference values (SNR) in the existing tables between 5Mhz and 10MHz bandwidth is small (<=0.3dB) we propose to use the same reference value for 3MHz as for 5MHz. This might relax the requirements by a small amount, but not enough to mandate new simulations. Appendix A shows an example of a change.

There is not enough time left in the work item to perform new simulations, however since the difference in existing requirements between 5MHz and 10MHz CBW is <= 0.3dB, it can be argued that the 5MHz reference values can be used also for 3MHz.

For all new added PDSCH CA performance requirements for 3MHz, use the same reference value as for 5MHz from the same table.

Conclusion

In this paper we have provided our view on the topic adding CA UE performance requirements for the topic of Lessthan 5MHz.

The following Observations and Proposals were made:

- CA is now in focus for Lessthan5MHz, hence PDSCH CA performance requirements for 3MHz CBW needs to be considered.

For 3MHz CBW only FDD and SCS=15MHz is relevant for requirement definition.

- Introduce PDSCH CA requirements for 3MHz CBW with FDD and SCS=15kHz.

There is not enough time left in the work item to perform new simulations, however since the difference in existing requirements between 5MHz and 10MHz CBW is <= 0.3dB, it can be argued that the 5MHz reference values can be used also for 3MHz.

For all new added PDSCH CA performance requirements for 3MHz, use the same reference value as for 5MHz from the same table.

- References

- RP-241622 - Revised WID: NR channel BW less than 5MHz for FR1 Phase 2

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

Appendix A

The following shows an example of one tables with added 3MHz bandwidth requirements for CA to TS38.101-4. Note that the proposal here is to re-use the SNR values from 5MHz. If reusing the SNR can be accepted for all impacted tables no new simulations are needed.
For each added row, there will be a need to define additional corresponding reference channel in section A.3.

<start of change>

### 5.2A.2	2RX requirements

#### 5.2A.2.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.2.1-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.2.1-1 ~ Table 5.2A.2.1-3, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.2.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 35 | R.PDSCH.1-10.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.9 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 45 | R.PDSCH.1-10.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.4 |