3GPP TSG-RAN WG4 Meeting #114bis 	    R4-2503929

Wuhan, China, 07 – 11 April 2025

Agenda Item:	7.21.2.1

Source: 	Ericsson

Title:	On requirements for RRM measurement prediction for AI/ML based mobility

Document for:	Approval

# Introduction

In this contribution issues related to requirements for RRM measurement prediction for AI/ML based mobility are discussed. The discussion presented in this paper aim to resolve the FFS items agreed in previous meeting and present Ericsson’s view on other issues relevant from the requirement point of view for the AI/ML based mobility.

# Discussion

## Beam Level Measurement Prediction

RAN4#112bis discussed the performance metric/KPIs to be considered by RAN4 for AI/ML based mobility requirement. The discussion concluded by reaching to the agreement below, where L1 beam-level measurement prediction at point A1 and L3 beam-level measurement prediction at point E in Figure 1 were agreed FFS [1].

Agreement from RAN4#112bis:

- RAN4 to first study the impacts on requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

  - FFS L1 beam-level (Point A1)

  - FFS L3 beam-level (Point E)

Figure 1.  Measurement model as defined in TS 38.300 [2].

Studying and evaluating the potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover is one of the objectives for this study item. For L3-based mobility, UE can also be configured to report L3 filtered beam level measurements. From the requirement point of view, it is relevant to study the impact on RAN4 requirements due to the prediction of L3 filtered beam level measurements at point E. Considering the scope of the study item is to reuse the existing mobility framework for L3-based mobility, where the L1 filtered beam level measurement at point A1 is not reported by the UE, in our understanding the impact of the L1 beam level prediction at point A1 should not be the focus of study in RAN4. The focus should rather be on the beam level prediction at point E and its impact on the RAN4 requirements.

Observation 1: Objective of the study item is to study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover.

Observation 2: The scope of the study item is to reuse the existing mobility framework for L3-based mobility.

Proposal 1: RAN4 to not consider the impact of L1 beam level measurement prediction at point A1, on RAN4 requirements unless agreed to be considered for AI/ML based mobility by RAN2.

Proposal 2: RAN4 to consider the impact of L3 beam level measurement prediction at point E, on RAN4 requirements.

## Temporal Prediction

Two cases, as elaborated in Figure 2 and Figure 3, are being considered in RAN2 evaluations for the temporal prediction of RRM measurements. In both cases, measurements performed on a cell is used by the UE to predict the RRM measurements for the future time instances for the same cell.

Temporal prediction case-A:

In temporal prediction case-A, UE performs measurement within the observation window (OW) and uses these measurements to perform prediction of measurements valid for the prediction window (PW). An OW consists of one or more measurement occasions and within a PW there can be one or more than one prediction occasions. A high-level overview of OW, measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 2. The prediction occasions or PW starts after the completion of the measurement occasions or OW in temporal prediction case-A.

Figure 2.  Example elaboration of temporal prediction case-A.

Temporal prediction case-B:

Like the temporal prediction case-A, an OW consists of one or more measurement occasions and within a PW there can be one or more prediction occasions in temporal prediction case-B. The UE is expected to perform measurements in measurement occasions and predict the measurements valid for the prediction occasions inside the prediction window. Unlike temporal prediction case-A, within the OW the UE can skip performing measurements on some of the measurement occasions in temporal prediction case-B. A high-level overview of OW, measurement occasions within OW, skipped measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 3. Like temporal prediction case-A, the prediction occasions or PW starts after the completion of the measurement occasions or OW in temporal prediction case-B.

Figure 3. Example elaboration of temporal prediction case-B.

In RAN4#112bis meeting prioritization of scenarios for cell level RRM measurement prediction was discussed. Based on the discussions, it was agreed to align prioritization of scenarios in RAN4 with the prioritization of the scenarios in RAN2, where two of the prioritized scenarios relate to temporal prediction (scenario 2 for temporal prediction case-B and scenario 4 for temporal prediction case-A) of RRM measurements [1].

Agreement from RAN4#112bis [1]

- RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

For each of the prioritized scenarios, the following sub-cases are being considered in RAN2 evaluations:

- Sub-case 1: To predict beam level results, then generate cell level results based on the predicted beam measurements.

- Sub-case 2: To directly predict cell level results based on cell level measurements.

- Sub-case 3: To directly predict cell level results based on beam level measurements.

Simulation results submitted by the participating companies for the intra-frequency intra-cell scenarios were analyzed in RAN2#127bis meeting and the following were concluded.

Agreements from RAN2#127bis [3]

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors.

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy. RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy.

Agreements from RAN2#128 [4]

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet.

- We can capture in the TR how the skipping pattern affects the performance.

From the RAN2 agreements, based on the evaluation of the simulation results, it can be observed that the duration of OW or the number of measurements performed inside the OW has an impact on the accuracy of measurements predicted by the UE to be valid for the prediction instances inside the PW for both temporal prediction cases and the prediction accuracy in temporal prediction cases is highest in sub case 2 for temporal prediction case-A. For temporal prediction case-B, it was observed that the prediction accuracy decreases when MRRT is increased and for a fixed MRRT different skipping pattern can provide different prediction performances.

In the last RAN4 meeting it was agreed to consider the impact of UE speed, MRRT, OW, PW and the number of measurements performed during the OW on the prediction accuracy requirements for temporal predictions. The agreement however does not consider the impact of measurement skipping pattern on the prediction accuracy requirements for the temporal prediction case-B for a fixed MRRT. This issue shall be considered for further study in RAN4.

Observation 3: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW.

Agreement from RAN4#114 [5]

- RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

Proposal 3: RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B.

## Frequency Prediction

Frequency domain prediction is another type of prediction considered for AI/ML based mobility. Frequency domain prediction refers to the predictions of measurements of cells in one frequency based on the measurements performed by the UE on cells in another frequency. The scope of inter-frequency prediction is limited to the prediction of RRM measurements for the carriers belonging to the same frequency region.

Simulation results submitted by the participating companies for frequency prediction was analyzed in RAN2 in RAN2#128 meeting and concluded the following:

Agreements from RAN2#128 [4]

- For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations.

- For inter-frequency, cluster as input (i.e. measurements from different cells as inputs) can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster-based approach.

- In cluster approach the model takes measurements from more than one cell as inputs.

Based on the RAN2 agreements above, the following observations can be made:

Observation 4: Higher the correlation coefficient between the carriers of the input and output cells, higher is the accuracy of RRM measurements predicted for the output cell in frequency prediction case (scenario 3 in prioritized scenario).
 

Observation 5: Clustering approach (measurement from multiple spatial cells is used for RRM measurement prediction of an output cell) provides higher prediction accuracy in comparison to the case where the measurement from single cell in one frequency is used as an input to predict the RRM measurement for an output cell in another frequency.

To ensure reliable RRM measurements are predicted and reported by the UE to the network in scenario 3, RAN4 needs to identify an optimum distance in terms of frequency between the measured and the predicted cells such that the requirements for frequency prediction is defined to be met by the UE for the output cell provided the measured cell(s) and output cell(s) are within the identified frequency distance within a frequency region. Moreover, from the requirements point of view the following issues also need to be studied:

- Requirements when the measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the cell in another carrier frequency.

- Requirements when the measurement from a group of cells in one carrier frequency is used by the UE as input to predict the RRM measurement for the cell(s) in another carrier frequency(ies).

Proposal 4: RAN4 to consider the impact of frequency relationship between the measured and predicted cells on the performance of frequency domain prediction of RRM measurement for mobility.

Proposal 5: RAN4 to study requirement aspects for the following cases:

- When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR cell(s) in another carrier frequency(ies).

## RLF Prediction

In RAN4#112bis meeting it was agreed to start the discussion on RLF prediction use case from RAN4#114bis meeting. RLF prediction was discussed in RAN2#129 meeting where it was agreed to not study the RLF prediction as a part of Rel. 19 study item. RAN4 discussions should therefore also be aligned with the RAN2 agreement such that the impact on requirement for RLF prediction is not discussed in RAN4 in Rel. 19 study item.

Agreement from RAN4#112bis [1]

- RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2.

- The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025).

- The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

Agreement from RAN2#129 [6]

- RLF prediction will not be studied in Rel-19.

- Further RRM prediction simulations are not expected.  For next specification general impact study phase, we will consider spatial, temporal, and frequency domain cell level prediction.   FFS if L3 beam level needs to be considered.  For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

- Continue with generalization across cell configurations for RRM prediction

- Continue with measurement events

Observation 6: RLF prediction is out of Rel. 19 study item scope.

# Summary

In this contribution issues related to requirements for RRM measurement prediction for AI/ML based mobility are discussed. The observations and proposals below summarize the discussion presented in this paper.

Observation 1: Objective of the study item is to study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover.

Observation 2: The scope of the study item is to reuse the existing mobility framework for L3-based mobility.

Proposal 1: RAN4 to not consider the impact of L1 beam level measurement prediction at point A1, on RAN4 requirements unless agreed to be considered for AI/ML based mobility by RAN2.

Proposal 2: RAN4 to consider the impact of L3 beam level measurement prediction at point E, on RAN4 requirements.

Observation 3: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW.

Proposal 3: RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B.

Observation 4: Higher the correlation coefficient between the carriers of the input and output cells, higher is the accuracy of RRM measurements predicted for the output cell in frequency prediction case (scenario 3 in prioritized scenario).

Observation 5: Clustering approach (measurement from multiple spatial cells is used for RRM measurement prediction of an output cell) provides higher prediction accuracy in comparison to the case where the measurement from single cell in one frequency is used as an input to predict the RRM measurement for an output cell in another frequency.

Proposal 4: RAN4 to consider the impact of frequency relationship between the measured and predicted cells on the performance of frequency domain prediction of RRM measurement for mobility.

Proposal 5: RAN4 to study requirement aspects for the following cases:

- When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR cell(s) in another carrier frequency(ies).

Observation 6: RLF prediction is out of Rel. 19 study item scope.

# References

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).

- TS38.300, Stage 2: NR and NG-RAN Overall Description, 3GPP.

- Chair’s note from RAN2#127bis, RAN2 Chair (InterDigital).

- Chair’s note from RAN2#128, RAN2 Chair (InterDigital).

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).

- Chair’s note from RAN2#129, RAN2 Chair (InterDigital).