3GPP TSG RAN WG4 Meeting #114bis	 R4-2503934

Wuhan, China, 07 Apr. 2025 – 11 Apr. 2025

Agenda Item:	7.22.3.1

Source: 	Ericsson

Title:	Discussion on UE initiated BM requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for NR MIMO Phase 5.

# Discussion

Event triggered measurement requirements

Event triggered measurement delay:

For defining event triggered measurement reporting requirements, RAN4 needs to discuss and decide on the start point, end point, measurement period, the event evaluation time, and the measurement report preparation time.

Regarding the start point and end point, following was agreed in last meeting [R4-2420096].

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a first PUCCH triggered by the event.

Further, regarding the measurement period definition following WF is agreed in last meeting.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

  - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4

    - TReport = 0 or be removed from the measurement period

    - N: Reuse the legacy definition

    - M: FFS pending RAN1 further agreement

    - TSSB (taking SSB as example)

      - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new

      - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1

        - Option A: Max{TSSB_current, TSSB_new}

        - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

        - Option C: others are not precluded

One open issue in the measurement reporting delay is if RAN1 decide to support different SSB periodicity between current and new beams, whether to use the maximum of the TSSB of current beam and TSSB of new beam or not. We think it is not reasonable to use the maximum value because the UE should evaluate the event after obtaining measurement result of each SSB burst occasion when they have different periodicity. Using the maximum value would lead to the UE assessing the event only during the longest measurement intervals, potentially causing delays in measurement reporting.

For example, if the current beam has a SSB period of 20 ms and the new beam has a SSB period of 80ms, using the maximum value would result in the UE evaluating or measuring the event only every 80ms. This could lead to delays in measurement reporting and potentially leading to late beam switch and there by resulting in beam failure and there by resulting in link recovery. Instead, by not using the maximum value, the UE can evaluate the event every 20 ms, ensuring more timely reporting and efficient beam management.

To further clarify our concern, let's consider an example where the current beam has an SSB period of 20 ms, and the new beam has an SSB period of 80 ms. Suppose the counter configured by the network is 80 ms, and M=3. In the scenario where the current beam triggers the event due to its quality falling below that of the new beam (as illustrated in Figure 1), if max value is not used, the UE will send a measurement report at point C (as shown in figure). However, with using max value, it is uncertain whether the UE will send a measurement report at point D or not. Our understanding is that when the number of measurement samples for the measurement report is 1, the measurement sampling time equals the event-triggered reporting time.

- When the RS periodicity for current beam and new beam is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

Measurement window/counter consideration in UE initiated beam report:

We would like to point that RAN1 is still discussing how the counter or timer is considered for the UE initiated beam report. It is unclear to us how this might impact the RAN4 requirements design. Hence, we would like to wait for RAN1 conclusion on this issue. RAN1 agreement from last meeting is shown below for reference.

RAN1#120 agreement: Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. - Note: Other options are not precluded.

- RAN4 to wait for RAN1 progress on consideration of counter or timer in the UE initiated beam reporting in RAN4 requirements.

Other components:

Since the L1-RSRP from the event triggered report is used for beam management, UE should send the report as soon as event condition is met. Event triggering conditions can be changed as fast as the measurement occasions. Hence, UE shall evaluate the event after every measurement occasion to check if any of the configured event is met. We understand that RAN1 is also discussing this issue, and we believe RAN1 should decide on the same.

- Event evaluation period for a SSB or CSI-RS can be decided in RAN1.

Once the event is triggered, the UE must send an indication on the first UL channel to notify the NW that the event has occurred, referred to as "TTime to First UL Channel." After the event is triggered, the UE may require time to prepare the new UCI for transmission on the first UL channel. Typically, the preparation time for the PUCCH or PUSCH is defined in RAN1 specification, but it is crucial to account for this in the RAN4 requirement design. We believe this is important because the UE might miss the first occasion of first UL channel if the interval between the RS that triggers the event, and the first UL channel grant is insufficient for the UE to prepare and send the new UCI on the first UL channel.

- RAN4 to agree TTime to First UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time. Where, Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1.

Inter-cell scenario for UEIBM

Whether to consider intra-cell and inter-cell scenario for defining event triggered reporting for CSI-RS was discussed in last meeting. We provide our view on the inter-cell scenario in this section.

In Rel-15, SSB based L1-RSRP requirements were defined for intra-cell scenario where all the SSB in a cell are associated with single PCI. In Rel-17, RAN1 introduced inter-cell beam management and inter-cell multi-TRP scenario where SSB of the cell can be associated with different PCI, where the SSB with different PCI are typically transmitted from different TRP associated with same serving cell. In Rel-17, RAN4 defined L1-RSRP measurement requirements for inter-cell multi-TRP scenario where one TRP can be associated with serving cell PCI and other TRP can be associated with different PCI than the serving cell PCI. However, in that WI, only SSB was considered, and CSI-RS based inter-cell L1-RSRP measurements were not considered during that WI.

CSI-RS based L1 measurement requirements are defined in Rel-15 and CSI-RS is assumed to be QCL’ed with SSB. As per the applicability of CSI-RS requirements shown below, SSB which is QCLed with CSI-RS can be intra-cell and inter-cell.

For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set. For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON.

One of the first agreement RAN1 made for this work item is shown below. Since supporting intra-cell and inter-cell scenario was one of the first agreement by RAN1, it signifies the importance of considering both the scenarios.

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

Hence, we propose

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

When UE is configured with CSI-RS measurements in inter-cell scenario, SSB associated with CSI-RS may be from different TRP (different PCI) and in FR2, they may need to be received with different QCL Type D. Since multi-Rx may not always be assumed, UE may be able to measure only one TRP CSI-RS at a time when the CSI-RS from different TRP are overlapping in time. RAN4 may need to define sharing or scaling rules for the inter-cell CSI RS measurements. One may argue that SSB based inter-cell L1-RSRP measurements are defined during a dedicated WI in Rel-17 and may contain big workload to define CSI-RS based inter-cell L1-RSRP measurement requirements. We think that RAN4 can reuse the principles of sharing agreed for SSB based measurements to derive the requirements for CSI-RS based inter-cell L1-RSRP measurements.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

Impact on unified TCI state switching delay

In last meeting there was a discussion on whether to define new TCI state switching delay or existing requirements are sufficient for the unified TCI state switching based on event triggered reporting. To analyze the requirements, we first look at the events agreed in RAN1 to discuss whether there is an impact on the unified TCI state switch delay.

RAN1 agreed on following events (event 1 and 7 are working assumption)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

  - M is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

The intention or use case of each of the event is provided below

- Event-1: when the quality of the current beam is lower than certain threshold NW may request the aperiodic report and schedule a beam change.

- Event 2: when quality of the new beam is better than the current beam, NW can trigger the beam switch based on the report from the UE.

- Event-7: When a new beam which was not in the active TCI state list is better than beams in active TCI state list, based on the UE report, NW update the active TCI state list.

Currently RAN1 supports two alternatives for unified TCI state activation. Based on the use case for which the events are defined, we think the RAN1 defined events support the existing TCI state activation framework listed below.

Alternative 1:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE activating some of the TCI states from the RRC configured TCI states

- Step 3: DCI indicating one of the MAC CE activated TCI states.

Alternative 2:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE directly activating a TCI state from the RRC configured TCI states

In some implementations, after receiving the measurement report when the event 2 trigger conditions are met, if a new beam which was not in the active TCI state list is better than the current beam, then NW may decide to switch the current beam to the new beam. In this scenario, Alternative 1 may cause extra delay for switching the current beam to best beam. However, we think that this can be supported with Alternative 2 as described above. Hence, we do not see a need for new requirements definition.

- Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

UE capability for UE IBM

UE IBM is for the MIMO and MIMO generally is defined for single serving cell. Even for the ICBM or multi-TRP, the TRPs are considered as a single cell. For a single cell, as per our understanding RAN4 generally does not define intra-frequency or inter-frequency. We agree with proposal 2 in the above WF that RAN1 should handle the capability definition.

- UE Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Compatibility with LTM:

In LTM serving cell can be configured as a candidate cell and event triggered measurement reporting can be configured on serving cell. That means UE may be configured with UEIBM for serving cell and event triggered reporting for the same cell from LTM candidate cell perspective. Though the cell may be same, the event frameworks between MIMO and LTM are different. To avoid frequent reports due to fast fading, MIMO agreed on counter-based mechanism and LTM agreed on timer-based mechanism, We think UE may implement them as a different HW capability. Though the same RS and thresholds are configured, the counter and timer should be implemented separately. Hence, we think it may be reasonable to assume they need to be treated as different event from the UE capability perspective.

Having said that, there are further details like which layer maintain or evaluate the events are still discussed in RAN1/2. Hence, we think it may be little premature to agree on the UE capability without all the details in place. We also would like to point that UE capability discussions are takes place generally in RAN1 at the end of the core part of the WI. Hence, we think RAN4 should leave the UE capability discussion to RAN1. If RAN1 do not define this, then RAN4 can define.

- RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- When the RS periodicity for current beam and new beam is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

- RAN4 to wait for RAN1 progress on consideration of counter or timer in the UE initiated beam reporting in RAN4 requirements.

- Event evaluation period for a SSB or CSI-RS can be decided in RAN1.

- RAN4 to agree TTime to First UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time. Where, Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1.

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

- Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

- UE Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

- RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

# References

[1]		R4-2420096, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung

[2] 	RP-234007, New WID: NR MIMO Phase 5