3GPP TSG RAN WG4 Meeting #114bis	R4-2503935

Wuhan, China, 07 Apr. 2025 – 11 Apr. 2025

Agenda Item:	7.22.3.2

Source: 	Ericsson

Title:	Discussion on CJTC reporting and asymmetric UL TRP requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for the CJT calibration reporting.

# Discussion

CJT enhancements

In RAN4#112bis meeting, RAN4 agreed to specify the core requirements based on the outcome of the simulation study. Agreement is shown below.

•	RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later.

•	Further discuss the simulation assumptions offline during this meeting.

Way forward:

•	Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

In RAN4#113 and RAN4#114 meeting, following simulation assumptions [2], [3] were agreed. It was further agreed that the number of samples should be determined based on their impact on the reporting interval, taking into account the granularity required for the given report resolution.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| AD | CP |
| MD | 128 |
| AFO | 0.2ppm |
| MFO | 256 (agreed as 128 in WF but that value is not supported in RAN1. We considered 256 based on moderator suggestion in email discussion) |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz, slot 20 and 21 for 30KHz |

In the following sections we present the simulation results based on the above simulation assumptions for the CDF accuracy at 5%, 50% and 95% and make few observations and proposals concerning the core requirements definition.

### Simulation results for frequency offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_FO=200 ppb; FO error at 100 ppb (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) |
| 1 TRS | 5.854 | 1 | 8 | 26 | 2.849 | 1 | 4 | 12 |
| 2 TRS | 4.853 | 1 | 7 | 18 | 2.165 | 1 | 3 | 8 |
| 3 TRS | 3.794 | 1 | 5 | 16 | 1.916 | 1 | 3 | 7 |
| 5 TRS | 3.106 | 1 | 4 | 12 | 1.433 | 1 | 2 | 6 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_FO=200 ppb, FO error at 100ppb (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) |
| 1 TRS | 3.265 | 1 | 5 | 14 | 1.553 | 1 | 2 | 7 |
| 2 TRS | 2.396 | 1 | 4 | 9 | 1.107 | 1 | 2 | 5 |
| 3 TRS | 1.905 | 1 | 3 | 8 | 0.991 | 1 | 2 | 4 |
| 5 TRS | 1.547 | 1 | 2 | 6 | 0.794 | 1 | 2 | 3 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

As we can see from the above simulation results, for frequency offset, for SNR of -3dB and 3dB, frequency offset averaged over multiple samples (e.g., averaged over 3 samples or 5 samples) gives better accuracy. Hence, we suggest defining the CJT requirements with multi-sample measurement. Further from the simulations results, 5 sample averaging do not give significant improvement over the 3-sample averaging.

- RAN4 to define the delay requirements of the CJTC frequency offset with 3 sample averaging.

### Simulation results for delay offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_DO=CP, DO error at 0.5CP (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.012 | 1 | 2 | 5 | 0.012 | 1 | 2 | 5 |
| 2 TRS | 0.010 | 1 | 2 | 4 | 0.010 | 1 | 2 | 4 |
| 3 TRS | 0.009 | 1 | 2 | 4 | 0.008 | 1 | 2 | 4 |
| 5 TRS | 0.007 | 1 | 1 | 3 | 0.007 | 1 | 1 | 3 |
| 10 TRS | 0.005 | 1 | 1 | 2 | 0.005 | 1 | 1 | 2 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_DO=CP, DO error at 0.5CP (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.009 | 1 | 2 | 4 | 0.008 | 1 | 1 | 3 |
| 2 TRS | 0.007 | 1 | 1 | 3 | 0.006 | 1 | 1 | 3 |
| 3 TRS | 0.006 | 1 | 1 | 3 | 0.005 | 1 | 1 | 3 |
| 5 TRS | 0.006 | 1 | 1 | 2 | 0.004 | 1 | 1 | 2 |
| 10 TRS | 0.004 | 1 | 1 | 2 | 0.003 | 1 | 1 | 2 |

As we can see from the above simulation results, for delay offset, for SNR of -3dB and 3dB, delay offset averaged over multiple samples (e.g., averaged over 3 samples or 5 samples) gives better accuracy. Hence, we suggest defining the delay offset reporting requirements with multi-sample measurement. Further from the simulations results, 5 sample averaging provide significant improvement over the 3-sample averaging and single sample measurement.

- RAN4 to define the delay requirements of the CJTC delay offset with 5 sample averaging

### Simulation assumptions and results for Phase offset

Table below summarizes the simulation parameters for reciprocity-based CJTC phase offset reporting:

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz TDD |
| SCS | 30kHz SCS |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| TRP antenna configuration | 1x2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | cjtc-p, periodic CSI-RS using 1 or 2 ports. CSI-RS is precoded by phase of channel estimate from latest SRS with a 1-1 map between SRS and CSI-RS port number |
| CSI-RS periodicity | 20 slots, with slot offset 2 and 3 |
| SNR | -3dB, 3dB |
| Initial UL/DL phase offset between TRPs | 0 degree, 60 degree, 90 degree |
| SRS configuration | Periodic SRS every 10 slots, 48 RB bandwidth |

Impact of number of reported bits on wideband UL/DL phase offset estimation error (50 and 95-percentile) is shown in the tables below:

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 50%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) |
| 0 deg, 1 port | 7.595 | 1 | 1 | 2 | 5.255 | 1 | 1 | 1 |  |
| 60 deg, 1 port | 7.674 | 1 | 1 | 2 | 5.422 | 1 | 1 | 1 |  |
| 90 deg, 2 ports | 5.026 | 1 | 1 | 1 | 3.483 | 1 | 1 | 1 |  |

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 95%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) |
| 0 deg, 1 port | 23.001 | 2 | 3 | 5 | 14.757 | 1 | 2 | 3 |  |
| 60 deg, 1 port | 22.372 | 1 | 2 | 4 | 14.977 | 1 | 2 | 3 |  |
| 90 deg, 2 ports | 15.261 | 1 | 2 | 3 | 10.229 | 1 | 1 | 2 |  |

As we can see from the above simulation results, the error measured at 50% point was 5 degrees at SNR of 3dB. If we use the 16 levels in reporting, UE will report it as 22.5 degrees. That means the quantization introduce additional 17.5 degrees error. This error makes the report almost useless for the CJT. As we analyzed in our previous contribution to RAN4, R4-2416335, phase offset has to be within 30 degrees and 10 degrees for some of the applications to work in the field. Even when UE can estimate the PO as low as 5 degrees, this kind of quantization error makes the report to be useless for CJTC. Hence, we suggest RAN4 sending LS to RAN1 to increase the number of levels to avoid the quantization error impact the quality of the CJT phase offset report.

- RAN4 to send LS to RAN1 to increase the number of levels for the phase offset reporting.

Asymmetric DL sTRP/UL mTRP

In this section we further discuss the 2TA requirements and UL TCI state switching requirements for UL only TRP.

2TA for the asymmetric DL sTRP/UL mTRP

Based on the RAN1 LS, in last meeting RAN4 discussion is  based on the following scenarios.

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

- Scenario#3: if UE is not configured with PL offset in joint/UL TCI state(s), and UE is not provided with SSB.

For Scenario#1 RAN4 agreed to define the timing requirements for FR1, while it is FFS on FR2.

For Scenario#2 it is FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not).

Regarding how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP, following agreement is reached.

For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signals(s) of the reference cell associated with DL or Joint TCI state.

  - The above agreement can be revisited if not aligned with RAN1 further agreement.

We note that the agreement above pertains to PUCCH, PUSCH, and SRS. However, in Rel-18 mTRP, RAN4 also specified the downlink timing reference requirement for PRACH triggered by PDCCH order (the requirement is shown in the table below). We believe it is crucial to discuss the downlink reference timing for PRACH, especially as PRACH is supported for uplink-only TRP.

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point. . For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal. For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1. For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. Where NTA and NTA offset are commanded by the network independently for each TAG [TS 38.331]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in Table 7.1.2-2. In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18.

To enable two TAs with respect to two TRPs, UE need to obtain DL reference timing and UL TA command towards UL-only TRP. Next, we further discuss the aspects of DL reference timing and UL TA towards the UL-only TRP. The two-TA feature in Rel-18 assumes non-ideal synchronization (i.e., TRPs slot boundaries are not perfectly time aligned and there can be synchronization error of up to e.g., 3µs) between the TRPs. As per the WID, asymmetric and non-collocated deployment is supported for UL-only TRP. That means the synchronization assumptions in Rel-18 are applied for Rel-19 UL-only TRP also as operators may not deploy separate TRPs for using the UL-only TRP feature (e.g., operators may use same deployment for Rel-18 and Rel-19).

As UL-only TRP do not transmit any SSB, UE may have to rely on Anchor TRP SSB to derive the DL reference timing. When slot boundaries of Anchor TRP and UL-only TRP are perfectly aligned in time (i.e., synchronization error is 0µs), UE may use the DL reference timing of Anchor TRP to perform random access procedure and obtain TA command to handle the asymmetric propagation delay difference between Anchor TRP to UE and UL-only TRP to UE.

However, in the practical deployments (such asymmetric deployments considered in this WI) it may be sometimes impossible to maintain perfect synchronization between different TRPs even for intra-DU deployments, since most of the NW deployments use switched fronthaul technic between non-collocated TRPs. Figure 1 illustrates the synchronization scenario where the timing of UL-only TRP is ahead of Anchor TRP. When UL-only TRP is ahead of Anchor TRP slot timing, when UE acquires DL timing from Anchor TRP and transmits PUSCH using the same DL reference timing, it may result in inter-slot interference at UL-only TRP.

Figure 1: Synchronization scenario UL-only TRP ahead of Anchor TRP slot timing

As per TS 38.133, initial transmit timing for RACH transmission, UE needs to acquire frame boundary within ±Te error (Te is much smaller compared to possible synchronization error between TRP), and transmits RACH after acquiring DL synchronization, RACH preamble is always expected to receive after the start of the RACH slot at the NW.

To analyze the issue with practical synchronization error for UL-only TRP, we consider three possible synchronization scenarios as shown in Figure 2. When the UL slot boundaries of anchor TRP and UL- only TRP are not perfectly synchronized (scenario 2 and 3) as shown in Figure 2, using DL timing of anchor TRP to transmit RACH preamble to UL TRP may result in early arrival of RACH preamble or later arrival of the RACH preamble at the UL- only TRP.

Figure 2: Different synchronization scenarios at UL- only TRP and anchor TRP

When UL- only TRP is behind anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in early arrival of RACH preamble at UL- only TRP (i.e., before the start of RACH occasion or RACH slot at the reception window at the UL TRP as shown in Figure 3). This results in inter-slot interference at the UL- only TRP and UL TRP may not be able to determine the timing advance correctly for the UE.

Figure 3: RACH arrival at UL- only TRP and Anchor TRP with anchor TRP DL reference timing

When UL- only TRP is ahead of anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in late arrival of RACH preamble at UL- only TRP. Based on the below table showing guard period (GP) for different PRACH preamble formats, we think late arrival may not be an issue for the UL- only TRP deployment scenario where the UE is expected to be close to UL- only TRP and GP is large compared to synchronization error.

Table 1: Guard period for different PRACH preamble formats

| Preamble format | 0 | 1 | 2 | 3 | B1 | B2 | B3 | B4 | C0 | C2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GP in µs | 97 | 716 | 953 | 97 | 2.3 | 7 | 11.7 | 25.8 | 37 | 94 |
| Propagation delay (in km) that GP can cover | 29km | 215km | 285km | 29km | 0.7700m | 2.1km | 3.7km | 7.5km | 11km | 28km |

Based on the above discussion, we have the following proposal to consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in an asymmetric deployment, which for example could be handled by introducing a DL timing offset between the Anchor TRP for the RACH transmission towards the UL-only node. The DL reference timing offset can be a fixed value, which may be of the order of TAE or 0.5 TAE. The value can be further discussed.

- UE to apply additional DL reference timing offset for the PRACH transmission. The DL reference timing offset can be [0.5*TAE].

In addition to DL reference point for PUCCH/PUSCH/SRS/PRACH, RAN4 defined following requirements for 2TA in Rel-18 as well.

- UL transmit timing requirements

- Gradual timing adjustment requirements

UE UL transmit timing requirements:

Transmit timing requirements can follow the legacy principle. That means, for each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the reception of the downlink reference point. N_TA offset can be used as per the RAN1 definition.

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the downlink reference point of the respective TAG.

Gradual timing adjustment requirements:

We think Rel-18 principles can be reused for the requirements definition. We copy the Rel-18 requirement below for reference.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG.

Active uplink TCI state switching delay for UL only TRP

Way forward after last meeting:

Further discuss the two proposals and down-select one proposal in the next meeting:

  - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement.

  - Proposal 3 (Xiaomi, Nokia):

    - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP.

    - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

    - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay.

    - Further refine the wording for P3 in the WF, if needed

The UL TCI state switch for FR1 involves both timing acquisition and pathloss acquisition. We believe the NW is unlikely to use different PL-RS for the anchor TRP TCI state and the UL TRP TCI state. Any potential differences in power control between the UL TRP and anchor TRP can be managed using a PL-offset. For timing acquisition, we anticipate the NW will configure the SRS in alignment with the SSB associated with the activated TCI state. Consequently, we foresee no additional delays beyond the PL-offset update delay. In our previous meeting, we agreed not to specify the processing time for the PL-offset update MAC CE in RAN4. However, that agreement is concerning the separate MAC CE for PL-offset update. In summary, UL TCI state switch requirements for FR1 includes only MAC CE processing time and we are fine to specify it as 3ms.

- RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI state and the switch delay is 3ms.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

CJTC reporting

- RAN4 to define the delay requirements of the CJTC frequency offset with 3 sample averaging.

- RAN4 to define the delay requirements of the CJTC delay offset with 5 sample averaging.

- RAN4 to send LS to RAN1 to increase the number of levels for the CJTC phase offset reporting.

Asymmetric DL sTRP/UL mTRP:

- UE to apply additional DL reference timing offset for the RACH transmission. The DL reference timing offset can be [0.5*TAE].

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the downlink reference point of the respective TAG.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG.

- RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI state and the switch delay is 3ms.

# References

[1] RP-234007, New WID: NR MIMO Phase 5
[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5

[3] R4-2420209 Simulation assumptions for CJT calibration