# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503951

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			7 MHz channel bandwidth – BS impacts

Agenda item:		7.8.3

Document for:		Approval

# Introduction

In RAN#106, a WI (1) has been approved to specify 7 MHz channel bandwidth for NR bands n5 and n26.

Based on the agreed WF ([2]) from last RAN4#114 meeting, this contribution further discusses the remaining open issues related to BS RF.

# Discussion

## Open issues

### New 7 MHz FRC

In last RAN4#114 meeting, it was proposed to introduce new 7 MHz FRCs for the Rx requirements.

Until now, 5 MHz FRCs are used to test up to 15 MHz channel bandwidth, while 20 MHz FRCs are used to test from 20 to 100MHz channel bandwidth. The drawback of using 5 MHz FRCs to test 10 and 15 MHz channel bandwidth is that 10 and 15 MHz shall be tested in 2 shots because they don’t have a spectrum utilization which is a multiple of 5 MHz one. The situation would then be similar for 7 MHz channel bandwidth, meaning it shall be tested in 2 shots as well.

If we introduce 7 MHz FRC, this situation could then be avoided, and this channel bandwidth could be tested in one shot. On the other hand, when testing Rx requirements for 5, 7, 10 and 15 MHz channel bandwidth, the tester will have to switch between FRCs, which would reduce the benefit of introducing a 7 MHz FRC.

Also, when a new narrow channel bandwidth would be introduced (e.g. 6 MHz), to be consistent, RAN4 would have to specify yet another new FRC (e.g. 6 MHz FRC) which would increase again the number of specified FRCs, and this for a limited gain.

Also, to test Rx requirements for 7 MHz channel bandwidth, we only need to consider 2 overlapping 5 MHz signals which is a low number.

To conclude, considering as well that both 5 and 10 MHz are all tested with 5 MHz FRCs, we don’t think 7 MHz FRC needs to be specified.

Proposal1: Do not specify 7 MHz FRC and reuse the existing 5 MHz FRCs to test Rx requirements for 7 MHz channel bandwidth.

## Tx requirements

### Total power dynamic range

Following the requirement calculation for the other NR channel bandwidths and SCS, which itself follows the requirement calculation for E-UTRA, the total power dynamic range for 7 MHz channel bandwidth and 15kHz SCS shall be 15.4 dB.

Proposal2: Specify the total power dynamic range for 7 MHz channel bandwidth and 15kHz SCS with 15.4 dB value.

## Rx requirements

In the following, it’s assumed that the 5 MHz channel BW FRC will be reused to specify 7 MHz channel BW Rx requirements. The following proposals are based on TR 38.817-02.

### REFSENS

Proposal3: Specify REFSENS for 7 MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

### Dynamic range

The interferer signal mean power for 7MHz channel BW / 35 RBs shall be -81.0 dBm for WA BS, -76.0 dBm for MR BS and -73.0 dBm for LA BS.

Proposal4: Specify Dynamic Range for 7MHz channel BW according to the following table:.

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 7 | 15 | G-FR1-A2-1 (Note 1) | -70.7 | -65.7 | -62.7 | -81.0 | -76.0 | -73.0 |

### ACS

Proposal5: Specify the ACS interferer frequency offset with ±2.5075 MHz value.

### Narrow band blocking

Proposal6: Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value: ±(360+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

### Intermodulation

Proposal7: Specify the interfering signals for the intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

### In-channel selectivity

Instead of specifying a new FRC to specify ICS for 7 MHz channel bandwidth, we would propose to specify it based on the requirement for 5 MHz channel bandwidth, similar to what is done for 10, 15, 20, 25, 30 and 35 MHz channel bandwidth, reusing G-FR1-A1-7 FRC and a 10RBs interferer.

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5, 7 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

Proposal8: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW.

# Conclusion

In this contribution, we gave an overview of the BS specifications impacts when introducing 7 MHz channel bandwidth. We made the following observations and proposals:

Proposal1: Do not specify 7 MHz FRC and reuse the existing 5 MHz FRCs to test Rx requirements for 7 MHz channel bandwidth.

Proposal2: Specify the total power dynamic range for 7 MHz channel bandwidth and 15kHz SCS with 15.4 dB value.

Proposal3: Specify REFSENS for 7 MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

Proposal4: Specify Dynamic Range for 7MHz channel BW according to the following table:.

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 7 | 15 | G-FR1-A2-1 (Note 1) | -70.7 | -65.7 | -62.7 | -81.0 | -76.0 | -73.0 |

Proposal5: Specify the ACS interferer frequency offset with ±2.5075 MHz value.

Proposal6: Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value: ±(360+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

Proposal7: Specify the interfering signals for the intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

Proposal9: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW.

# References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA

- R4-2502870, WF on requirements for 7 MHz channel bandwidth, Ericsson