3GPP TSG-RAN4 Meeting # 114bis	R4-2503963

Wuhan, China, Apr.7th – 11th, 2025

# Title: 	Discussion the methods for reducing the number of UE Rx chain

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.2

# Document for:	Approval

# 1 Introduction

In the last meeting, companies continue discussing the methods of reducing the number of UE Rx chain[1], and there are some agreements captured in the approved WF [1]. Meanwhile, there still some open issues which was not discussed due to lack of time.

In this contribution, we give some views on some left open issues on the methods for reducing the number of Rx chain for fragment carriers.

# 2	Discussion

For the baseband assumption, we think different FFT size implementation for intra-band DL NC CA is a modern design, no matter whether or not fragment carriers feature is supported. Nevertheless, it still possible to use same FFT size implementation when the same numerology are used for each carrier for feasibility purpose, and also the baseband assumption seems out of the scope for fragment carrier discussion due to only RF assumption, i.e. shared Rx RF architecture, is mentioned in the SID, and it will impact the RF requirements using shared Rx RF architecture for intra-band DL NC CA.

- Given that, RAN4 agreed to keep the implementation feasibility for the baseband, and to keep the previous agreements in WF R4-24020519.

Regarding the triggering condition to enable “one RF Rx chain” or “shared RF Rx chain”, the agreements were:

- Triggering condition to enable ”One RF Rx chain” FFS: - FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network. - Triggering condition for UE working in one Rx RF chain is needed - Triggering condition for UE working in legacy Rx RF chains is needed - The switching should be not be dynamic and be under network control with UE indication of condition.

In our understanding, the triggering conditions are needed to enable UE ‘one RF Rx chain’, and such conditions shall be under the control of network. Considering the DL PCC or SCC performance may be degraded when UE enable ‘one RF Rx chain’, then if there are no other methods such as more MIMO layer which can compensate the performance, then NW can make decision to enable whether or not enable UE ‘one RF Rx chain’ with the consideration of the whole system performance.

The purpose for the condition is to make NW configure/schedule UE band configuration correctly, which is when UE reports the capability of shared Rx RF chain to the NW after initial access, NW would configure/schedule the band configuration for the UE whose report the capability. However, when there are some conditions such as the power measured in the gap is larger which makes the DL CC performance poorer comparing with traditional way or single carrier due to poor filter attenuation in the gap, the benefit of shared Rx RF chain would loss. In this case, NW should disable the shared RF Rx chain feature to maintain the DL performance.

For the triggering condition, it is straightforward to use the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC as the trigger condition. In addition to that, we think the frequency span should be also considered as one of the condition. In this SID, only frequency span≤100MHz between two DL CC is considered. However, up to 200MHz for one single Rx RF chain can be supported in current design. If we only focus on frequency span ≤100MHz, then the ≤100MHz frequency span can be seen as the precondition for the shared RF Rx chain. For those UE who can support >100MHz frequency range but report the capability of shared Rx RF chain, NW should disable this capability.

Proposal 1. Both frequency span and the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled.

In addition to the above condition, how UE reports the shared Rx RF chain to inform the network of appropriate CA configuration it can support is also discussed in last meeting, but no consensus were achieved. The WF is:

- How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. Other options are not precluded - Discussion on semi-statically switch hardware resources procedure and fallback considerations

As we discuss in [2], the inter-band DL NC CA band configurations supported by a UE capable of shared Rx RF chain are essentially traditional CA band configuration, where the existing Tx requirement shall be applied. In other words, there are no impacts on Tx requirements. The only impacts are the Rx requirements. Therefore, we think legacy band combination reporting scheme + Rx sharing capability indication is straightforward and simple, and also the impacts on RAN2 is minimum since it is normal procedure similar as other new feature introduce in later releases.

For the capability granularity, since there are only two DL carriers considered in fragment carrier SID, so ‘per-band per-BC’ capability’ is enough. However, if considering future proof which is more than two DL carriers are considered but there are some of DL carrier can be implemented in fragment carrier mode, in this case ‘per-band per-BC’ capability’ is not enough, instead ‘per CC per band per BC’ would be needed. Therefore, considering it is SID, and future proofing should be taken into accounted,  ‘per CC per band per BC’ would be fine.

Proposal 2. We prefer to adopt legacy band combination reporting scheme + Rx sharing capability indication for fragment carrier feature reporting.

- ‘per CC per-band per-BC’ capability

For the Rx sharing capability indication, there may exist several approaches, including implicit and explicit indications, such as shared Rx RF capability IE, frequency separation IE, or other IEs, and it seems it belongs to RAN2’s action, and it would better to make decision in the WI phase.

Proposal 3: Implicit and explicit indications for the Rx sharing capability indication should be considered, more details can be discussed in the WI phase.

Another thing is fallback solution, which is when a UE report the shared Rx RF chain related capability, but the condition to enable this capability is not met, then what is the UE behaviour. It seems there are two options:

1. Fallback to legacy CA behaviour

2. Fallback to non-CA behaviour, i.e. single carrier

For the first one, as pointed out by some companies in last meeting, it seems it would cause UE’s complexity since legacy Rx RF architecture (i.e. partial shared) and one Rx RF architecture should be supported for UE, and also switching between these two architectures are needed to be considered and LO retuning may be needed which would cause like-Rx switching related discussion. Meanwhile, the condition for the switching between the architectures may need to be discussed. Therefore, we prefer to fallback to non-CA behaviour for the same RF architecture, it is simple and compliance to the legacy band combination fallback behaviour.

Proposal 4: We prefer to fallback to non-CA behaviour for the band combination supported by a UE enable of full shared Rx RF chain.

For the applicability for PCell and SCell, it seems there are no clear criteria to define which CC can be used for PCell or Scell before UE initial access to NW. In general, when UE access to a cell successful then this cell CC is configured as Pcell CC, and when handover happens, UE may successful access to another CC then the original Pcell CC would be changed. Usually all of the carriers can be either a Pcell or an Scell from NW perspective, unless this cell have very poor performance which can not be accessed by a UE in handover procedure.

Moreover, the first step for this SID only includes 2 DL CC to be implemented by one Rx RF chain, which means one CC shall be Pcell CC.

Proposal 5: Any of the CCs in the FC mode can be either a Pcell or an Scell.

# 3 Conclusion

In this contribution, we give some discussions on some left open issue on the methods for reducing the number of Rx chain for fragment carriers:

Proposal 1. Both frequency span and the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled.

Proposal 2. We prefer to adopt legacy band combination reporting scheme + Rx sharing capability indication for fragment carrier feature reporting.

- ‘per-band per-BC’ capability

Proposal 3: Implicit and explicit indications for the Rx sharing capability indication should be considered, more details can be discussed in the WI phase.

Proposal 4: We prefer to fallback to non-CA behaviour for the band combination supported by a UE enable of full shared Rx RF chain.

Proposal 5: Any of the CCs in the FC mode can be either a Pcell or an Scell.

# 4	Reference

[1] R4-2502873, WF on fragmented DL, Mediatek

[2] R4-2500941, Discussion the methods for reducing the number of UE Rx chain, ZTE Corporation, Sanechips.