3GPP TSG-RAN4 Meeting # 114bis	R4-2503964

Wuhan, China, Apr.7th – 11th, 2025

# Title: 	Discussion on the RF requirement for fragmented carrier

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.3

# Document for:	Approval

# 1 Introduction

In the previous meetings, companies continue discussing the methods of reducing the number of UE Rx chain, and there are some agreements captured in the approved WF [1][2]. Meanwhile, there still some open issues which was not discussed due to lack of time.

In this contribution, we give some views on some left open issues on the RF requirement for reducing the number of Rx chain for fragment carriers.

# 2	Discussion

For the PSD different, as we discussed in [3], we observed that the power difference, PSD difference and the carrier channel bandwidths are dependent each other, since PSD difference = Power different - 10log(BWinterferer/BWwanted). In current ACS/IBB test, the BWinterferer =5MHz is adopted for <2700MHz bands. We think this test configuration should be reused for fragment carriers in a single Rx RF chain. However, the channel bandwidth PCC or SCC are different for different band combination, and for some band combinations, 5MHz is not used for PCC or SCC. Looking at the existing intra-band DL NC CA case, the largest PCC channel bandwidth is 40MHz (for CA_n25(2A) where the aggregated channel bandwidth (PCC+SCC)= 40MHz +5MHz), which is the wanted carrier with the lower PSD (assume the power for 40MHz PCC and 20MHZ SCC are the same).

For the power different, 25dB is defined in the specification, and it is for non-collocated. But by using the value, the PSD difference = 25 - 10log(5/min(BWPCC, BWSCC).

Proposal 1: The [PSD] difference could be [25] - 10log(5/min(BWPCC, BWSCC).

For the ΔRIBNC re-evaluation for PCC and SCC, in our understanding, there are no discussions for the parameter assumptions although the table format was approved in [1]. Nevertheless, some companies still use their owns table for the calculations. This situation somehow was similar before fragment carrier feature is introduced. Consequently, ΔRIBNC values are sometimes averaged among companies’ proposals.

In our contributions[3], we gave some initial link budge calculation for n3. There are two configurations for CA_n3(2A) in the specification.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0 |  |

In this contribution, we consider the approaches in [4] to update the values, which are:

For PCC=5MHz, SCC=5MHz with Wgap =65.0MHz, RIBNC_PCC_5M = 4.7dB. RIBNC_SCC_5M = 13.2dB

For PCC=5MHz, SCC=5MHz with Wgap =45.0MHz, RIBNC_PCC_5M = 4.3dB. RIBNC_SCC_5M = 13.2dB

For n7(2A), it is:

For PCC=10MHz, SCC=5MHz with Wgap =55MHz, RIBNC_PCC_5M = 2.7dB. RIBNC_SCC_5M = 3.3dB

For PCC=10MHz, SCC=5MHz with Wgap =30MHz, RIBNC_PCC_5M = 2.4dB. RIBNC_SCC_5M = 2.9dB

# 3 Conclusion

In this contribution, we give some views on some left open issues on the RF requirement for reducing the number of Rx chain for fragment carriers.

Proposal 1: The [PSD] difference could be [25] - 10log(5/min(BWPCC, BWSCC).

Proposal 2: For CA_n3(2A) using fully-shared Rx chain:

For PCC=5MHz, SCC=5MHz with Wgap =65.0MHz, RIBNC_PCC_5M = 4.7dB. RIBNC_SCC_5M = 13.2dB

For PCC=5MHz, SCC=5MHz with Wgap =45.0MHz, RIBNC_PCC_5M = 4.3dB. RIBNC_SCC_5M = 13.2dB

Proposal 3: For CA_n7(2A) using fully-shared Rx chain:

For PCC=10MHz, SCC=5MHz with Wgap =55MHz, RIBNC_PCC_5M = 2.7dB. RIBNC_SCC_5M = 3.3dB

For PCC=10MHz, SCC=5MHz with Wgap =30MHz, RIBNC_PCC_5M = 2.4dB. RIBNC_SCC_5M = 2.9dB

# 4	Reference

[1] R4-2420519, WF on fragmented DL study, Mediatek

[2] R4-2502873, WF on fragmented DL, Mediatek

[3] R4-2500942, Discussion on the RF requirement for fragmented carrier, ZTE Corporation, Sanechips

[4] R4-2500118, Discussion on UE RF requirement impact for DL fragmented carriers, Murata Manufacturing Co Ltd.