3GPP TSG-RAN4 Meeting # 114bis	R4-2503969

Wuhan, China, Apr.7th – 11th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Consideration on R19 HPUE MSD rule

# Agenda item:	7.1.1.1.1

# Document for:	Approval

# 1 Introduction

In this contribution, we give some general consideration on HPUE MSD rule.

# 2	Discussion

- In previous meeting, the HPUE MSD rule was extensively discussed. The latest WF [1] was not approved due to there still are some issue which are not be acceptable to everyone. Good thing is that companies’ view come to converge.

- 2.1	 HPUE MSD v.s. PC3 MSD=0

- Obviously, the idea for HPUE MSD requirements is to use the fundamental PC3 MSD requirements as baseline and to develop a so-called LUT/equation approach, which is delta value relative to the default PC3 MSD.

- However, there exists exception in the case of there is no PC3 MSD define, as sated in the paper[1]:

- The PC2/1.5 MSD requirements are no longer studied, except the exceptional case that PC2/1.5 MSD is needed while default power class MSD is 0, or the case that PC2/1.5 MSD does not fit the new MSD rules (which are supposed to be rare case). The exceptional PC2/1.5 MSD will be captured into current PC2/1.5 MSD tables. - Default power class TPs for TR or draft CRs that propose MSD=0 shall state that the need for exceptional MSD for PC2/1.5 has been checked and what was the conclusion. - TPs for TR or draft CRs for PC3 which do not propose an MSD for PC3 shall be flagged IF a company/delegate believes a PC2/1.5 MSD is needed, otherwise PC2/1.5 MSD is not specified.

- During the discussion for PC3 band combination, there are several reason for PC3 MSD requirements are not defined, such as:

- 1: No valid MSD test point

- ==>For example, there are IMD frequency-hits problem in the victim band using FUL_low and FUL_high of each band for calculation. But when define UL/DL Fc considering the minimum supported UL/DL BWs, then the valid UL/DL Fc may not be found. In this case, MSD will not be defined.

- 2: Based on operator’s spectrum

- ==>It is indeed happened although it is not a common case. No MSD defined considering the operator holdings although there are frequency-hits problem.

- 3: MSD is too small to negligible

- ==> At early stage, MSD<0.5dB can be seen as a small value which can be ignored. Usually in this case, MSD may not be defined in the spec.

- For the former two, we think it is also applicable for HPUE band combination. When it happens for PC3 band combination, it is time-consuming to track the history although there are TRs in each release.

- For the third one, when UL power for aggressor is increased like PC3->PC2, then the MSD would become larger than 0.5dB which make PC2 MSD cannot be ignored. In this case, PC2 MSD should be studied although there are no PC3 MSD defined. In our understanding, this is the exceptional case that PC2/1.5 MSD is needed while default power class MSD is 0.

- Regarding the HPUE MSD rule, we DO NOT recommend to handle the HPUE band combination in the PC3 basket WID when PC3 MSD=0 with the following consideration:

- RAN4 never state whether or not there are PC2/1.5 combination when companies do the PC3 combs, and we think it is unfair to do so, and also it will mix the work and break the PC3 procedure.

- PC3 band combination is fundamental combination. For example, company A only has interesting in PC3, but no interesting in PC2 or PC1.5, and indeed there is no PC3 MSD is found, why forcing company A to propose an artificial MSD value to define PC2/1.5 MSD?

- It is bad to flag PC3 combs when PC3 MSD=0 with the reason that PC2/PC1.5 MSD should be considered. If the TP in PC3 basket get flagged, the flag needs to be resolved to approve the TP.

- Non-zero PC3 MSD is not the prerequisite for PC2/PC1.5 MSD definition. PC3 MSD shall be studied for a new configurations, but it doesn't mean MSD should not be equal 0 even there are PC2/PC1.5 band combination.

- With the above consideration, we have the following proposal:

- Proposal 1. When PC3 MSD=0, there is no need to state/flag PC3 TP/draft CR to check the PC2/1.5 MSD.

- 2.2	 HPUE 2Tx MSD v.s. HPUE 3Tx MSD

- For the HPUR MSD rule discussing, both 2Tx and 3Tx HPUE band combinations shall be included. For 3Tx HPUE band combination, PC3 3Tx band combinations were supported by default.

- In R18, when 3Tx band combination was introduced, there are a few 3Tx band combination included. For the 3Tx MSD, in general it reuse the same MSD requirements of the band combination with either 1Tx or 2Tx UL with the same power class for harmonic/harmonic mixing/cross-band isolation MSD, and reuse the 1Tx-1Tx UL configurations for 1Tx-2Tx UL configuration to derive the 3Tx MSD requirement. In other words, the MSD requirements for HPUE 3Tx band combination were reused from the corresponding HPUE 2Tx band combination.

- In R19, more types HPUE band combination were introduced. But in our understanding, the above rule shall also be applied for all the applicable 3Tx HPUE band combination in R19.

- We have the following proposals:

- Proposal 2: For harmonic/harmonic mixing/cross-band isolation for 3Tx PC2/1.5 inter-band UL CA, the MSD requirements are reused from the corresponding HPUE 2Tx band combination with either 1Tx or 2Tx UL with the same power class.

- Proposal 3: For IMD for 3Tx PC2 inter-band UL CA, the requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration.

- 2.3 1 band 1 UL CC PC2/PC1.5 LUT

- In terms of the previous discussion, it seems the rule comes to converge for some types of MSD. However, still there are some other FFS issue like harmonic mixing of ULx/DLy (x>1, y>1). For 1 band 1 UL CC, there are three types of MSD, which are harmonic/harmonic mixing/cross-band isolation.

- In general, for PC2 harmonic/harmonic mixing/cross-band isolation MSD, it highly depends on the power class of the aggressor band, since there are several PA configurations in each band, like 23+26dBm, 23+23dBm, 26+26dBm.

- - if the aggressor UL band is 23dBm, there are no additional MSD requirements, instead to reuse the existing PC3 MSD

- - if the aggressor UL band is 26dBm, new additional MSD requirements are needed.

- In our understanding, the second bullet is in the scope of the LUT discussion.

- It seems it would be a bit difficult to use one delta MSD value in the LUT table to apply for all MSD types. Therefore, if using one delta MSD value cannot apply for all MSD types, then different delta MSD value would be a compromise choice for different MSD types.

- With the equation of PCx MSD = PC3 MSD + PCx, the example for the PCx for 1band 1UL CC different MSD types can be found below:

|  | [Harmonic] |  |  | [Harmonic mixing] |  |  | [Cross band isolation] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| 0<MSD≤ 1 |  |  |  |  |  |  |  |  |  |
| 1<MSD≤ 2 |  |  |  |  |  |  |  |  |  |
| 2<MSD≤ 3 |  |  |  |  |  |  |  |  |  |
| 3<MSD≤ 4 |  |  |  |  |  |  |  |  |  |
| 4<MSD≤ 5 |  |  |  |  |  |  |  |  |  |
| 5<MSD≤ 7 |  |  |  |  |  |  |  |  |  |
| 7<MSD≤ 9 |  |  |  |  |  |  |  |  |  |
| MSD ≥10 |  |  |  |  |  |  |  |  |  |

- Proposal 4: For 1band 1UL CC PC2/PC1.5, to consider different PC value for harmonic, harmonic mixing and cross-band isolation MSD if any.

# 3 Conclusion

In this contribution, we give some general consideration on HPUE MSD rule. The proposals and conclusions are:

- Proposal 1. When PC3 MSD=0, there is no need to state/flag PC3 TP/draft CR to check the PC2/1.5 MSD.

- Proposal 2: For harmonic/harmonic mixing/cross-band isolation for 3Tx PC2/1.5 inter-band UL CA, the MSD requirements are reused from the corresponding HPUE 2Tx band combination with either 1Tx or 2Tx UL with the same power class.

- Proposal 3: For IMD for 3Tx PC2 inter-band UL CA, the requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration.

- Proposal 4: For 1band 1UL CC PC2/PC1.5, to consider different PC value for harmonic, harmonic mixing and cross-band isolation MSD if any.

# 4	Reference

[1] R4-2502864, WF on MSD rules, Skyworks, Nokia