3GPP TSG-RAN WG4 Meeting #114bis							        	R4-2503988

Wuhan, CN, 7th-11st April 2025

Title:	RRM Core requirements on CSI-RS based L1 measurement for LTM

Source:	China Telecom

Agenda item:			7.27.2.2

Document for:	Discussion

- Introduction

In the RAN4 #114 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on CSI-RS based L1 measurement for LTM.

- Discussion

Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell

Candidate options: - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

For this issue, we think the purpose of CSI-RS based L1 measurement here is same as SSB based L1 measurement in R18 LTM. So, we think they could share the same side condition, i.e., use legacy side condition of SINR=-3dB.

Proposal 1: Use legacy side condition of SINR=-3dB.

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

- Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

This issue is related to the measurement procedure. If we remove SSB based L1 measurement for FR2-1 in step 2, UE can get better Rx beam with beam sweeping, however the delay will also increase by N times. If we keep SSB based L1 measurement for FR2-1 in step 2, the delay will be shorter.

Although we prefer to keep SSB based L1 measurement for FR2-1 in step 2 with the reason that the purpose of LTM is shorter latency. We can also see the benefaction of skipping it. So, we could agree that whether step 2 can be skipped or not can be based on UE capability reporting.

Proposal 2: Whether step 2 can be skipped or not can be based on UE capability reporting

Issue 4-6: Measurement on CSI-RS with repetition ON/OFF

Candidate options: - Option 1: (CATT, CT, vivo) In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. In FR2, RAN4 will define the requirements for the following case: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: (Huawei) Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. - Option 3: (Nokia) Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration. - Option 4: (Ericsson, Apple, Nokia) RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

For CSI-RS resources with repetition ON in FR2, UE needs to perform L1 CSI-RS measurement with RX beam sweeping, which will introduce extra latency and overhead. This is unbenefited with the objective of LTM to reduce the delay. So we do not support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

For the case which measurement on CSI-RS with repetition OFF, UE perform measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement. and UE performs L1 CSI-RS based measurement without RX beam sweeping.

So when the CSI-RS L1-measurement resources are configured as repetition OFF, previous SSB L1-RSRP measurement is necessary. We support that for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1, and we could define related requirements.

Proposal 3:

Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. And define the related requirements.

Issue 4-7: RTD assumption

Candidate options: - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP. Agreement: - Conclude this issue in the next meeting.

If not support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, many cases will be limited. From the deployment point of view, it is normal that the CSI-RS resources of neighbor cells are outside active BWP. In addition, RAN1 also agreed that there was no restriction on the frequency location of CSI-RS used for L1-measurement. So, we think that CSI-RS based L1-RSRP measurement with gap should be considered in R19 LTM.

If restrict the RTD between serving cell and neighbor cell to be within CP, some cases with higher SCS are also restricted. So, we think it better that support RTD>CP case for CSI-RS based L1-RSRP measurement, and UE capability for supporting RTD>CP can be introduced.

Proposal 4:

-	Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

-	Define requirements for RTD > CP, as optional UE feature. FFS details

Issue 4-12: Whether to consider CSI-RS based L1-SINR measurement

Candidate options: - Option 1: (CATT, Apple) RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. - Option 2: (ZTE, CT) Focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

As we know, in real field, the CSI-RS measurement is not widely used. So, we think RAN4 could only focus on CSI-RS based L1-RSRP measurement.

Proposal 5: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Use legacy side condition of SINR=-3dB.

Proposal 2: Whether step 2 can be skipped or not can be based on UE capability reporting

Proposal 3:

Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. And define the related requirements.

Proposal 4:

-	Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

-	Define requirements for RTD > CP, as optional UE feature. FFS details

Proposal 5: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

- References

[1] R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.