3GPP TSG RAN WG4 Meeting #114bis	R4-2504050

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.29.4.1

Source:	vivo

Title:	Discussion on RRM requirements on (e)RedCap for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on Rel-19 NR NTN WI, which mainly included the objectives of downlink coverage enhancements, support of regenerative payload and Redcap NTN. The considerations from companies and the latest progress on (e)RedCap NTN were summarized in WF [1]. In this contribution, we would like to provide our further analysis on aspects on RRM requirements on (e)RedCap NTN.

- 2.	Discussion

  - General consideration on RRM requirements

    - What RRM requirements are defined for (e)RedCap UE with FR1-NTN

      - Requirements to be further determined

#### Issue 1-1-2: SDT requirements Online agreement: Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS

Regarding the definition of SDT requirements, RAN4 has already agreed in the previous meeting that TA validation based on the RSRP change criterion for RedCap in TN shall be removed, and only the check of whether cg-SDT-TimeAlignmentTimer is running will be retained in TA validation. A minor issue left in the last meeting was whether to send an LS to RAN2 to inform them of this conclusion.

We understand the motivation for sending an LS stems from the fact that RAN4 has not yet defined any SDT-related requirements for NR NTN. Therefore, when RAN4 agrees to define SDT requirements for RedCap in NTN, there might be some impact on RAN2’s specifications.

On this matter, our view is, according to the existing TS 38.213 specification and the IE design in TS 38.331 summerized as follows, from RAN2’s procedure perspective, the RSRP change condition is an optional configuration. If it is not configured, only the case where cg-SDT-TimeAlignmentTimer (configured as mandatory) is running is considered for TA validation in CG-SDT.

TS38.213 ### 5.27.2 TA Validation for CG-SDT RRC configures the following parameters for TA validation for CG-SDT: - cg-SDT-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation. The MAC entity shall, upon the reception of CG-SDT configuration: 1> store the current RSRP of the downlink pathloss reference for TA validation as defined in TS 38.331 [5] clause 5.7.17. The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and - cg-SDT-TimeAlignmentTimer is running. TS38.331 <Relevant part 1> cg-SDT-TA-ValidationConfig Configuration for the RSRP based TA validation. If this field is not configured, then the UE does not perform RSRP based TA validation. <Relevant part 2> CG-SDT-TA-ValidationConfig-r17 ::= SEQUENCE { cg-SDT-RSRP-ChangeThreshold-r17 ENUMERATED { dB2, dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare5, spare4, spare3, spare2, spare1} <Relevant part 3> cg-SDT-TA-ValidationConfig-r17 SetupRelease { CG-SDT-TA-ValidationConfig-r17 } OPTIONAL, -- Need M

From this perspective, RAN4 conclusion on the definition of RedCap NTN requirement based solely on cg-SDT-TimeAlignmentTimer is already covered by the existing procedure, meaning there should be no additional impact on RAN2’s specifications. Therefore, the necessity of sending an LS is not observed.

Proposal 1: For the conclusion on how to define SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2

We also noticed that we reached one related agreement in Issue 2-1-1 RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN in the RAN4#113, it was agreed that

#### Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Online agreement < Agreement >: - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

To align with what we agreed in the last meeting, for 1Rx Redcap NTN UE, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 will be not considered for RRC_INACTIVE mode. From this perspective, the agreement needs to be reverted as:

•	For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

Proposal 2: For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as:

- For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

  - Reduction in the number of UE Rx branches

#### Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional in Redcap over NTN. - Proposal 1a: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused. - Proposal 1b: To define enhanced cell reselection requirements for RedCap UE, - Re-use the requirements for normal UE except that Tdetect is extended by [2] samples - Same delay requirements for 1Rx and 2Rx UEs. - Proposal 1c: - For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements - For 1 Rx (e)RedCap NTN UE, RAN4 to - Option 1: Reuse the existing enhanced cell reselection measurement requirements - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined. - Proposal 1d: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands, - For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

For supporting the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands, the issue of how to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands was left in the last meeting. In our understanding, for 2Rx, the existing enhanced cell reselection measurement requirements can be reused. The focus is on the discussion of the requirements for 1Rx.

According to discussions in the previous release, for Redcap UE, the core requirement does not distinguish between 1Rx and 2Rx, but distinguishes between RSRP thresholds in performance. From this perspective, we can also apply the same enhanced requirements for 1Rx and 2Rx.

On the other hand, one concern for enhanced measurement mainly lies in that, compared with the normal requirement, the number of measurement samples for enhanced measurement is reduced to 1. For 1Rx, it needs to be discussed whether the measurement accuracy can be satisfied when the current enhanced cell reselection requirement for NTN LEO is directly applied and further extend more measurement samples for Red-cap NTN UE.

For this issue, we are open to discussing potential relaxation methods. One approach under consideration is doubling the number of Tmeasure sample, specifically by extending Tmeasure from 1 sample to [2] samples, the corresponding Tevaluate and Tdetect would also be adjusted to [6] samples and [11] samples, respectively.

Proposal 3: To define enhanced cell reselection requirements for RedCap NTN UE,

•	By extending Tmeasure samples from the existing enhanced requirement defined for NTN,

- Extend Tmeasure from 1 sample to [2] samples, the Tevaluate and Tdetect would also be adjusted to [6] samples and [11] samples accordingly

•	Same delay requirements for 1Rx and 2Rx UEs.

  - The impact of HD-FDD

#### Issue 3-1-1: General HD-FDD related impact <Way Forward>: - Proposal 1 (CATT, Xiaomi, Samsung, QC, CMCC, ZTE, Ericsson, HW): - The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario. - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions. - Proposal 1a (CATT, HW, MTK): - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects: - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions. - Proposal 1c (CMCC): - Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline. - Wait for further RAN1 progress which may impact Scheduling availability requirement.

Regarding the impact of HD-FDD, based on the discussion progress at RAN1#120, the defined prioritization rules still include cases where UL takes precedence over DL—for instance, when the network explicitly indicates UL overriding DL or leaves it to UE implementation to decide whether to prioritize UL or DL.

In such scenarios, UL prioritization may adversely affect DL measurements. From RAN4 perspective, there are two potential solutions: 1) Introducing additional measurement delay and defining scheduling restrictions to accommodate UL-prioritized cases; 2) Adding applicability rule in RAN4 clarifying that, from requirements perspective, only cases where DL resources are not dropped will be considered.

RAN1#120 Agreement When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS. Conclusion For Rel-19 HD-FDD (e)Redcap UE with CG-SDT procedure ongoing in RRC-inactive mode, for collision case 3, - Handling of collision with PDCCH CSS in RRC-inactive mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. - UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle. Working assumption For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized. - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases - This is signaled by one UE specific RRC parameter - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Proposal 4: For HD-FDD related impact, RAN4 to discuss how to handle the case of UL overriding DL. The potential solutions include:

- Introducing additional measurement delay and defining scheduling restrictions to accommodate UL-prioritized cases

- Adding applicability rule in RAN4 clarifying that, from requirements perspective, only cases where DL resources are not dropped will be considered

- Summary

Proposal 1: For the conclusion on how to define SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2

Proposal 2: For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as:

- For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

Proposal 3: To define enhanced cell reselection requirements for RedCap NTN UE,

•	By extending Tmeasure samples from the existing enhanced requirement defined for NTN,

- Extend Tmeasure from 1 sample to [2] samples, the Tevaluate and Tdetect would also be adjusted to [6] samples and [11] samples accordingly

•	Same delay requirements for 1Rx and 2Rx UEs.

Proposal 4: For HD-FDD related impact, RAN4 to discuss how to handle the case of UL overriding DL. The potential solutions include:

- Introducing additional measurement delay and defining scheduling restrictions to accommodate UL-prioritized cases

- Adding applicability rule in RAN4 clarifying that, from requirements perspective, only cases where DL resources are not dropped will be considered

- References

- R4-2502604, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT