# 3GPP TSG-RAN WG4 Meeting #114bis		  R4-2504055

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.19.3

Source:		Qualcomm Incorporated

Title:			Testability for AI/ML beam management

Document for:	Approval

# 1  Introduction

In RAN4 #114 meeting, testability issues for beam management were discussed. The following agreements regarding test setup are captured in [1].

Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

In this paper, we provide our further considerations on the testability issues for AI/ML beam management.

# 2  Discussion

## 2.1 General

Single AoA vs multiple AoA test setup has been discussed for several meetings but there was no conclusion. As agreed in [1], RAN4 starts to develop the test procedure for both single AoA and multiple AoA test setup assuming CDL channel model is emulated with Set A = 32 beams, Set B = 8 beams.

First of all, the test steps for AI/ML BM-case 1 can be described below:

- Progress A: UE BM performance with AI/ML for BM-case 1

  - Step 1: Model training step - Channel parameters set #1

    - Contuct by offline

  - Step 2: Measurement step - Channel parameter set #2

    - TE transmits Set B beams

  - Step 3: Prediction step

    - UE reports top K predicted beams of set A based on measurement of set B beams.

- Progress B: Ground truth of UE BM performance without AI/ML for BM-case 1

  - Step 1: Measurement step – Channel parameters set #2

    - TE transmits set A beams.

    - UE reports measured RSRP of all beams of set A.

  - Setp 2: TE compares the measured RSRP of the strongest beam and the maximum RSRP of top K predicted beam.

Figure 1: Test steps for AI/ML BM-case 1

In addition, it is essential to randomize the transmission of setA beams during step1 of progress B of Figure 1 to properly test UE’s AI-ML model. Let us look at a particular prediction metric to understand the importance of this.

3GPP studied different performance metrics for AI/ML based beam management during study item discussion. These metrics are captured in TR 38.843:

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

Assume that TE wants to test option 2’s Top-1(%) metric. If the transmission of setA beams is not randomized during measurement step of progressB, UE can simply inform TE that its top-1 strongest beam is same as its top-1 predicted beam of progressA. UE does not even need to measure the beams to predict it. However, UE won’t be able to cheat the test like thisif the transmission of setA beams is randomized during measurement step of progressB.

Based on the illustration of the test steps, we have the following observations and proposal.

Observation 1: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - CDL or simplified CDL to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Model training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic. For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification.

## 2.2 Single AoA test setup

One example test setup with single AoA is shown in Figure 1. For this Single AoA test setup, since there is one physical probe, only TDL channel model can be applied. In the setup of Figure 1, UE can easily predict that beam 1 is stronger than beam 2 even without using any AI-ML model. UE will deterministically know that it will receive beam 1 with higher power than beam 2. No neural network is needed. Moreover, when TE sweeps more beams, many beams will look similar from UE perspective, since in the entire beam pattern of each beam (as plotted above), we only select one AoD angle to observe, and therefore many of the beams look alike even though their beam pattern may look different. That means it would be difficult to differentiate Set A and Set B from UE RSRP perspective. Hence, verifying the AI/ML beam management performance in a TDL channel with single AoA without spatial characteristics makes the test too simple and inconsistent with the actual field environment.

Base station Single path channel DUT Chamber TE emulation to determine probe power Beam 1: arrives at UE with higher gain probe Base station Single path channel DUT Chamber TE emulation to determine probe power Beam 2: arrives at UE with lower gain probe

Figure 1: Predictability of strongest beam with single AoA/probe

Observation 2: Verifying the AI/ML beam management performance (e.g., prediction of top K beams) in a TDL channel with single AoA without spatial characteristics makes the test too simple and inconsistent with the actual field environment. Thus it is not feasible to general proper Set A and Set B with single AoA setup .

Another single AoA test setup solution which is it is quite similar as the radiated two step (RTS) approach used in MIMO OTA testing was proposed in [2]. But the detailed test procedure is not given. Per our understanding, the test procedure could be described as the following two steps:

Step 1: Obtain UE antenna pattern

- Step 1a: For UE position from one of pre-defined Y positions, after dwell time, lock the UE beam and rotating with X directions

  - X and Y are depending on the measurement grid. X and Y could be from the same of different measurement grids

- Step 1b: At each of UE position from X* Y test points, UE reports RSRP values

  - If Rx beam ID is one of the input for AI/ML model, Rx beam ID needs to be recorded with corresponding Tx beam ID and RSRP value

    - Test mode or new signalling are needed

Step 2: Generate BS Tx power under CDL channel model

- Step 2a: Lock the UE Rx beam at a specific direction such as Rx beam peak direction

- Step 2b: Generate BS/TE Tx power according to BS Tx power per Tx beam ID (from Set A or B) and UE antenna pattern got from Step 1

- Step 2c: The received power, Tx beam ID, with or without Rx beam ID as the input for AI/ML model for testing

Observation 3: For single AoA test setup, there are following limitations:

- UE has to report the UE antenna pattern with X*Y test points that is very time consuming.

- UE needs to support additional test function for RSRP reporting

- If Rx beam ID is one of the input for AI/ML model, additional test mode/signalling is needed

- UE’s beam management performance can’t be verified properly since for each of UE position there is enough dewll time and UE beam is locked.

Figure 2: Example test procedure for single AoA test setup

## 2.3 Multiple AoA test setup with 3D MPAC as the baseline

The number of probes supporting multiple Tx beams from the gNB was studied in [3][4][5]. In [5], as shown in Figure 3, when the Tx beam number from the gNB is increased and points to other clusters, the number of probes in the chamber will increase significantly to achieve the PSP metric.

Observation 4: There will be plenty of additional probes if BS Tx beams point to other clusters even though some of the probes could be reused.

Figure 3: Probe number of supporting multiple Tx beam from gNB

It is obvious that the existing system can not be upgraded to support so many probes. To keep the number of probes within a reasonable range, we can consider reducing the number of clusters, i,e., removing the weak clusters in which the power is less than a threshold.

Table 1: Channel model parameters for UMi CDL-C at 28 GH

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.25 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 | -3.4684 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 | -5.2294 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -2.5215 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 | -2.2185 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 | -3.9794 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -7.4215 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -7.1215 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -10.7215 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -11.1215 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -5.1215 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -6.8215 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -8.7215 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -13.2215 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -13.9215 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -13.9215 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -15.8215 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -17.1215 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -16.0215 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -15.7215 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -21.6215 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -22.8215 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

The principle of removing weak clusters is two-fold.

- First, the weak clusters have less impact on the UE beam management performance due to the narrow UE beam width, especially if the clusters fall outside UE’s spherical coverage. For example, an over 10dB gap between peak and 50%-tile spherical coverage is defined for FR2 PC3 UE. And the clusters highlighted in yellow, which have power less than [-10dB], could be removed as they might have a limited impact on beam management.

- Second, the weak clusters are unlikely to contribute to the strongest gNB Tx beam selection. TE will create its own beams based on the predefined multi-beam codebook of setA and setB. The beams created by the TE may not be exactly aligned with the AoDs/ZoDs of different clusters of a CDL model. Hence, the strongest beam may not be adjacent to the AoD and ZoD of the strongest cluster. However, the AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of setA and setB.

Observation 5: The UE’s antenna gain gap between peak and 50%-tile spherical coverage is over 10dB for PC3. The very weak clusters would not influence the UE beam management even if they reach the UE within the UE’s spherical coverage.

Observation 6: The AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of Set A and Set B.

Observation 7: The weak clusters that have a limited impact on the beam management could be removed to reduce the number of clusters in the CDL channel model.

In addition, the clusters that have the same/similar AoAs could be merged (e.g., highlighted by green in Table 1). And the angle spread intra cluster, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup.

Observation 8: The clusters that have the same/similar AoAs could be merged. And the intra-cluster angle spread, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup. And in this case, a new simplified channel model is emulated by test setup.

## 2.4 Multiple AoA test setup with FR2 RRM test setup as the baseline

TDL with multiple AoA/probes test setup was proposed by companies to verify the AI/ML beam management. Figure 4 illustrates an example TDL based multiple AoAs/probes test setup. At each of Tx beam transmitting timeslot, if only one probe is active with TDL channel. It can not emulate multiple AoAs scenario for AI/ML testing so it should not be considered.

Figure 4: Multiple AoAs/probes with TDL channel

Observation 9: To generate Set A and Set B, multiple AoAs/probes test setup is necessary.

To reuse the legacy legacy FR2 RRM test setup, another solution is that multiple AoAs/probes with TDL channel are transmitting simultaneously with additional power offset among the probes using FR2 RRM test setup. An example of test setup is given in Figure 5.

Figure 5: An example test setup for multiple AoAs with TDL on top of legacy RRM test setup

It should be noted that there are 4 physical probes in the current FR2 RRM test setup. The combination of 4 probes with power offset can be considered to emulate more Tx beams.

Theoretically, the TDL based multiple AoAs/Probes setup can be considered as a further simplification on top of simplified CDL model since there is no need to consider implementing multiple clusters as CDL channel model. Following the same principle as section 2.3, the number of Tx beam and probes in TDL shall be similar as simplified CDL channel model. Otherwise, it would not be able to verify realistic AI/ML BM performance.

Observation 10: Using Multiple AoA test setup with FR2 RRM test as the baseline can legacy the FR2 existing test setup to save the test cost and a simplified CDL channel model needs to be specified.

We put the analysis of single AoA and Multiple AoA test setup in below table.

Table 2: Analysis for single AoA and Multiple AoA test setup

|  | Single AoA test setup | Single AoA test setup with two steps | Multiple AoA test setup with 3D MPAC as the baseline | Multiple AoA test setup with FR2 RRM test setup as the baseline |
| --- | --- | --- | --- | --- |
| Chanel model | TDL | CDL | Simplified CDL | Simplified CDL |
| Number of probes | 1 | 1 | 6-8 | 4 |
| Complexity of testing and TE implementation | Low | High Note: Very time consuming for Step 1. And additional test mode or signaling is needed. | Medium Note: Additional probes are needed on top of 3D-MPC | Low Note: Reuse existing FR2 RRM test setup |
| Realistic environment | Low | Low Note: UE beam management performance can not be verified properly. | High/Medium Note: It depends on what does simplified CDL look like | High/Medium Note: It depends on what does simplified CDL look like |

Proposal 1: Single AoA test setup incluidng the use or non-use of the two-step method should not be considerd for AI/ML BM testing. RAN4 should consider the follwoing two multiple AoAs/probes options as the test seup for AI/ML beam management testing

- Option 1: Multiple AoA test setup with 3D MPAC as the baseline

  - Consider reducing the cluster number with weak power level, merging the cluster with the same/similar AoA, and decreasing intra-cluster angle spread in CDL channel model to generte a new simplified CDL channel model

  - Define the paramters for the new simplified CDL channel model based on that 3D-MPAC test setup is used as the baseline

  - Companeis to evlaute whether the new simiplied CDL channel model be used to properly verify AI/ML BM perforance

- Option 2: Multiple AoA test setup with FR2 RRM test setup as the baseline

  - Conider multiple AoA test setup with FR2 RRM test setup as the baseline emulate compareble Tx beam with simplifed CDL, e.g., generating enough Tx beam number the solutions such as combinaiton of probes transmission and power offset for each of probe can be considered.

  - Define the paramters for the new simplified CDL channel model based on that FR2 RRM test setup is used as the baseline

  - Companeis to evlaute whether the new simiplied CDL channel model be used to properly verify AI/ML BM perforance

# 3. Conclusion

In summary, we have following observations and proposals.

Observation 1: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - CDL or simplified CDL to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Model training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic. For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification.

Observation 2: Verifying the AI/ML beam management performance (e.g., prediction of top K beams) in a TDL channel with single AoA without spatial characteristics makes the test too simple and inconsistent with the actual field environment. Thus it is not feasible to general proper Set A and Set B with single AoA setup .

Observation 3: For single AoA test setup, there are following limitations:

- UE has to report the UE antenna pattern with X*Y test points that is very time consuming.

- UE needs to support additional test function for RSRP reporting

- If Rx beam ID is one of the input for AI/ML model, additional test mode/signalling is needed

- UE’s beam management performance can’t be verified properly since for each of UE position there is enough dewll time and UE beam is locked.

Observation 9: To generate Set A and Set B, multiple AoAs/probes test setup is necessary.

Observation 4: There will be plenty of additional probes if BS Tx beams point to other clusters even though some of the probes could be reused.

Observation 5: The UE’s antenna gain gap between peak and 50%-tile spherical coverage is over 10dB for PC3. The very weak clusters would not influence the UE beam management even if they reach the UE within the UE’s spherical coverage.

Observation 6: The AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of Set A and Set B.

Observation 7: The weak clusters that have a limited impact on the beam management could be removed to reduce the number of clusters in the CDL channel model.

Observation 8: The clusters that have the same/similar AoAs could be merged. And the intra-cluster angle spread, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup. And in this case, a new simplified channel model is emulated by test setup.

Observation 10: Using Multiple AoA test setup with FR2 RRM test as the baseline can legacy the FR2 existing test setup to save the test cost and a simplified CDL channel model needs to be specified.

Proposal 1: Single AoA test setup incluidng the use or non-use of the two-step method should not be considerd for AI/ML BM testing. RAN4 should consider the follwoing two multiple AoAs/probes options as the test seup for AI/ML beam management testing

- Option 1: Multiple AoA test setup with 3D MPAC as the baseline

  - Consider reducing the cluster number with weak power level, merging the cluster with the same/similar AoA, and decreasing intra-cluster angle spread in CDL channel model to generte a new simplified CDL channel model

  - Define the paramters for the new simplified CDL channel model based on that 3D-MPAC test setup is used as the baseline

  - Companeis to evlaute whether the new simiplied CDL channel model be used to properly verify AI/ML BM perforance

- Option 2: Multiple AoA test setup with FR2 RRM test setup as the baseline

  - Conider multiple AoA test setup with FR2 RRM test setup as the baseline emulate compareble Tx beam with simplifed CDL, e.g., generating enough Tx beam number the solutions such as combinaiton of probes transmission and power offset for each of probe can be considered.

  - Define the paramters for the new simplified CDL channel model based on that FR2 RRM test setup is used as the baseline

  - Companeis to evlaute whether the new simiplied CDL channel model be used to properly verify AI/ML BM perforance

# 4  References

- R4-2502856, ‘WF on the requirements for AI/ML air interfance’, Qualcomm

- R4-2501726, ‘Discussion on test setup for AI/ML based beam management’, R&S.

- R4-2418848, ‘On testability issues for AI/ML beam management’, Qualcomm

- R4-2411254, ‘OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases’, Keysight

- R4-2411587, ‘Discussion on test setup for AI/ML based beam management’, R&S

- R4-1705831, ‘Metrics for evaluating RRM/Demodulation Measurement Setup’, Keysight

- TR38.827

- TR 38.901

[9] 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.