# 3GPP TSG-RAN WG4 Meeting #114bis                                                R4-2504063

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Sony

Title:	Further views RF requirements of ambient IoT device 1

Agenda item:	7.21.3.2

Document for:	Approval

# Background

The RF requirements of Ambient IoT (AIoT) device type 1 was discussed in RAN4#114,  and some preliminary agreements have been made in [1]. In this contribution, we share our further views on how the RF requirement shall be specified for device type 1.

# System parameters

## R2D bandwidth

In RAN4#114, the following agreements have been made for the values of R2D channel BW:

- Define R2D transmission bandwidth configuration:

  - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS

  - FFS on other RB numbers

- Further discuss the R2D channel bandwidth considering additional guard band.

- If RAN1 has the agreement different than above, RAN4 will revisit the agreements

Meanwhile, RAN1 has made the following agreements with respect to the transmission BW of R2D:

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

- Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

Based on the current situation in RAN1 and RAN4, it is not necessary for RAN4 to determine any other RB numbers than the agreed one at this stage until RAN1 makes a clear conclusion on the minimum RB number for each M value.

Proposal 1: RAN4 waits for RAN1 to determine the minimum RB number for each M value and if any additional RB numbers for D2R transmission BW will be considered.

## D2R bandwidth

For D2R bandwidth, the following agreements have been made:

- D2R channel bandwidth for A-IoT device

  - Define D2R channel bandwidth for A-IoT device considering the following aspects:

    - [1SB or] 2SB

    - D2R maximum data rate (depending on RAN1 conclusion)

    - SFO

    - FDMA, i.e., considering the different device 1 has different frequency shift

    - Reference Filter performance

    - Other aspects are not precluded

Moreover, RAN1 has agreed that

For D2R, only 2SB (Double sideband) modulation is supported.

The DSB spectrum of D2R signal is illustrated in Fig. 1, with also consideration of FDMA with multiple users. In our understanding, the spectrum of the D2R is determined as:

- The center frequency of lower SB and upper SB will be determined by the small frequency shift to be applied

- The bandwidth of lower SB and upper SB is determined by the bit rate of the R2D signal.

Figure. 1. Illustration of DSB modulation of OOK signal.

Moreover, based on the outcome of the SI, an accuracy of 10^5 PPM frequency error can be expected on the tag side. Therefore, a guardband must also be defined to accommodate such an RF impairment if the FDMA scheme would be applied to multiple tags access.

Proposal 2: The bandwidth of D2R needs to cover both lower SB and upper SB and also consider a guardband to acoomandate the frequency error from the tag.

# RF requirement

Currently, the RF core requirement discussion is in parallel to the OTA test setup. Considering the choice of performance metric has large impact on the OTA test system, it is proposed that RAN4 shall first focus on determine the correspondenign performance metric (e.g., TRP/TIS or EIRP/EIS) and test condition (free space or installed on a platform),  then deterimn on the actual value.

Proposal 3: To facilitate the work in OTA test setup, it is proposed that RAN4 shall first focus on determining the correspondenign performance metric (e.g., TRP/TIS or EIRP/EIS) and test condition (free space or installed on a platform).

## Reception requirements.

### REFSENS

One of the key issues in defining the REFSENS requirement is to select a proper performance metric and the corresponding target level. As an example, the main receiver REFSENS of legacy UE is defined at the DL power level at the antenna connector, and the throughput is higher than 95% of the maximum throughput of the reference measurement channels. Meanwhile, the REFSENS level is tentatively agreed to be measured at the power level with a 1% miss detection rate for the low-power wake-up receiver currently under discussion. For AIoT devices, considering the waveform of R2D link is similar to the wake-up sequences and the R2D link is also for paging AIoT devices, adopting the miss detection rate similar to the low-power wake-up receiver is more feasible. The detailed percentage numbers can be further studied in RAN4.

Proposal 4: RAN4 can consider adopting X% miss detection rate as the performance metric to define the REFSENS requirements.

Moreover, for how to derive the REFNSE, RAN4# has also made the following agreement in RAN4#114:

- For R2D reference sensitivity, consider following options:

  - Option 1: define reference sensitivity using legacy approach, e.g. PREFSENSdBm=-174dBm+10×log10BW+NF+IM+SNR

  - Option 2: define reference sensitivity based on link budget and/or physical limits of the receiver circuitry.

  - Other options are not precluded.

For an RF-ED detector, how to derive the REFSENS may highly depend on the receiver's implementation, particularly if it is implemented with a fixed comparator or 1-bit ADC would be used.

#### RF-ED detector with a 1-bit ADC without a fixed threshold

A legacy UE receiver comes with an ADC to quantize the signal level, which does not have fixed threshold levels. Similarly, a 1-bit ADC can be used for RF-ED receivers, e.g., sigma-delta ADC, which measures the derivative of the signal rather than the absolute level. In this case, the legacy approach can be used to calculate the REFSENS as in (1).

REFSENS=-174 dBm+10*logBW+NF+SNR+IM.                              (1)

Based on the agreed performance metric, RAN4 needs to derive the required SNR at the input to baseband (BB) to meet the target performance metric.

In addition to SNR, the noise figure (NF) is another key parameter that needs to be determined. NF measures the SNR degradation between the input and output of the receiver, and thus, aligning the assumption on the reference architecture of the R2D receiver is critical as well. In this paper, the following assumed architecture is adopted:

Fig. 2. The assumed architecture of R2D receiver to derive the REFSENS requirement.

(2)

A 1-bit ADC can have an NF in the range of 10-15 dB. The RF-ED receiver, including the BB-LP, has a lower bound of 10 dB NF. Assuming an ideal circuit with square law modeling, such a high NF comes to form the fact that the noise outside the BW will still be partially mixed into the BB via the self-mixing effect of the RF-ED receiver. In addition, for a practical implementation, the NF can be in the range of up to 20 dB, considering the loss, impedance mismatch, non-ideal diode, etc. Thus, the cascaded NF of the receiver shown in Fig. 1 would be dominated by the RF-ED receiver, which is around 22 dB.

For legacy NR receivers, a 2 dB implementation margin (IM) is usually considered while considering the extremely low cost of the ambient IoT device 1, a higher IM, e.g., 3dB, may be assumed. Assuming that 10 dB SNR as input to BB is required to meet the proposed performance metric and 180 kHz signal BW, this will result in a REFSENS level of about -85 dBm.

Observation 1: The REFSENS level of R2D receiver could be in the range of -85 dBm/180 kHz, pending on the agreed performance metric if a 1-bit ADC without a fixed threshold is used.

#### RF-ED detector with a comparator with a fixed threshold

However, unlike a 1-bit ADC, a simple comparator circuit usually comes with a fixed threshold value, and the legacy approach may not be feasible. Operating with a comparator requires the incoming power level for "1" to be above the threshold level; thus, the comparator threshold level will determine the REFSENS of the receiver. For example, a typical low-power comparator can support a threshold level of about -30 dBm. Since this power level is likely much higher than the noise floor if the receiver, the SNR for decoding on the baseband is no longer critical, and the REFSENS can be defined based on the threshold value.

Observation 2: The REFSENS level of the R2D receiver would be determined by the threshold level of the comparator if a comparator with a fixed threshold is used, and likely this level is much higher than the noise level of the receiver.

Considering the huge performance difference between the different implementation choices, RAN4 needs to determine the reference architecture of the R2D receiver with consideration of the link budget of the system, then decide the proper approach to derive the REFSENS.

Proposal 5: RAN4 needs to determine the reference architecture of the R2D receiver and then decide the proper approach to derive the REFSENS.

### Maximum input level

For maximum input level, it was agreed in RAN4#114 that further discussion is needed. For the RF-ED receiver, typically, the diode operates in the square-law region below the linear region, as illustrated in Fig. 3. Therefore, it is important to ensure that the device can still operate properly. A typical square law region for a Schottky diode is in the range below -20 dBm, which can be a starting point to be considered as the maximum input level.

Fig. 3. The operating region of a diode.

Proposal 6: The maximum input level of R2D needs to be defined to ensure the RF-ED receiver operates in the proper region, where -20 dBm may be considered a starting point.

## Transmission part (D2R)

### The transmitted power

For transmitted power from the tag on the D2R, it depends on both the tag implementation and the property of the incoming CW signal, and it was agreed in RAN4#114 that measurement configuration, including input CW power and potentially other conditions, needs to be defined.

From the tag implementation aspect, the RCS of the tag determines the backscattering loss (or gain), which includes the realized gain on the direction of the incoming and outcoming signal and the reflection coefficient on the load impedance. Moreover, the modulation scheme would also affect the power level of the backscatter signal, as the tag would absorb some of the signal due to the "0" if the tag adopts an on-off keying scheme. Therefore, all those factors need to be taken into account.

Proposal 7: To determine the backscattering loss of the tag, the RCS of the tag and the modulation scheme needs to be taken into account.

### Output RF spectrum emissions

For the RF spectrum emission, the following agreement was made in RAN4#114:

- Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point.

- Further discuss whether to define TRP or EIRP requirement for unwanted emission.

At least for the spurious emission region, it is proposed to re-use the legacy requirements as the spurious emission's purpose is to co-exist with the system in other frequency bands. Then, RAN4 can further discuss how to integrate the SEM into the same composite emission requirements.

Proposal 8: Re-use the legacy spurious emission requirement for AIoT device 1 as a starting point to determine the RF spectrum emission requirement.

### Transmitted signal quality

For transmitted signal quality, as the small frequency shift method has been considered in the WI [1], it is critical to ensure the frequency accuracy after such an operation. Therefore, at least a frequency error requirement is needed.

For device type 1, considering the extremely low power budget, it is impossible to implement any additional oscillators in the receiver. The only method to support the frequency shift is via switching speed control.

Observation 3: The limited power budget of device 1 may not allow any additional oscillator, and the small frequency shift needs to be realized via controlling the switching speed of the load impedance.

The typical RF switch at 900 MHz used in smartphones has a switching speed of a couple of us, and thus the maximum frequency shift would be around a few kHz to 1 MHz. The stability of the switching speed may be affected by the thermal effect, power supply noise, and the technology of the RF switch. Based on the outcome of the SI, a accuracy of 10^5 PPM can be expected, which is also inlne with our analysis [3].

For the AIoT system, as the FDMA access is expected, it is important to define the frequency error requirements to ensure the users do not interfere each other’s transmission.

Observation 4: The small frequency accuracy would be in the range of about 10^5 PPM after clock sync.

Proposal 9: The frequency error requirement will need to be specified to support FDMA scheme.

In addition to the frequency error, some requirements are also needed to ensure the modulation quality of the backscattered signal. For on-off keying, the legacy EVM definition may not be able to be applied directly, so some new measures may need to be specified here.

Proposal 10: For BPSK, the EVM requirement can be defined, while for OOK, Some requirements to ensure the modulation quality of the backscattered signal need to be studied and specified.

# Conclusion

In this paper, we provide on the RF requirement for AIoT device 1, where the following observations and proposals have been made:

Observation 1: the REFSENS level of R2D receiver could be in the range of -85 dBm/180 kHz, pending on the agreed performance metric if a 1-bit ADC without a fixed threshold is used.

Observation 2: The REFSENS level of the R2D receiver would be determined by the threshold level of the comparator if a comparator with a fixed threshold is used, and likely this level is much higher than the noise level of the receiver.

Observation 3: The limited power budget of device 1 may not allow any additional oscillator, and the small frequency shift needs to be realized via controlling the switching speed of the load impedance.

Observation 4: The small frequency accuracy would be in the range of about 10^5 PPM after clock sync.

Proposal 1: RAN4 waits for RAN1 to determine the minimum RB number for each M value and if any additional RB numbers for D2R transmission BW will be considered.

Proposal 2: The bandwidth of D2R needs to cover both lower SB and upper SB and also consider a guardband to acoomandate the frequency error from the tag.

Proposal 3: To facilitate the work in OTA test setup, it is proposed that RAN4 shall first focus on determining the correspondenign performance metric (e.g., TRP/TIS or EIRP/EIS) and test condition (free space or installed on a platform)

Proposal 4: RAN4 can consider adopting X% miss detection rate as the performance metric to define the REFSENS requirements.

Proposal 5: RAN4 needs to determine the reference architecture of the R2D receiver and then decide the proper approach to derive the REFSENS.

Proposal 6: The maximum input level of R2D needs to be defined to ensure the RF-ED receiver operates in the proper region, where -20 dBm may be considered a starting point.

Proposal 7: To determine the backscattering loss of the tag, the RCS of the tag and the modulation scheme needs to be taken into account.

Proposal 8: Re-use the legacy spurious emission requirement for AIoT device 1 as a starting point to determine the RF spectrum emission requirement.

Proposal 9: The frequency error requirement will need to be specified to support FDMA scheme.

Proposal 10: For BPSK, the EVM requirement can be defined, while for OOK, Some requirements to ensure the modulation quality of the backscattered signal need to be studied and specified.

# References

- R4-2502858 WF on requirements for system parameters and A-IOT device CMCC