3GPP TSG-RAN WG4 Meeting #114bis	R4-2504081
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.14.2.3

Source:	CMCC

Title:	On dynamic MIMO OTA channel models reference target values

Document for:	Approval

# 1	Introduction

In last RAN4 #114 meeting, some agreements above dynamic MIMO OTA channel model PDP target values have been achieved, which is as follows.

In this contribution, we update the reference target values for both UMa and UMi channel models based on the agreements, and comparison results between CMCC and CE vendors have been presented for convenience.

# 2	Discussion

The updated reference target values of FR1 dynamic channel models include PL, PDP, TCF, SCF and XPR resutls for UMa and UMi channel models. The details are as follows.

- PL target values

- PL target values for Uma

Figure 2.1.1-1 PL comparison results for UMa channel model

Table 2.1.1-1 PL comparison results for UMa channel model

| Segment | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -0.81 | -0.8 | -0.01 |
| 2 | 0 | 0 | 0 |
| 3 | -29.51 | -29.50 | -0.01 |
| 4 | -27.59 | -27.60 | 0.01 |
| 5 | -21.55 | -21.50 | -0.05 |
| 6 | -30.11 | -30.10 | -0.01 |
| 7 | -0.06 | -0.1 | 0.04 |
| 8 | -19.85 | -19.80 | -0.05 |
| 9 | -0.69 | -0.7 | 0.01 |

- PL target values for Umi

Figure 2.1.2-1 PL comparison results for UMi channel model

Table 2.1.2-1 PL comparison results for UMi channel model

| Segment | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -7.53 | -7.5 | -0.03 |
| 2 | 0 | 0 | 0 |
| 3 | -16.58 | -16.60 | 0.02 |
| 4 | -12.92 | -12.90 | -0.02 |
| 5 | -16.47 | -16.50 | 0.03 |
| 6 | -0.93 | -0.9 | -0.03 |
| 7 | -0.67 | -0.7 | 0.03 |

- PDP target values

- PDP target values for UMa

Figure 2.2.1-1: PDP comparison results for UMa channel model segment 1

Table 2.2.1-1: PDP comparison results for UMa channel model segment 1

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 130 | -18.53 | -18.50 | -0.030 |
| 170 | -20.15 | -20.20 | 0.050 |
| 245 | -22.55 | -22.50 | -0.050 |
| 380 | -36.02 | -36 | -0.020 |
| 745 | -25.53 | -25.50 | -0.030 |
| 885 | -31.87 | -31.90 | 0.030 |

Figure 2.2.1-2: PDP comparison results for UMa channel model segment 2

Table 2.2.1-2: PDP comparison results for UMa channel model segment 2

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 130 | -19.18 | -19.20 | 0.020 |
| 170 | -20.91 | -20.90 | -0.010 |
| 245 | -23.18 | -23.20 | 0.020 |
| 380 | -32.76 | -32.80 | 0.040 |
| 745 | -23.87 | -23.90 | 0.030 |
| 885 | -34.49 | -34.50 | 0.010 |

Figure 2.2.1-3: PDP comparison results for UMa channel model segment 3

Table 2.2.1-3: PDP comparison results for UMa channel model segment 3

| Cluster Index | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -9.830 | -9.8 | -0.030 |
| 2 | 0 | 0 | 0 |
| 3 | -1.850 | -1.9 | 0.050 |
| 4 | -6.3 | -6.3 | 0 |
| 5 | -8.210 | -8.2 | -0.010 |
| 6 | -6.550 | -6.6 | 0.050 |
| 7 | -3.310 | -3.3 | -0.010 |
| 8 | -8.9 | -8.9 | 0 |
| 9 | -8.350 | -8.3 | -0.050 |
| 10 | -11.64 | -11.70 | 0.060 |
| 11 | -8.540 | -8.5 | -0.040 |
| 12 | -16.27 | -16.30 | 0.030 |
| 13 | -10.03 | -10 | -0.030 |
| 14 | -13.36 | -13.40 | 0.040 |
| 15 | -12.10 | -12.10 | 0 |
| 16 | -12.81 | -12.80 | -0.010 |
| 17 | -29.26 | -29.30 | 0.040 |
| 18 | -28.96 | -29 | 0.040 |
| 19 | -18.41 | -18.40 | -0.010 |
| 20 | -19.34 | -19.30 | -0.040 |
| 21 | -17.54 | -17.50 | -0.040 |
| 22 | -31.88 | -31.90 | 0.020 |
| 23 | -29.78 | -29.80 | 0.020 |

Figure 2.2.1-4: PDP comparison results for UMa channel model segment 4

Table 2.2.1-4: PDP comparison results for UMa channel model segment 4

| Cluster Index | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -15.03 | -15 | -0.030 |
| 2 | 0 | 0 | 0 |
| 3 | -2.020 | -2 | -0.020 |
| 4 | -9.7 | -9.7 | 0 |
| 5 | -11.80 | -11.80 | 0 |
| 6 | -14.22 | -14.20 | -0.020 |
| 7 | -3.850 | -3.9 | 0.050 |
| 8 | -13.31 | -13.30 | -0.010 |
| 9 | -6.870 | -6.9 | 0.030 |
| 10 | -20.79 | -20.80 | 0.010 |
| 11 | -6.490 | -6.5 | 0.010 |
| 12 | -16.01 | -16 | -0.010 |
| 13 | -18.94 | -19 | 0.060 |
| 14 | -17.54 | -17.50 | -0.040 |
| 15 | -12.32 | -12.30 | -0.020 |
| 16 | -11.31 | -11.30 | -0.010 |
| 17 | -28.17 | -28.20 | 0.030 |
| 18 | -29.06 | -29.10 | 0.040 |
| 19 | -22.33 | -22.30 | -0.030 |
| 20 | -18.22 | -18.20 | -0.020 |
| 21 | -17.20 | -17.20 | 0 |
| 22 | -30.33 | -30.30 | -0.030 |
| 23 | -27.96 | -28 | 0.040 |

Figure 2.2.1-5: PDP comparison results for UMa channel model segment 5

Table 2.2.1-5: PDP comparison results for UMa channel model segment 5

| Cluster Index | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -8.220 | -8.2 | -0.020 |
| 2 | -0.80 | -0.80 | 0 |
| 3 | -3.680 | -3.7 | 0.020 |
| 4 | -3.480 | -3.5 | 0.020 |
| 5 | -5.230 | -5.2 | -0.030 |
| 6 | 0 | 0 | 0 |
| 7 | -1.610 | -1.6 | -0.010 |
| 8 | -3.710 | -3.7 | -0.010 |
| 9 | -10.35 | -10.40 | 0.050 |
| 10 | -10.06 | -10.10 | 0.040 |
| 11 | -13.35 | -13.40 | 0.050 |
| 12 | -22.48 | -22.50 | 0.020 |
| 13 | -7.930 | -7.9 | -0.030 |
| 14 | -13.52 | -13.50 | -0.020 |
| 15 | -16.16 | -16.20 | 0.040 |
| 16 | -18.89 | -18.90 | 0.010 |
| 17 | -23.05 | -23.10 | 0.050 |
| 18 | -21.38 | -21.40 | 0.020 |
| 19 | -23.33 | -23.40 | 0.070 |
| 20 | -22.46 | -22.50 | 0.040 |
| 21 | -20.27 | -20.30 | 0.030 |
| 22 | -26.47 | -26.50 | 0.030 |
| 23 | -33.04 | -33.10 | 0.060 |

Figure 2.2.1-6: PDP comparison results for UMa channel model segment 6

Table 2.2.1-6: PDP comparison results for UMa channel model segment 6

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | -12.99 | -13 | 0.010 |
| 80 | -3.350 | -3.3 | -0.050 |
| 235 | 0 | 0 | 0 |
| 3 | -12.66 | -12.60 | -0.060 |
| 340 | -33.29 | -33.30 | 0.010 |
| 445 | -15.84 | -15.80 | -0.040 |
| 475 | -19.74 | -19.70 | -0.040 |
| 790 | -25.14 | -25.10 | -0.040 |
| 985 | -18 | -18 | 0 |
| 1550 | -17.90 | -17.80 | -0.10 |
| 1675 | -16.46 | -16.40 | -0.060 |
| 2040 | -20.58 | -20.40 | -0.18 |
| 2295 | -19.20 | -19 | -0.20 |
| 2415 | -20.89 | -20.70 | -0.19 |
| 2565 | -29.56 | -29.40 | -0.16 |

Figure 2.2.1-7: PDP comparison results for UMa channel model segment 7

Table 2.2.1-7: PDP comparison results for UMa channel model segment 7

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 50 | -17.05 | -17.10 | 0.050 |
| 65 | -21.15 | -21.20 | 0.050 |
| 175 | -15.03 | -15 | -0.030 |
| 240 | -21.04 | -21 | -0.040 |
| 335 | -24.92 | -24.90 | -0.020 |
| 495 | -19.92 | -19.90 | -0.020 |
| 1090 | -32.87 | -32.80 | -0.070 |
| 1875 | -32.87 | -32.80 | -0.070 |

Figure 2.2.1-8: PDP comparison results for UMa channel model segment 8

Table 2.2.1-8: PDP comparison results for UMa channel model segment 8

| Cluster Index | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | 0 | 0 | 0 |
| 2 | -5.6 | -5.7 | 0.10 |
| 3 | -7.810 | -7.9 | 0.090 |
| 4 | -11.36 | -11.50 | 0.14 |
| 5 | -15.10 | -15.20 | 0.10 |
| 6 | -15.25 | -15.40 | 0.15 |
| 7 | -8.950 | -9.1 | 0.15 |
| 8 | -13.92 | -14.10 | 0.18 |
| 9 | -13.79 | -14 | 0.21 |
| 10 | -19.39 | -19.60 | 0.21 |
| 11 | -12.10 | -12.30 | 0.20 |
| 12 | -19.72 | -19.90 | 0.18 |
| 13 | -25.09 | -25.20 | 0.11 |
| 14 | -20.70 | -20.90 | 0.20 |
| 15 | -26.50 | -26.60 | 0.10 |
| 16 | -27.73 | -27.90 | 0.17 |
| 17 | -22.96 | -23.10 | 0.14 |
| 18 | -25.44 | -25.60 | 0.16 |
| 19 | -26.13 | -26.30 | 0.17 |
| 20 | -33.22 | -33.40 | 0.18 |
| 21 | -35.74 | -35.90 | 0.16 |
| 22 | -27.39 | -27.50 | 0.11 |
| 23 | -38.64 | -38.80 | 0.16 |

Figure 2.2.1-9: PDP comparison results for UMa channel model segment 9

Table 2.2.1-9: PDP comparison results for UMa channel model segment 9

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 60 | -36.14 | -36.10 | -0.040 |
| 130 | -17.93 | -17.90 | -0.030 |
| 170 | -19.44 | -19.50 | 0.060 |
| 245 | -21.97 | -22 | 0.030 |
| 380 | -38.85 | -38.80 | -0.050 |
| 745 | -26.98 | -27 | 0.020 |
| 885 | -29.55 | -29.50 | -0.050 |
| 1180 | -38.75 | -38.70 | -0.050 |

- PDP target values for UMi

Figure 2.2.2-1: PDP comparison results for UMi channel model segment 1

Table 2.2.2-1: PDP comparison results for UMi channel model segment 1

| Cluster number | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | 0 | 0 | 0 |
| 2 | -21.4 | -21.9 | 0.500 |
| 3 | -22.09 | -22.5 | 0.410 |
| 4 | -23.86 | -24.3 | 0.440 |
| 5 | -16.45 | -16.8 | 0.350 |
| 6 | -16.29 | -16.7 | 0.410 |
| 7 | -16.15 | -16.5 | 0.350 |
| 8 | -17.88 | -18.3 | 0.420 |
| 9 | -25.14 | -25.5 | 0.360 |
| 10 | -24.11 | -24.5 | 0.390 |
| 11 | -29.17 | -29.6 | 0.430 |
| 12 | -23.64 | -24 | 0.360 |
| 13 | -29.55 | -30 | 0.450 |
| 14 | -34.15 | -34.7 | 0.550 |
| 15 | -39.3 | -40 | 0.700 |
| 16 | -39.22 | -39.9 | 0.680 |
| 17 | -37.16 | -37.8 | 0.640 |

Figure 2.2.2-2: PDP comparison results for UMi channel model segment 2

Table 2.2.2-2: PDP comparison results for UMi channel model segment 2

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 35 | -34.84 | -35 | 0.160 |
| 75 | -17.41 | -17.4 | -0.010 |
| 100 | -21.11 | -18.8 | -2.31 |
| 140 | -21.46 | -21.5 | 0.040 |
| 430 | -31.57 | -28.8 | -2.77 |
| 510 | -30.84 | -28.2 | -2.64 |
| 680 | -56.56 | -37.5 | -19.06 |

Figure 2.2.2-3: PDP comparison results for UMi channel model segment 3

Table 2.2.2-3: PDP comparison results for UMi channel model segment 3

| Cluster number | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -17.01 | -17.1 | 0.090 |
| 2 | 0 | 0 | 0 |
| 3 | -2.82 | -2.80 | -0.020 |
| 4 | -2.84 | -2.80 | -0.040 |
| 5 | -4.79 | -4.80 | 0.010 |
| 6 | -1.56 | -1.60 | 0.040 |
| 7 | -2 | -2 | 0 |
| 8 | -4.42 | -4.40 | -0.020 |
| 9 | -5.87 | -5.90 | 0.030 |
| 10 | -9.03 | -9 | -0.030 |
| 11 | -6.08 | -6.10 | 0.020 |
| 12 | -26.15 | -26.4 | 0.250 |
| 13 | -7.54 | -7.50 | -0.040 |
| 14 | -21.31 | -21.5 | 0.190 |
| 15 | -20.15 | -20.3 | 0.150 |
| 16 | -10.74 | -10.7 | -0.040 |
| 17 | -15.07 | -15.2 | 0.130 |
| 18 | -16.1 | -16.2 | 0.100 |
| 19 | -16.83 | -16.8 | -0.030 |
| 20 | -16.81 | -16.8 | -0.010 |
| 21 | -15.55 | -15.5 | -0.050 |
| 22 | -19.06 | -19.2 | 0.140 |
| 23 | -23.57 | -23.6 | 0.030 |

Figure 2.2.2-4: PDP comparison results for UMi channel model segment 4

Table 2.2.2-4: PDP comparison results for UMi channel model segment 4

| Cluster number | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -11.18 | -11.3 | 0.120 |
| 2 | 0 | 0 | 0 |
| 3 | -2.50 | -2.50 | 0 |
| 4 | -4.25 | -4.30 | 0.050 |
| 5 | -6.13 | -6.10 | -0.030 |
| 6 | -2.02 | -2 | -0.020 |
| 7 | -1.99 | -2 | 0.010 |
| 8 | -5.21 | -5.20 | -0.010 |
| 9 | -7.73 | -7.80 | 0.070 |
| 10 | -11.46 | -11.5 | 0.040 |
| 11 | -8.33 | -8.40 | 0.070 |
| 12 | -21.67 | -21.8 | 0.130 |
| 13 | -9.26 | -9.30 | 0.040 |
| 14 | -15.82 | -15.9 | 0.080 |
| 15 | -16.45 | -16.6 | 0.150 |
| 16 | -13.56 | -13.6 | 0.040 |
| 17 | -20.77 | -21 | 0.230 |
| 18 | -20.58 | -20.8 | 0.220 |
| 19 | -19.49 | -19.5 | 0.010 |
| 20 | -19.27 | -19.3 | 0.030 |
| 21 | -17.93 | -18 | 0.070 |
| 22 | -24.19 | -24.4 | 0.210 |
| 23 | -27.65 | -27.7 | 0.050 |

Figure 2.2.2-5: PDP comparison results for UMi channel model segment 5

Table 2.2.2-5: PDP comparison results for UMi channel model segment 5

| Cluster number | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 1 | -10.95 | -11 | 0.050 |
| 2 | 0 | 0 | 0 |
| 3 | -1.95 | -2 | 0.050 |
| 4 | -5.59 | -5.60 | 0.010 |
| 5 | -7.52 | -7.50 | -0.020 |
| 6 | -5.85 | -5.90 | 0.050 |
| 7 | -3.07 | -3.10 | 0.030 |
| 8 | -8.12 | -8.20 | 0.080 |
| 9 | -8.04 | -8.10 | 0.060 |
| 10 | -11.13 | -11.2 | 0.070 |
| 11 | -8.44 | -8.40 | -0.040 |
| 12 | -17.45 | -17.6 | 0.150 |
| 13 | -9.42 | -9.40 | -0.020 |
| 14 | -14.3 | -14.4 | 0.100 |
| 15 | -13.21 | -13.3 | 0.090 |
| 16 | -12.51 | -12.5 | -0.010 |
| 17 | -26.78 | -27.1 | 0.320 |
| 18 | -26.46 | -26.7 | 0.240 |
| 19 | -17.51 | -17.5 | -0.010 |
| 20 | -18.8 | -18.8 | 0 |
| 21 | -16.61 | -16.6 | -0.010 |
| 22 | -29.83 | -30.1 | 0.270 |
| 23 | -28.8 | -28.9 | 0.100 |

Figure 2.2.2-6: PDP comparison results for UMi channel model segment 6

Table 2.2.2-6: PDP comparison results for UMi channel model segment 6

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 30 | -20.45 | -20.5 | 0.050 |
| 100 | -16.9 | -17.4 | 0.500 |
| 140 | -28.81 | -25.9 | -2.91 |
| 195 | -33.54 | -33.6 | 0.060 |
| 285 | -20.04 | -20 | -0.040 |

Figure 2.2.2-7: PDP comparison results for UMi channel model segment 7

Table 2.2.2-7: PDP comparison results for UMi channel model segment 7

| Delay [ns] | CMCC | KS | Offset |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 30 | -20.88 | -20.9 | 0.020 |
| 100 | -16.08 | -16.5 | 0.420 |
| 140 | -27.09 | -24.2 | -2.89 |
| 195 | -30.54 | -30.6 | 0.060 |
| 285 | -20.04 | -20 | -0.040 |

- TCF target values

- TCF target values for UMa

Table 2.3.1-1 TCF comparison results for UMa channel model at Δt=4.6 ms

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.97 | 0.96 | 0.98 | 0.45 | 0.67 | 0.55 | 0.81 | 0.79 | 0.94 |
| KS | 0.97 | 0.96 | 0.99 | 0.44 | 0.65 | 0.52 | 0.80 | 0.84 | 0.94 |
| Offset | 0 | 0 | -0.010 | 0.010 | 0.020 | 0.030 | 0.010 | -0.040 | 0 |

Table 2.3.1-2 TCF comparison results for UMa channel model at Δt=20.2 ms

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.88 | 0.88 | 0.87 | 0.36 | 0.35 | 0.12 | 0.23 | 0.40 | 0.88 |
| KS | 0.88 | 0.88 | 0.89 | 0.36 | 0.35 | 0.10 | 0.23 | 0.40 | 0.88 |
| diff | 0 | 0 | -0.020 | 0 | 0 | 0.020 | 0.010 | 0 | 0 |

- TCF target values for UMi

Table 2.3.2-1 TCF comparison results for UMi channel model at Δt=4.6 ms

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.66 | 0.95 | 0.54 | 0.75 | 0.75 | 0.96 | 0.97 |
| KS | 0.87 | 0.95 | 0.51 | 0.70 | 0.74 | 0.96 | 0.97 |
| diff | -0.21 | 0 | 0.030 | 0.050 | 0.010 | 0 | 0 |

Table 2.3.2-2 TCF comparison results for UMi channel model at Δt=20.2 ms

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.33 | 0.86 | 0.19 | 0.050 | 0.37 | 0.82 | 0.85 |
| KS | 0.44 | 0.86 | 0.20 | 0.070 | 0.37 | 0.81 | 0.85 |
| diff | -0.11 | 0 | 0 | -0.020 | 0 | 0 | 0 |

- SCF target values

- SCF target values for Uma

Table 2.4.1-1 SCF comparison results for UMa channel model at sampling point 2

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.99 | 0.96 | 0.80 | 0.70 | 0.83 | 0.79 | 0.93 | 0.83 | 0.96 |
| KS | 0.99 | 0.96 | 0.80 | 0.70 | 0.83 | 0.79 | 0.93 | 0.83 | 0.96 |
| Offset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -0.010 | 0 |

Table 2.4.1-2 SCF comparison results for UMa channel model at sampling point 5

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.91 | 0.61 | 0.16 | 0.72 | 0.42 | 0.39 | 0.48 | 0.27 | 0.58 |
| KS | 0.91 | 0.61 | 0.16 | 0.72 | 0.41 | 0.39 | 0.48 | 0.28 | 0.57 |
| Offset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -0.010 | 0 |

Table 2.4.1-3 SCF comparison results for UMa channel model at sampling point 11

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.93 | 0.32 | 0.11 | 0.49 | 0.16 | 0.46 | 0.37 | 0.55 | 0.21 |
| KS | 0.93 | 0.33 | 0.11 | 0.49 | 0.16 | 0.45 | 0.37 | 0.61 | 0.22 |
| Offset | 0 | -0.010 | 0 | 0 | 0 | 0 | 0.010 | -0.050 | -0.010 |

- SCF target values for UMi

Table 2.4.2-1 SCF comparison results for UMi channel model at sampling point 2

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.94 | 0.98 | 0.79 | 0.76 | 0.86 | 0.97 | 0.98 |
| KS | 0.94 | 0.98 | 0.79 | 0.76 | 0.86 | 0.97 | 0.98 |
| Offset | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 2.4.2-2 SCF comparison results for UMi channel model at sampling point 5

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.76 | 0.94 | 0.22 | 0.33 | 0.58 | 0.81 | 0.95 |
| KS | 0.76 | 0.94 | 0.23 | 0.33 | 0.57 | 0.81 | 0.95 |
| Offset | 0 | 0 | -0.010 | 0 | 0.010 | 0.010 | 0 |

Table 2.4.2-3 SCF comparison results for UMi channel model at sampling point 11

| Segment Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC | 0.74 | 0.94 | 0.16 | 0.32 | 0.29 | 0.69 | 0.94 |
| KS | 0.73 | 0.94 | 0.15 | 0.31 | 0.29 | 0.69 | 0.94 |
| Offset | 0.010 | 0 | 0.010 | 0.010 | 0 | 0.010 | 0 |

- XPR target values

- XPR target values for Uma

Table 2.5.1-1 XPR comparison results for UMa channel model

| Segment | CMCC | KS | diff |
| --- | --- | --- | --- |
| 1 | 22.20 | 22.20 | 0 |
| 2 | 22.38 | 22.40 | -0.02 |
| 3 | 8.620 | 8.5 | 0.12 |
| 4 | 9.550 | 9.5 | 0.05 |
| 5 | 7.390 | 7.4 | -0.01 |
| 6 | 7 | 7 | 0 |
| 7 | 18.51 | 18.50 | 0.01 |
| 8 | 11.39 | 11.60 | -0.21 |
| 9 | 21.95 | 22 | -0.05 |

- XPR target values for Umi

Table 2.5.2-1 XPR comparison results for UMi channel model

| Segment | CMCC | KS | diff |
| --- | --- | --- | --- |
| 1 | 17.85 | \ | \ |
| 2 | 21.73 | \ | \ |
| 3 | 8.710 | \ | \ |
| 4 | 8.210 | \ | \ |
| 5 | 8.630 | \ | \ |
| 6 | 21.14 | \ | \ |
| 7 | 20.67 | \ | \ |

Observation 1: For UMa channel model, reference target values from CMCC have extremely small offset with those from CE vendors, we suppose the alignment of UMa channel model is achieved and final reference results can derive from these two sets of target values.

Observation 2: For UMi channel model, there is some offset of PDP at some segments, and it maybe related to the delay resolution issue, one more meeting is needed to get the final alignment.

Proposal 1: The XPR results from CE vendors of UMi channel model is missing now, CE vendors can supply the XPR results for further comparison.

Proposal 2: RAN4 can give one more meeting time for UMi channel model alignment between CMCC and CE vendors.

# 3	Conclusion

Observation 1: For UMa channel model, reference target values from CMCC have extremely small offset with those from CE vendors, we suppose the alignment of UMa channel model is achieved and final reference results can derive from these two sets of target values.

Observation 2: For UMi channel model, there is some offset of PDP at some segments, and it maybe related to the delay resolution issue, one more meeting is needed to get the final alignment.

Proposal 1: The XPR results from CE vendors of UMi channel model is missing now, CE vendors can supply the XPR results for further comparison.

Proposal 2: RAN4 can give one more meeting time for UMi channel model alignment between CMCC and CE vendors.

# References

- R4- 2502176, TP for TR 38.762: DynMIMO CM Validation Details and Corrections, Keysight Technologies UK Ltd, Spirent Communications, RAN4 #114, Feb. 2025.