3GPP TSG-RAN WG4 Meeting #114bis                                        R4-2504090

Wuhan, China, April 7 - 11, 2025

Source: 	Huawei, HiSilicon

Title: 	On RF requirements for Rel-19 NR-MIMO

Agenda Item:	7.22.2

Document for:	Approval

Introduction

In previous meeting, RAN4 further discussed on this WI. The progress on FR1 3Tx and the proposed scenario for mTRP operation in FR2 can be found in [1].

<Way forward>: PC for 3Tx requirement - For PC3 and PC2, specify requirements for FWA and handheld UE - FFS on PC1.5 with 3Tx, will be decided in the next meeting - Option 1: Consider it in the next release - Option 2: Specify requirements in Rel-19 <Way forward>: PC2 for 3Tx - Option 1: Use the same MPR as MPR for PC2 with 2Tx, as it’s for UL-MIMO and from simulation results - Option 2: Use the same MPR as MPR for PC2 with 1Tx, as it’s the worst case <Way forward>: RAN4 will decide between following options in the next meeting - Option 1: No UE RF impact of asymmetric DL sTRP/UL mTRP - Option 2: Adopt following dCR, where the companies are encouraged to propose wording improvements in the next meeting

In this contribution, we would like to provide our views on this topic.

Discussion

On FR1 3Tx operation

Per RAN4 discussion progress so far, most of the issues regarding the introduction of FR1 3Tx for single carrier operation have been tackled. One remaining issue that haunts people is whether to introduce PC1.5 for 3Tx in Rel-19. Like we have expressed, which seems also shared among companies, it may be better not to rush for a wider spec implementation for 3Tx in this release especially considering how many limitations had been set during the WI establishing phase. Also in light of the concerns about whether the industry could be ready to support high power classes with more activated Tx chains, our preference is not to further consider PC1.5 for 3Tx in Rel-19.

Proposal 1: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5.

As for the PC2 MPR requirement for 3Tx, we don’t think the application of 1Tx PC2 MPR requirements would be the “worst case”. Given that 3 layers UL-MIMO is still allowed per WI scope, normally some implementation margin could be reserved so that UE can handle the e.g. intermodulation between Tx chains. Thus we think an acceptable solution would be directly reuse the PC2 MPR requirement for 2Tx.

Proposal 2: Reuse the PC2 2Tx MPR requirement for the MPR requirement for PC2 with 3Tx.

Conclusion

In this contribution we discussed on the RF requirements for Rel-19 NR MIMO. We have the following proposals:

Proposal 1: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5.

Proposal 2: Reuse the PC2 2Tx MPR requirement for the MPR requirement for PC2 with 3Tx.

- References

- R4-2503037, “WF on UE RF requirements for MIMO enhancement”, Samsung, 3GPP TSG RAN4 Meeting #114, Athens, Greece, February 17-21, 2024.