3GPP TSG-RAN WG4 Meeting #114bis                                        R4-2504092

Wuhan, China, April 7 - 11, 2025

Source: 	Huawei, HiSilicon

Title: 	On potentially new requirements for SBFD

Agenda Item:	7.23.2.1

Document for:	Approval

Introduction

During RAN4#114, potentially new requirements for SBFD have been discussed. Following conclusions and FFS points are reproduced from [1].

##### Issue 3-2-1: In-channel adjacent subband leakage ratio: General Agreement: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. ##### Issue 3-2-2: In-channel adjacent subband leakage ratio: Configuration Way Forward: - Further study the configuration of the in-channel adjacent subband leakage ratio requirement, by taking the following table as the baseline: BS DL subband bandwidth BWSBFD,DL (MHz); BS UL subband bandwidth BWSBFD,UL (UL); Filter on UL subband and corresponding filter bandwidth; In-Channel Adjacent Subband Leakage Ratio limit. BWSBFD,DL as configured by gNB; BWSBFD,UL as configured by gNB; Square (BWSBFD,UL); TBD. ##### Issue 3-2-4: In-channel adjacent subband blocking requirement Way Forward: - RAN4 continue to study In-channel adjacent subband blocking requirement, by following the agreement achieved in RAN4#113.

In this contribution, we would like to share our views regarding this topic.

Discussion

Potential adjacent subband leakage

Since it was agreed to define in-channel adjacent subband leakage and the discussion direction is to check whether absolute ACLR can be considered as baseline, we would like to provide some analysis accordingly. In the current spec, there are ACLR and CACLR defined for different cases, and for each of them the relative limit and absolute limit are defined separately while the verification is only required with whichever less stringent.

Viewing the Rel-18 SI simulation assumptions, the 45 dB ACLR was selected for the example 5GHz operating frequency. If we consider the BS Tx power as to output 53dBm within 100MHz RF bandwidth, then it is equivalent to 33dBm/MHz, so roughly -12dBm/Hz could be the residual noise level in the adjacent channel. However, such level would be higher if we consider U6G bands due to the ACLR requirement becomes 38dB.

Observation 1: Assuming 45dB ACLR and 53dBm BS output power over 100MHz, roughly -12dBm/Hz level residual noise level in the adjacent channel can be expected. However, it will get higher for U6G since the ACLR requirement is relaxed to 38dB.

Taking the above findings into consideration, we have the following proposal on this requirement.

Proposal 1: Suggest as follows in order to specify in-channel adjacent subband ACLR requirements covering both DU and DUD SBFD configurations.

- The ACLR definition can be reused for the DU case, while the CACLR definition can be reused for the DUD case.

- The existing absolute limit values cannot be reused w/o any relaxation, otherwise RAN4 can consider to have both relative and absolute limit and adopt whichever less stringent as the applied requirements.

Potential adjacent subband blocking

Regarding in-channel adjacent subband blocking, it has been agreed that new requirement is needed. But the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes should also be considered. To our understanding, such DL sub-band interference might cause some nonlinearity to LNA or other RF components but that interference is anyway coming from the same operator so some methods e.g. beam nulling/coordination can be applied in order to mitigate the interference.

Observation 2: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

Following simulation based on our co-ex simulation platform is conducted to provide some inputs for further discussion, note that CLI handling is NOT included.

Figure 1. CDF of in-channel adjacent subband blocking w/o CLI handling

According to the result, it is -28.03 dBm @99%-tile. Further discussion on how to combine the CLI handling with this interference level should be further discussed.

Observation 3: How to accommodate the CLI handling capability with the in-channel adjacent subband interference should be further discussed.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observation and proposals:

Observation 1: Assuming 45dB ACLR and 53dBm BS output power over 100MHz, roughly -12dBm/Hz level residual noise level in the adjacent channel can be expected. However, it will get higher for U6G since the ACLR requirement is relaxed to 38dB.

Observation 2: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

Observation 3: How to accommodate the CLI handling capability with the in-channel adjacent subband interference should be further discussed.

Proposal 1: Suggest as follows in order to specify in-channel adjacent subband ACLR requirements covering both DU and DUD SBFD configurations.

- The ACLR definition can be reused for the DU case, while the CACLR definition can be reused for the DUD case.

- The existing absolute limit values cannot be reused w/o any relaxation, otherwise RAN4 can consider to have both relative and absolute limit and adopt whichever less stringent as the applied requirements.

- References

- R4-2502310, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #114, Athens, Greece, February 17-21, 2025.