3GPP TSG-RAN4 Meeting #114bis	R4-2504099

# Wuhan, China, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft objective BigCR for HPUE_NR_CADC_SUL_R19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_ NR_CADC_SUL_R19-Core |  |  |  |  |  | Date: |  |  | 2025-04-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce band combination specific requirements for PC2 and PC1.5 inter band CA with y(y=2,3,4) bands downlink and x bands uplink (x =1,2) for Clause 6 and Clause 7 on TS38.101-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requested high power CA combinations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Above PC2 CA configurations are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4, 7.3A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF 1st CHANGE >>>

7.3A.4	Reference sensitivity exceptions due to harmonic interference for CA

<<Unchanged Omitted>>

For a PC2 aggressor NR UL band for NR DL CA FR1, the maximum amount of REFSENS degradation is specified in Table 7.3A.4-2a.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n2 | n48 | 5 | 15 | 12 | 5 | 10.6 | 13.1 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n77 | 5 | 15 | 12 | 10 | 26.9 | 30.6 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 16.7 | 20.2 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 | 10 | 27.1 | 32.3 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 | 100 | 16.6 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 | 10 | 15.9 | 20 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 | 100 | 6.5 | 9.5 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 | 10 | 14.9 | 19 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 | 100 | 6.2 | 8.4 | NOTE 2 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 10 | 13.1 | 16.5 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 4.4 | 5.7 | NOTE 5 | UL5/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 100 | 16.8 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 | 5 | 30.1 | 33.1 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 | 10012 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 100 | 16.7 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.8 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n77 | 5 | 15 | 12 | 100 | 16.7 | 19.5 | NOTE 2 | UL2/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 | 5 | 9.4 | 12.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 10 | 13.6 | 17.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 100 | 5.1 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 8 | 5 | 12.5 | 15.1 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 8 | 25 | 5.9 | 7.4 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n77 | 5 | 15 | 5 | 10 | 13.2 | 17.4 | NOTE 5 | UL5/DL1 direct-hit |
| n85 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 13.2 | 17.4 | NOTE 5 | UL5/DL1 direct-hit |
| n85 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 1.3 | 1.7 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWCℎannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. NOTE 9: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 10: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 11: Void. NOTE 12: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |  |  |  |  |

<<Unchanged Omitted>>

7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

<<Unchanged Omitted>>

Table 7.3A.5-2b: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | Duplex mode |  |  |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A |  | FDD |  | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 38.7 |  | FDD |  | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A |  | TDD |  | N/A |
| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A |  | FDD |  | N/A |
|  | n7 | N/A | 5 | N/A | 2627.5 | 25.2 |  | FDD |  | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A |  | TDD |  | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 29.5 | FDD |  | IMD4 |  |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD |  | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD |  | N/A |  |
| CA_n1-n26-n78 | n1 | N/A | 5 | N/A | 2122 | 31.9 |  | FDD |  | IMD3 |
|  | n26 | 829 | 5 | 25 | 874 | N/A |  | FDD |  | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A |  | TDD |  | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A |  | FDD |  | N/A |
|  | n26 | N/A | 5 | N/A | 885 | 23.9 |  | FDD |  | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A |  | TDD |  | N/A |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 29.4 |  | FDD |  | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A |  | FDD |  | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A |  | TDD |  | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A |  | FDD |  | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 21.3 |  | FDD |  | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A |  | TDD |  | N/A |
| CA_n3-n7-n78 | n3 | N/A | 5 | N/A | 1820 | 31.4 | FDD |  | IMD3 |  |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD |  | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD |  | N/A |  |
|  | n3 | N/A | 5 | N/A | 1820 | 29.4 | FDD |  | IMD4 |  |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD |  | N/A |  |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD |  | N/A |  |
| CA_n3-n26-n78 | n3 | N/A | 5 | N/A | 1862 | 29.4 |  | FDD |  | IMD3 |
|  | n26 | 839 | 5 | 25 | 884 | N/A |  | FDD |  | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A |  | TDD |  | N/A |
| CA_n3-n28-n78 | n3 | N/A | 5 | N/A | 1850 | 31 |  | FDD |  | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A |  | FDD |  | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A |  | TDD |  | N/A |
| CA_n7-n26-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A |  | FDD |  | N/A |
|  | n26 | N/A | 5 | N/A | 879 | 41 |  | FDD |  | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A |  | TDD |  | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A |  | FDD |  | N/A |
|  | n26 | N/A | 5 | N/A | 875 | 24.3 |  | FDD |  | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A |  | TDD |  | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 40.9 | FDD |  | IMD2 |  |
|  | n26 | 844 | 5 | 25 | 889 | N/A | FDD |  | N/A |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD |  | N/A |  |
| CA_n7-n28-n78 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD |  | N/A |  |
|  | n28 | N/A | 5 | N/A | 782.5 | 39.6 | FDD |  | IMD21 |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD |  | N/A |  |
|  | n7 | N/A | 5 | N/A | 2650 | 41.3 |  | FDD |  | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A |  | FDD |  | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A |  | TDD |  | N/A |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A |  | FDD |  | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A |  | TDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 33.1 |  | TDD |  | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A |  | FDD |  | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A |  | TDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3775 | 31.2 |  | TDD |  | IMD5 |
| CA_n41-n66-n77 | n41 | 2600 | 5 | 25 | 2600 | N/A | TDD |  | N/A |  |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD |  | N/A |  |
|  | n77 | N/A | 10 | N/A | 3470 | 34.4 | TDD |  | IMD31,2 |  |
|  | n41 | N/A | 5 | N/A | 2670 | 23.4 | TDD |  | IMD55 |  |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD |  | N/A |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD |  | N/A |  |
|  | n41 | 2640 | 5 | 25 | 2640 | N/A | TDD |  | N/A |  |
|  | n66 | N/A | 5 | N/A | 2160 | 32 | FDD |  | IMD4 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD |  | N/A |  |
| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A |  | TDD |  | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3308 | 39.8 |  | TDD |  | IMD21,5 |
|  | n41 | 2564 | 5 | 25 | 2564 | N/A |  | TDD |  | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3950 | 29.9 |  | TDD |  | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A |  | TDD |  | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3774 | 31.3 |  | TDD |  | IMD41 |
|  | n41 | N/A | 5 | N/A | 2615 | 39.4 |  | TDD |  | IMD25 |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A |  | TDD |  | N/A |
|  | n41 | N/A | 5 | N/A | 2564 | 29.1 |  | TDD |  | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A |  | TDD |  | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped NOTE 6: This band combination is specified with support of two band inter-band UL CA with UL MIMO or Tx diversity capabilities, and the transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) for the UL band with single Tx antenna connector as defined in clause 6.2A.4, and set at min (+27.8 dBm, PCMAX_L,f,c) for the UL band with two Tx antenna connectors as defined in clause 6.2H.3.4 or 6.2L.3.4. |  |  |  |  |  |  |  |  |  |  |

<<< END OF 1st CHANGE >>>