3GPP TSG-RAN WG4 Meeting #114bis	R4-2504114

Wuhan, China, 7th – 11th, April 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	7.17.2.2

Document for:	Discussion

# Introduction

As per WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, In last meeting 3GPP TSG RAN WG4 Meeting #114, the outcome of the debate, encompassing consensuses and unresolved issues, were documented in WF [2]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Topic #2-2: L3 measurement delay reduction by optimizing CSSF for CSSF outside gap in CA/DC

Issue 1-3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF:

Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

As reference, which already have been presented in the last meeting, there are some typical multi-carrier configurations in real deployment as follows:

- For FR1 only, 5 bands/6 CCs combinations include:

  - CA_n25A-n41C-n66A-n71A-n77A

  - CA_n25A-n41(2A)-n66A-n71A-n77A

  - CA_n25(2A)-n41A-n66A-n71A-n77A

- For FR1+FR2, 3 bands combination includes:

  - DC_n25A-n41A-n258(G-J)

Although the 3rd searcher resource sharing is same as the second searcher sharing and indeed shortens the ‘total’ measurement delay of all SCells, motivated by operators’ network configurations and scheduling plans, we expect at least one specific serving cell can be measured and reported more promptly without waiting for the completion of the ‘total’ measurement.

In this context, NW is able to indicate the intended serving cell. To simplify the CSSF management from UE implementation perspective, we can compromise that only one intended serving cell is able to be served by the third searcher as per indication by NW.

Observation 1: CSSF solution shall aim to practical multiple bands scenario in real field.

As reference, some typical multi-carrier configurations in real deployment are presented as follow:

- For FR1 only, 5 bands/6 CCs combinations include:

  - CA_n25A-n41C-n66A-n71A-n77A

  - CA_n25A-n41(2A)-n66A-n71A-n77A

  - CA_n25(2A)-n41A-n66A-n71A-n77A

- For FR1+FR2, 3 bands combination includes:

  - DC_n25A-n41A-n258(G-J)

Proposal 1: NW is able to indicate the single intended serving cell (FR1) to be exclusively served by the third searcher.

- In this case, the first and the second searcher manage other serving cells in the conventional manner.

- If no NW indication, the third searcher resource sharing is same as the second searcher sharing.

The above may be only applied in FR1 only configuration.

A leftover of CSSF solutions is whether Solution 1 and Solution 3 should be simultaneously supported by a UE applying CSSF enhancement. To enhance simplification, we propose that a UE should indicate support for both Solution 1 and Solution 3 through a single capability declaration.

Proposal 2: A UE may indicate support for both Solution 1 and Solution 3 through a single capability declaration.

# Conclusion

Observation 1: CSSF solution shall aim to practical multiple bands scenario in real field.

As reference, some typical multi-carrier configurations in real deployment are presented as follow:

- For FR1 only, 5 bands/6 CCs combinations include:

  - CA_n25A-n41C-n66A-n71A-n77A

  - CA_n25A-n41(2A)-n66A-n71A-n77A

  - CA_n25(2A)-n41A-n66A-n71A-n77A

- For FR1+FR2, 3 bands combination includes:

  - DC_n25A-n41A-n258(G-J)

Proposal 1: NW is able to indicate the single intended serving cell (FR1) to be exclusively served by the third searcher.

- In this case, the first and the second searcher manage other serving cells in the conventional manner.

- If no NW indication, the third searcher resource sharing is same as the second searcher sharing.

The above may be only applied in FR1 only configuration.

Proposal 2: A UE may indicate support for both Solution 1 and Solution 3 through a single capability declaration.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2502592, WF for NR_RRM_Ph5, Apple