3GPP TSG-RAN WG4 Meeting #114bis	R4-2504163

Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda item:	7.1.1.3.4

Source:	Intel Corporation

Title:	Views on SRS insertion loss compensation and reporting enhancements

Document for:	Discussion

# Introduction

The Rel-19 WID on UE RF enhancements for NR FR1/FR2 and EN-DC Phase 4 was approved in RAN#103 with the following objectives related to 6Rx for handheld and FWA UEs [1]:

### 4.1 Objective of Core part WI The objectives of core part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: … 6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. …

The topics of SRS insertion loss imbalance was discussed in previous meetings with agreements provided in [2-7].  In this contribution we provide further views on the topic.

# Discussion

## Legacy UE behavior for SRS insertion loss compensation

Based on Rel-18 discussions in RAN1/4 there was no clear understanding among companies on the anticipated UE behavior in terms of SRS IL compensation during SRS transmissions with the following options mentioned:

- Option 1: UE does not make Tx power imbalance compensation.

- Option 2: UE performs Tx power imbalance compensation up to its maximum capabilities in non-power limited conditions (i.e., applies power compensation in case there is sufficient power to do so). For example, in case of 20dBm configured Tx power and 8dB SRS IL, the UE should be capable to compensate 3dB loss in case of 23dBm maximum output power.

- Option 3: Keep up to UE implementation and UE can choose whether and when to apply compensation.

The legacy UE behavior can be understood based on current specifications of the SRS power control mechanism in TS 38.213 and based on configured maximum output Tx power in TS 38.101-1.

The SRS transmission power in TS 38.213 is defined in a way that unless the SRS transmission power exceeds the configured maximum output power PCMAX,f,c limit (defined in TS 38.101-1), the actual SRS transmission power depends on L1 power control parameters only such as p0 for active UL BW, SRS bandwidth, pathloss and others. It can be observed that the SRS insertion loss is not taken into consideration, and it may be understood that UE is required to compensate any SRS insertion loss as long as the total power is below PCMAX,f,c (as SRS IL is not present in equations).

The UE configured maximum output power parameter PCMAX,f,c is defined in TS 38.101-1. The effect of SRS insertion loss is accounted in the PCMAX via the parameter ∆TRxSRS, which reflects the worst-case implementation in terms of SRS insertion loss (i.e., upper bound). It can be observed that PCMAX is not a pre-defined fixed value and represents a certain range of values, which gives flexibility to possible UE implementations.

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

It should be also emphasized that for PCMAX_L,f,c is the SRS IL term applies to the SRS ports affected by the IL only, while the transmission from main branches is unaffected.

In our understanding the legacy UE behavior may depend on the specific SRS transmission power:

- Case 1 (non near max Tx power): In this scenario the SRS transmission power (PSRS) is below PCMAX. Based on TS 38.213 the UE is required to compensate any insertion loss and no further changes in the specification are needed.

- Case 2 (near max Tx power): In this scenario the actual required SRS transmission power based on equations is upper bounded by PCMAX. In this case the UE behavior is not fully defined and UE may or may not necessarily perform SRS IL compensation depending on UE-specific SRS IL for this UE and chosen PCMAX,f,c value.

Observation #1:	The legacy UE behavior is based on current specifications of the SRS power control mechanism in TS 38.213 and based on configured maximum output Tx power in TS 38.101-1.

Observation #2:	The existing specifications imply that UE applies SRS IL compensation, when the SRS transmission power (PSRS) is below PCMAX , and UE behavior for the near max Tx power region is not fully defined w.r.t. whether UE shall apply SRS IL compensation.

During prior discussions there were comments that legacy UE may not be required to perform SRS IL compensation. In the Figure 1 below we provide an illustration of the SRS transmission power levels and effective SRS power imbalance between ports as a function of PSRS,PC under different assumptions of PCMAX selection (equal to either PCMAX,H or PCMAX,L) based on the equations above for the following cases:

- Case A: ∆TRxSRS = 7dB, ∆TRxSRS,UE  =  7dB (UE-specific value is same as the spec value)

- Case B: ∆TRxSRS = 7dB, ∆TRxSRS,UE  =  3dB (UE-specific value is better than the spec value)

The following curves are illustrated in the figures:

- “P0” – SRS transmission power for the main SRS AP without SRS IL

- “P1” – SRS transmission power for the diversity SRS AP with SRS IL

- “P1,EFF” – SRS transmission power for the diversity SRS AP before applying SRS IL

- “P0-P1” – SRS transmission power imbalance between SRS AP with and without SRS IL (P0-P1)

The results are provided for 1) UE applying the SRS IL compensation for all regions, and for 2) UE, which does not make SRS IL compensation (i.e. hypothetical wrong implementation).

| Case A: UE applies SRS IL compensation |  |
| --- | --- |
| Case A: UE does not apply compensation |  |
| Case B: UE applies SRS IL compensation |  |
| Case B: UE does not apply compensation |  |

Based on the analysis above we make the following observations:

- For UEs applying the SRS IL compensation

  - When the effective SRS IL is same as the min requirement (∆TRxSRS = ∆TRxSRS,UE) the effective SRS AP power imbalance does not depend on the selection of PCMAX level

  - When the effective SRS IL is better than the min requirement (∆TRxSRS,UE <∆ TRxSRS,UE) and the effective SRS AP power imbalance may depend on the selection of PCMAX level. UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities in case PCMAX=PCMAX,L

- For UEs not applying the SRS IL compensation

  - UE underutilizes PA capabilities and does not perform SRS IL compensation up to its maximum capabilities for all cases

  - The actual power gap between different branches may further increase to be > ∆TRxSRS,UE  if UE chooses  PCMAX = PCMAX,L and does not apply SRS IL compensation

Given that there is no common understanding in RAN4 on the existing UE behavior, we recommend to explicitly clarify the legacy (or at least intended) UE behavior so that a common ground is clear:

Proposal #1:	Clarify the legacy UE behavior for SRS IL compensation as follows:

- For non near max Tx power scenario with PSRS < PCMAX the UE is supposed to always compensate any SRS insertion loss in SRS transmissions based on TS 38.213.

- For near max Tx power scenario the UE behavior in terms of SRS IL compensation for diversity branches is not fully specified and UE may or may not necessarily be required to perform SRS IL compensation up to its maximum power capabilities.

## Rel-19 enhancements for SRS insertion loss compensation

In RAN4 #114 the following further agreements on SRS IL compensation solutions were reached:

Issue 3-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1. Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: RAN4 to take the candidate solution framework below as baseline for further discussion Candidate solution framework: - Rel-19 UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on RAN4 specification impacts. - No RAN1 specification impact. - FFS if UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others

Self-compensation of SRS IL

The UE self-compensation is expected to be the baseline solution to handle the SRS IL issue, and the general approach should be to make sure that UE transmits SRS at the maximum power level whenever possible to make sure receiver side SNR is maximized. We recommend clarifying the principles for SRS IL compensation as follows:

- UE shall perform SRS IL compensation for the affected Tx branches for any SRS transmission power in TS 38.213

- The amount of SRS IL compensation is limited by the configured maximum Tx power excluding any SRS IL effects

The existing specification implies that UE performs SRS IL self-compensation in many cases and no changes are expected. However, during previous discussions it was mentioned that some of existing UEs may not necessarily follow the legacy UE behavior even for non near max Tx power scenario with PSRS < PCMAX. So, we recommended to introduce the corresponding new Rel-19 (optional) requirements/testing framework to ensure proper UE implementations for legacy UEs. The following framework is considered:

- Goal: Ensure that UE following existing RAN1/4 power control equations for SRS transmissions and provides equal power transmission across different transmissions and across Tx antennas.

- Ideally the requirements shall be introduced from early releases

- The testing shall cover a wide range of output transmit power configurations. At least scenarios with PSRS,PC ≤ PCMAX,L   shall be covered.

- The requirements can be defined in Rel-19 timeframe as optional new requirements

Proposal #2:	Specify solution for UE to perform SRS IL pre-compensation based on the following principles

- UE shall perform SRS IL compensation for the affected Tx branches for any SRS transmission power in TS 38.213

- The amount of SRS IL compensation is limited by the configured maximum Tx power excluding any SRS IL effects

Proposal #3:	Introduce SRS output power testing framework to guarantee proper UE behavior based on existing specifications at least for PSRS < PCMAX.

Improved self-compensation of SRS IL

As explained in section 2.1, when the effective SRS IL (∆TRxSRS,UE) is better than the minimum requirement (∆TRxSRS,UE < ∆TRxSRS), the UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities if PCMAX < PCMAX,H. To ensure that the UE performs more optimal SRS IL pre-compensation, we recommend to adjust the existing SRS PCMAX equations on PCMAX_L,f,c so that the UE applies compensation considering the UE-specific (or SRS port specific) SRS IL value ∆TRxSRS,UE. In the latter case, UE will be expected to perform self-compensation optimized for the UE-specific insertion loss .

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Proposal #4:	Adjust the PCMAX_L,f,c equations to include UE-specific SRS-IL component:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Assistance to the network on SRS IL compensation

Regarding signalling aspects, the following general approaches can be considered based on proposals in previous RAN4 meetings:

- Alt 1: The UE reports information on SRS insertion loss (or power imbalance) used for SRS transmissions to the network.

- Alt 2: The UE reports the configured maximum output power per SRS resource and the respective power headroom transmissions to the network.

Both mechanisms allow the network to obtain more precise information about the actual SRS transmission power imbalance between different transmit branches. The Alt 1 approach offers a reasonably good flexibility and can accommodate both static and dynamic assistance use cases with impact on RAN2/4 specs. Whereas the Alt 2 approach would imply dynamic power headroom reporting and potential updates in RAN1/2/4 specs. Per discussion in RAN4 #114 it is expected that RAN1 impacts shall be avoided and RAN2 impacts are minimized.

For the Alt 1 approach we recommend that UE provides information on the UE SRS IL (∆TRxSRS,UE), which may include either a static value (maximum SRS IL for the given UE) or more dynamic information on the actual SRS IL used by the UE. Static signalling results in a simplified UE implementation, but does not provide network with sufficient information on the actual SRS power imbalance. Dynamic reporting of SRS IL imposes extra overhead and UE complexity, but would allow NW to perform post-compensation of actual SRS power imbalance. Both options have their pros and cons but do not contradict each other. Therefore, it is suggested to consider both static and dynamic indications of SRS IL information as complementary solutions. The information can be provided with per SRS resource granularity.

Proposal #5:	Introduce UE assistance signalling on the information on the UE SRS IL (∆TRxSRS,UE).

    - Consider at least static reporting of SRS IL information

    - SRS IL information is provided with per SRS resource granularity

Other aspects

The considered framework is generic, and we do not see a motivation to limit it to 8Rx cases only. It is recommended to make it applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Furthermore, given the potential impact on UE implementation it is recommended to introduce new Rel-19 features as optional.

Proposal #6:	Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs.

# Conclusion

In this paper we provide views on the insertion loss imbalance across SRS ports. In summary, the following proposals are made:

Proposal #1:	Clarify the legacy UE behavior for SRS IL compensation as follows:

- For non near max Tx power scenario with PSRS < PCMAX the UE is supposed to always compensate any SRS insertion loss in SRS transmissions based on TS 38.213.

- For near max Tx power scenario the UE behavior in terms of SRS IL compensation for diversity branches is not fully specified and UE may or may not necessarily be required to perform SRS IL compensation up to its maximum power capabilities.

Proposal #2:	Specify solution for UE to perform SRS IL pre-compensation based on the following principles

- UE shall perform SRS IL compensation for the affected Tx branches for any SRS transmission power in TS 38.213

- The amount of SRS IL compensation is limited by the configured maximum Tx power excluding any SRS IL effects

Proposal #3:	Introduce SRS output power testing framework to guarantee proper UE behavior based on existing specifications at least for PSRS < PCMAX.

Proposal #4:	Adjust the PCMAX_L,f,c equations to include UE-specific SRS-IL component:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Proposal #5:	Introduce UE assistance signalling on the information on the UE SRS IL (∆TRxSRS,UE).

    - Consider at least static reporting of SRS IL information

    - SRS IL information is provided with per SRS resource granularity

Proposal #6:	Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs.

# References

- RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#103, March 2024

- R4-2406585 “WF on requirements for 6Rx”, AT&T, RAN4#110bis, April 2024

- R4-2410751 “WF on requirements for 6Rx”, AT&T, RAN4#111, May 2024

- R4-2414285 “WF on requirements for 6Rx”, AT&T, RAN4#112, August 2024

- R4-2417209 “WF on requirements for 6Rx”, AT&T, RAN4#112bis, October 2024

- R4-2420376 “WF on requirements for 6Rx”, AT&T, RAN4#113, November 2024

- R4-2503014 “WF on requirements for 6Rx”, AT&T, RAN4#114, February 2025