3GPP TSG-RAN WG4 Meeting #114bis		R4-2504172
Wuhan, China, 7– 11 April, 2025

Title:	R19 on NTN SAR

Source:	OPPO

Agenda item:	7.10.2.3.1

Document for:	Approval

- Introduction

This paper discusses the NTN SAR related issues per WF [1] and also the duty cycle-based SAR solution enhancements.

- Discussion

  - Problems of duty cycle approach in TN system

Duty cycle approach has been widely used in the TN system to solve the SAR issue in TDD bands, TDD+TDD band combinations and TDD+FDD band combinations at least from specification perspective. However, it was failed to be applied in FDD HPUE in TN system due to difficult in restricting UL transmit time in FDD system.

Observation 1:   TN system has failed to define duty cycle based SAR solution for FDD system.

Meanwhile, it was also recognized that this kind of duty cycle approaches have not been implemented in the real UE implementation and also base station where PMPR was widely adopted instead of duty cycle approach. One of the reasons is that after UE reports the duty cycle like 50%, this UE will always be restricted by network regardless of UE status like the Tx power or whether UE is used near human body or not.

For example, if UE is put on the table and far away from human body, in this case the SAR should not be considered and UE can transmit with 100% UL duty cycle, but with the current 3GPP duty cycle-based SAR solution, this UE will always be restricted to 50% UL duty cycle. This makes UE be degraded in the network.

Observation 2:   Current duty cycle based SAR solution in TN system has drawback of always restrict UE to the reported duty cycle and ignore whether SAR should be considered or not in current UE status. This makes UE be degraded in the network. And this is why no UE is willing to implement it.

Therefore, if duty cycle based SAR solution is further considered in NTN FDD system, these issues should be carefully considered.

  - SAR in NTN

Currently, many smartphones on the market already support satellite system though it is not 3GPP NTN. And after checking the SAR status, it seems there are some difference from TN system.

For example, in some region, SAR is required according to the UE using status:

- UE is not required to meet SAR, when it is used in the hand held status;

- UE is required to meet SAR, when it is used near the head;

This is different from current TN system where SAR is required to be met in both hand held status and near the head status.

Observation 3:   In some region, SAR is not required to be met when NTN UE is in the handheld status, and is only required when NTN UE is near the head.

This is different from current TN system where SAR is required to be met in both handheld status and near the head status.

  - SAR solution in NTN

The concept of duty cycle-based SAR solution has been well understood in TN system at least for TDD. Now the NTN mostly are FDD system, and there is no TN FDD solution can be followed.

If simply mimic TDD approaches and ask UE to report a duty cycle, and ignore the already known issues mentioned above, then this will be another paper work and will never be used in reality. UE will have to keep on using PMPR to solve the SAR issue.

Considering UE Tx power is so valuable in NTN system, we must make sure the designed duty cycle-based approach in NTN will be used as much as possible.

Observation 4:   If simply ask UE to report a duty cycle, and ignore the already know issues (e.g. UE status needs to be considered in calculating the UL duty cycle), then this will be another paper work and will never be used in reality. UE will have to keep on using PMPR to solve the SAR issue.

To optimise the maxUplinkdutycycle based SAR solution and solve the issues pointed out in section 2.1.1 and consider the SAR differences between TN and NTN in section 2.1.2, one area can be enhanced is that in addition to maxUplinkdutycycle capability, UE can additionally indicate whether the following Tx time duration should be counted into the UL duty cycle statistics.

Proposal 1:         UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2.

- Conclusion

In this paper we have analyzed the NTN SAR regulations and found there are some differences from TN SAR. Then analyzed the reason why current TN SAR solutions haven’t been implemented by any UE, and regarding these problems some enhancements are proposed.

Problems of duty cycle approach in TN system:

Observation 1:   TN system has failed to define duty cycle based SAR solution for FDD system.

Observation 2:   Current duty cycle based SAR solution in TN system has drawback of always restrict UE to the reported duty cycle and ignore whether SAR should be considered or not in current UE status. This makes UE be degraded in the network. And this is why no UE is willing to implement it.

SAR regulation in NTN:

Observation 3:   In some region, SAR is not required to be met when NTN UE is in the handheld status, and is only required when NTN UE is near the head.

This is different from current TN system where SAR is required to be met in both handheld status and near the head status.

SAR solution in NTN:

Observation 4:   If simply ask UE to report a duty cycle, and ignore the already know issues (e.g. UE status needs to be considered in calculating the UL duty cycle), then this will be another paper work and will never be used in reality. UE will have to keep on using PMPR to solve the SAR issue.

Proposal 1:         UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2.

- Reference

[1] R4-2503006 WF on IoT NTN HPUE, vivo, #114 2025-02