3GPP TSG RAN WG4 #114bis	R4-2504191

Wuhan, China, April 7 – 11, 2025

Agenda item:		7.13.3

Source:	Nokia

Title:	On OTA test enhancements

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#114 regarding OTA test enhancements. This contribution further elaborates Nokia’s views on the items related to OTA test enhancements.

# Discussion

## Horn antenna aspects

The following was agreed in the WF [1] of RAN4#114:

Way forward: Companies to investigate applicable frequency range for horn antenna studies (i.e., where the reference CLTA may not be available).

We have been studying extensively on evaluating the performance of wideband horn antennas as CLTA’s and our previous measurement results can be found in our contributions [2], [3] and [4]. From those measurements, it could be seen that SH400 performs well for frequencies up to ~2GHz. As it has been difficult to compare the performance of wideband horn antennas and CLTA’s with co-location measurements on higher frequencies, we further studied the measured response of wideband horn antenna and CLTA with known power level using signal generator and signal analyser and antenna panels of 3.5GHz and 7GHz.

SH400 and LB-2080 horn antennas were used in measuring the 3.5GHz antenna panel response. Both +/- 45° polarities were measured and sum results are presented. The horn antennas have a considerable size difference that is captured in the setup figures. This most likely affects the measurement results.

SH400 and LB-2080 have a significant size difference.

Figure 1. SH400 measurement setup.

Figure 2. LB-2080 measurement setup.

Figure 3. CLTA measurement setup.

Four closest sub-arrays were measured from the antenna panel with SH400, LB-2080 and CLTA for 3.5GHz and results are shown in Table 2.

Table 1. Measurement results for 3.5GHz.

|  | CLTA | SH400 | LB-2080 |
| --- | --- | --- | --- |
| SA1 | -37.0 | -48.9 | -47.7 |
| SA2 | -35.0 | -45.4 | -38.0 |
| SA3 | -33.5 | -45.5 | -46.7 |
| SA4 | -35.7 | -49.5 | -62.0 |

It can be seen from the results that SH400 is more consistent with comparison with CLTA, while keeping in mind that there is an offset in the result. This is most likely due to the beam width of the antenna and structure of the antenna. Horn antennas are structured to metallic sides on two sides of the antenna as they are intended to be used directly pointing to the DUT. Also wideband horn antennas receive different frequency bands in different parts of the antenna. The higher frequencies are received in the back part of the antenna as shown for SH400 in Figure 4, and it might be required to move the horn antenna forward according to measured frequency band to get similar results. It must be noted that SH400 is almost a half a meter long antenna so this distance most likely has an effect.

Wideband horn antenna receives different frequency bands in different part of the antenna.

Wideband horn antenna results might be more comprehensive if antenna was moved forward to reflect the received surface to the radome of the DUT.

During the measurements, the front part of the horn antenna was aligned with the radome of the DUT, which might not give correct results. LB-2080 differences might be explained by the small size of the antenna compared to the size of the DUT. SH400 is not that far from the size of the DUT.

Figure 4. SH400 structure view.

For the 7GHz measurements, LB-2080 was used again in similar setup as for 3.5GHz.

Results are shown in Table 2.

Table 2. Measurement results for 7GHz.

|  | CLTA | LB-2080 |
| --- | --- | --- |
| SA1 | 57.4 | 50.2 |
| SA2 | 51.3 | 54.8 |
| SA3 | 54.1 | 50.8 |
| SA4 | 58.0 | 58.4 |

For 7GHz, the LB-2080 performs much better than for 3.5GHz. Still there are some differences in the results, which can be explained by the horn antenna structure and size.

## BS-to-BS isolation analysis

Regarding the BS-to-BS isolation analysis, the following was agreed in RAN4#114:

Way forward: BS-to-BS isolation data collection to prioritize the following scenarios: - Horizontal domain prioritized (vertical BS stacking analyses not precluded); - Same-band analysis (inter-band analysis not precluded), - Array-to-array case (array to sub-array not precluded); - Same BS type for aggressor and victim (mixed cases of various BS types not precluded). Guidance for collection of future results: Companies to clarify on the statistical distribution of the results provided (i.e., worst case, CDF, etc.) For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements. BS-to-BS isolation data (simulations, or measurements) to be collected on top of the data already collected in the following table formats: - No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB); ZTE (M, R4-2501875): -54; ZTE (R4-2501875): -52; ZTE (M, R4-2501875): -56. Isolation in horizontal domain (dB); ZTE (M, R4-2501875): -31; ZTE (M, R4-2501875): -34 Huawei (M, R4-2502135): -46; ZTE (M, R4-2501875): -46. - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling. - Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 6.7 GHz --> 7 GHz. Isolation in vertical domain (dB). Isolation in horizontal domain (dB); S: Ericsson (R4-2418398): A-to-SA: -55±20 dB; S: Ericsson (R4-2502160): A-to-A: -77±25; A-to-SA: -70±20. For the deprioritized scenarios, reuse table templates as above. - Timeline for BS-to-BS isolation data collection: conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting (RAN4#116). - Data collection for other carrier frequencies is not precluded. - Any future conclusion on isolation numbers is subject to availability of sufficient amount of data from multiple companies and can be decided case-by-case.

CLTA was used to measure coupling loss between an antenna array and CLTA for 3.5GHz and 7GHz. CLTA arrangement was used with the same plane orientation, center alignment in vertical domain and 10cm distance between CLTA and DUT. Both +/- 45° polarizations were measured. Beamforming was not applied for this measurement. There were four different sub-arrays measured as shown in Figure 4.

Figure 5. Setup for the coupling measurements

Four sub-arrays (SA1…SA4) of the DUT were measured separately with keeping the CLTA arrangement constant. There are differences between sub-arrays as depicted in Table 3.

Table 3. Coupling results for 3.5GHz and 7GHz.

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in horizontal domain (dB) |  | 37.0 (SA1) 35 (SA2) 33.5 (SA3) 35.7 (SA4) |  | 57.4 (SA1) 51.3 (SA2) 54.1 (SA3) 58.0 (SA4) |

The results show that the coupling loss for 3.5GHz frequency without considering beam steering was ranging from 33.5dB to 37.0dB. For the 7GHz frequency results were ranging from 51.3dB to 58dB, respectively. The worst case value for 3.5GHz would be 33.5dB and the average would be 35.3dB. Similar results for 7GHz are 51.3dB and 55.2dB, respectively.

Coupling results for 3.5GHz without beam steering are in the neighborhood of 35dB.

Coupling results for 7GHz without beam steering are in the neighborhood of 55dB.

### Assumptions for the beam steering scenario analysis

Regarding the beam steering analysis, the following was agreed in RAN4#114:

Way forward: To improve simulation results alignment, the following assumptions are taken for the beam steering consideration: Data collection to prioritize the following scenarios: - Beam steering range +/- [60] degrees - Reuse CLTA arrangements, e.g., same plane orientation; 10 cm offset enclosure to enclosure, etc.

As agreed in RAN4#114, it is beneficial to have alignment with the priority of scenarios. This will make it easier to compare the results from companies. At this stage it makes sense to reuse the CLTA arrangements as described in the way forward. As beam steering has already been described as technology of paramount importance for AAS, it is important that it is studied with unified methodology between companies. Because typical sectorized setup covers 120° for one sector, we see the beam steering range for the scenario analysis should be +/- 60°.

When collecting data for beam steering scenarions, it makes sense to reuse CLTA arrangements.

Consider beam steering range for the scenario analysis of +/- 60°.

### Impacted specifications

Regarding the impacted specifications, the following was agreed in RAN4#114:

Way forward: Baseline set of specifications for implementation of the BS-BS isolation conclusions: - TS 37.105 - TS 37.145-1/-2 - TS 38.104 - TS 38.141-1/-2 - TS 38.174 - TS 38.176-1/-2 - TS 38.106 - TS 38.115-1/-2

As mentioned in the WF, the specifications listed are most likely to be impacted. One thing that needs to be discussed here, is if the impact is only related to AAS? Will there be effect also to RRH specifications? This might lead to a situation where AAS specifications are looser than the RRH specifications for the same frequency band.

If specifications for AAS are loosened for frequencies >2.5GHz, there might be mismatch between the RRH requirements.

### TX IMD co-location

Regarding TX IMD colocation, the following was agreed in RAN4#114:

Way forward: Companies are encouraged to provide inputs on the handling of the intra-system TX IMD requirements.

As discussed before, we think it is still premature to remove TX IMD from Rel-19 specification. As the TX IMD is a regulatory requirement, a lot of caution should be considered when removing the requirement. Coupling loss studies should be completed before any discussion about removing requirements.

Coupling loss studies need to be completed before any discussion about removing requirements.

- One additional part not to forget in the TX IMD requirement is the intra-system requirement. As the intra-system requirement can be more stringent than general TX IMD requirement, this requirement should not be removed.

Intra-system TX IMD requirements should not be removed.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- SH400 and LB-2080 have a significant size difference.

- Wideband horn antenna receives different frequency bands in different part of the antenna.

- Wideband horn antenna results might be more comprehensive if antenna was moved forward to reflect the received surface to the radome of the DUT.

- Coupling results for 3.5GHz without beam steering are in the neighborhood of 35dB.

- Coupling results for 7GHz without beam steering are in the neighborhood of 55dB.

- When collecting data for beam steering scenarions, it makes sense to reuse CLTA arrangements.

- If specifications for AAS are loosened for frequencies >2.5GHz, there might be mismatch between the RRH requirements.

- Consider beam steering range for the scenario analysis of +/- 60°.

- Coupling loss studies need to be completed before any discussion about removing requirements.

- Intra-system TX IMD requirements should not be removed.

# References

- R4-2502279 “Way Forward for [114][303] NR_BS_RF_Part2_CLTA”, RAN4#114, Athens, Greece, 17th – 21st February, 2025.

- R4-2415674 ”On OTA test enhancements - CLTA”, RAN4#112bis, Hefei, China, 14th – 18th October, 2024.

- R4-2418018 ”On OTA test enhancements”, RAN4#113, Orlando, USA, 18th – 22nd November, 2024.

- R4-2501323 “On OTA test enhancements”, RAN4#114, Athens, Greece, 17th – 21st February, 2025.