3GPP TSG-RAN WG4 Meeting #114bis	R4-2504197

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Ericsson

Title:	UE RF requirements for Type 4a/4b capable FWA UE for EN-DC/NR-CA

Agenda item:	7.4.1.1

Document for:	Discussion

# 1	Introduction

In RAN4#114 the following WF was agreed [1]:

… # Topic #2: Rel-19 Type 4 ## Sub-topic 2-1 : UE RF requirements for Type 4 EN-DC/NR-CA ### <Issue 2-1-1: How to capture the power imbalance and REFSENS requirements for Big CR> Way forward: - The CR leader take companies’ inputs of R4#114 and previous meetings into account when drafting actually. …

In this contribution we discuss proposals related to the issues above.

# 2	Discussion

## 2.1 	Issue 2-1-1:  How to capture the power imbalance and REFSENS requirements for Big CR

The WF [1] states that the CR leader take companies’ inputs of R4#114 meeting into account when drafting, so in this tdoc we repeat our position, from #114 for reference.

In current specification CA and EN-DC we specify power imbalance and REFSENS as defined by the reference sensitivity level for two antenna port in Table 7.3.2-1b.

… Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Center of BWanother Relative to edge of BWwanted. 1; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz. Another wanted carrier; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted > BWanother. Another wanted carrier; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS NOTE 4 + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier; Power of wanted carrier + 25. NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void.. … Table 7.10B.3-1: Test parameters for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Frequency relationship (Center of BWanother Relative to edge of BWwanted). 1; Wanted carrier; REFSENS + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz). Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS + 1; BWwanted > BWanother. Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25. NOTE 1: For NR carrier, the transmitter shall be set to 24dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 24 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 3: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with overlapping DL bands NOTE 4: REFSENS is the reference sensitivity level or two antenna port in Table 7.3.2-1a or in Table 7.3.2-1b of TS 38.101-1, or in Table 7.3.1-1 of TS 36.101. NOTE 5: For TDD-TDD combination, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between E-UTRA and NR carriers for the EN-DC combinations in Table 7.10B.3-2. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 6: For Inter-band EN-DC configurations with multiple contiguous E-UTRA CCs in one band, REFSENS in this table equals to 5MHz REFSENS+10*log(aggregated BW(MHz)/5) of all the contiguous E-UTRA CCs of the wanted band. BWwanted and BWanother represent the aggregated BWs of all the CCs of the wanted and another band, respectively. The maximum power spectral density imbalance between the contiguous E-UTRA CCs in one band, is within 6 dB.”. …

When it comes to 4 RX in existing specification we have, for REFSENS:

…[TS 38.101-1] when the UE is verified with 4 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 with 4 Rx antenna ports tested. …[TS 38.101-1] For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied. Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R Operating band; ΔRIB,4R (dB). n5, n8, n13, n26, n28, n71, n85, n105; -2.71. n5, n8, n28, n71, n20, n26; -2.42. n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70; -2.7. n48, n77, n78, n79, n104; -2.2. NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE.. …

In current specification we use NOTE 4 in TS 38.101-1 Table 7.10A.2-1 and TS 38.101-3 Table 7.10B.3-1 to point to the corresponding reference sensitivity for two ports for type 2 UE.

… NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. … NOTE 4: REFSENS is the reference sensitivity level or two antenna port in Table 7.3.2-1a or in Table 7.3.2-1b of TS 38.101-1, or in Table 7.3.1-1 of TS 36.101. …

Observation 1: In current specification we use NOTE 4 in TS 38.101-1 Table 7.10A.2-1 and TS 38.101-3 Table 7.10B.3-1 to point to the corresponding reference sensitivity for two ports for type 2 UE.

In future big CR we can keep existing TS 38.101-1 Table 7.10A.2-1 and TS 38.101-3 Table 7.10B.3-1 and update NOTE 4 to point to Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R to capture UE Type 4 power imbalance and reference sensitivity.

Proposal 1: In future big CR we can keep existing TS 38.101-1 Table 7.10A.2-1 and TS 38.101-3 Table 7.10B.3-1 and update NOTE 4 to point to Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R to capture UE Type 4 power imbalance and reference sensitivity.

# 3 	Summary

Observation 1: In current specification we use NOTE 4 in TS 38.101-1 Table 7.10A.2-1 and TS 38.101-3 Table 7.10B.3-1 to point to the corresponding reference sensitivity for two ports for type 2 UE.

Proposal 1: In future big CR we can keep existing TS 38.101-1 Table 7.10A.2-1 and TS 38.101-3 Table 7.10B.3-1 and update NOTE 4 to point to Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R to capture UE Type 4 power imbalance and reference sensitivity.

# References

[1] 		R4-2502924, WF on UE requirements for non-collocated scenario, Moderator (KDDI)