3GPP TSG-RAN WG4 Meeting # 114bis						        R4-2504207

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.24.1

Source: 	OPPO

Title: 	On system parameter

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the system parameter is provided in this paper.

- Discussion

During the RAN4#114 meeting, the agreed WF has been captured as below:

Issue 2-1-1: Operating band

Agreement:

- NR band n8 can be used as an example band, and other FDD bands below 1GHz are not precluded.

Issue 2-1-2: R2D transmission bandwidth definition

Agreement:

- Reuse the following definition of R2D transmission bandwidth in TR38.769

- Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D.

- RAN4 will re-visit the two agreed bullets above, if RAN1 has the different conclusion.

Issue 2-1-4: Values of R2D bandwidth

Agreement:

- Define R2D transmission bandwidth configuration:

  - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS

  - FFS on other RB numbers

- Further discuss the R2D channel bandwidth considering additional guard band.

- If RAN1 has the agreement different than above, RAN4 will revisit the agreements

Issue 2-1-5: R2D channel raster and channel spacing

Agreement:

- Do not define channel spacing for R2D transmission

- Define channel raster for R2D transmission.

  - Details need further discussion.

Issue 2-1-6: D2R bandwidth

Agreement:

- D2R channel bandwidth for A-IoT device

  - Define D2R channel bandwidth for A-IoT device considering the following aspects:

    - [1SB or] 2SB

    - D2R maximum data rate (depending on RAN1 conclusion)

    - SFO

    - FDMA, i.e., considering the different device 1 has different frequency shift

    - Reference Filter performance

    - Other aspects are not precluded

2.1 R2D Channel bandwidth

Firstly, for the R2D channel bandwidth, it has been agreed that 1 to 4 PRB with 15kHz SCS to be defined as the R2D transmission bandwidth configuration. Further guard band can be discussed. From our understanding, similar to the NB-IOT channel bandwidth definition, which 200kHz has been defined for 1 PRB as channel bandwidth. For the guard band definition, initially it has been agreed that this guard band will be used for co-existence solution. However, from the deployment scenario perspective, it has been agreed that only legacy UE outdoor will be considered in Rel-19. In such case, the co-existence problem has been solved. In such case, for 1 PRB, reuse 200kHz as channel bandwidth can be a starting point.

Proposal 1: Use 200kHz as channel bandwidth for 1 PRB D2R channel bandwidth.

Furthermore, for the larger number of 1 PRB cases, there can be two solutions. Firstly, to use the same spectrum utilization for different PRB numbers. That is to say, for 1 PRB, 200kHz channel bandwidth is defined, while for 2 PRB, 400kHz channel bandwidth is defined and so on. This is quite straightforward to maintain the 90% SU and leave large enough guard band for different number of PRBs.

Another solution is to keep the 10kHz guard band for 1 PRB for larger number of PRBs. In such case, the SU will be larger when larger number of PRBs are applied. However, this will also need to be considered together with the largest number of PRBs that to be used to transmit R2D and to see how large the guard band will be needed in that case. From RAN1 design perspective, they have not limited the largest number of R2D PRBs which has been captured as meeting agreement in the last RAN1 meeting as captured below:

Agreement

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

- Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

Considering current example band n8 is used for D2R and with 15kHz SCS, n8 has 35MHz channel bandwidth with as large as 188 PRBs and corresponding guard band is set as 572.5kHz. However, considering the energy consumption of device 1, it seems not possible that device 1 to work under such large bandwidth for receiving. Hence it is proposed to use 12 PRBs as the largest CBW at least for device 1. For further work on other device types, larger CBW can be introduced.

Proposal 2: To introduce 12 PRBs as the largest CBW and the number of PRBS will be 1,2,3…12.

In such case, going back to the guard band definition. For the scaling method which guard band is linearly scaled with PRBs, 240kHz is used for guard band and 120kHz at each side. If we use 20kHz as fixed value, the spectrum utilization can be up to (12*180)/(12*180+20)= 99%. Compared to 3MHz and 5MHz whose SU is 94.75% and 94.85%. In such case to maintain the 20kHz guard band seems not a very appropriate method. Hence we prefer to choose the scaling method whose guard band is defined as 20kHz*(#of PRBs).

Proposal 3: It is proposed to define the guard band as 20kHz*(#of PRBs).

Besides the above discussion, considering also RAN1 agreement as captured above that: “Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.” This means for the exact transmission bandwidth, 15kHz SCS can be further applied to the transmission channel bandwidth, this can be similar method as NR that configured RBs are defined but the exact RBs used for transmission can be configured. And for A-IOT case, the configured RBs are defined but the exact RBs + the Sub-carriers used for R2D transmission can be further configured, in such case, the below channel bandwidth definition is proposed:

Figure 1 A-IOT R2D channel bandwidth

Proposal 4: Define the A-IOT R2D channel bandwidth as figure 1.

2.2 D2R Channel bandwidth

For the D2R channel bandwidth, in the last RAN1 meeting, 2SB has been agreed. For the small frequency shift, it has been discussed in RAN1 and the conclusion is captured as below:

Agreement

For D2R, only 2SB (Double sideband) modulation is supported.

Agreement

For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length):

- the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation

- the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

In such case, from our understanding, the frequency shift is used for FDM transmission of D2R. Even though with the frequency shift, in time domain it is the same transmission data repetition. In such case, there is no higher data rate reached since the data are the same for repetition. So the frequency shift should not be counted for the D2R channel bandwidth and only the transmission bandwidth of D2R should be counted. This is further illustrated in below figure 2.

Observation 1: There is no higher data rate reached since the data are the same for repetition for small frequency shift.

Figure 2 D2R channel bandwidth

The green part show as above for D2R channel bandwidth is defined. From simulation perspective, 15kHz is used for BWD2R, and in our view, this 15kHz can be reused for device 1.

Proposal 5: Define 15kHz as D2R channel bandwidth.

The frequency shift should be used for FDM D2R transmission and should be decoupled from channel bandwidth definition of D2R. In our view, the frequency shift is more like a channel raster for D2R transmission, for example, fixed raster point for D2R transmission with fixed frequency shift. However, this might need to be further discussed in RAN1 and we can wait for the frequency shift defined.

Proposal 6: Wait for RAN1 to define the frequency shift and it can be defined as channel raster for D2R.

Besides the D2R channel raster, for the R2D channel raster, it has been agreed that to define the channel raster for R2D, however, how to define it has not been agreed. As currently discussed in the deployment scenario part, the in-band and stand-alone scenario has been clarified that only Reader R2D will be deployed and no NR will be transmitted in the same A-IOT reader base station. In such case, the stand-alone scenario is enough. Hence, to reuse the NR channel raster is the most straightforward and easy way to do so.

Proposal 7: To reuse the NR channel raster for R2D.

3	Conclusions

In this paper, we give discussion on the system parameter, the observations and proposals are as below:

Observation 1: There is no higher data rate reached since the data are the same for repetition for small frequency shift.

Proposal 1: Use 200kHz as channel bandwidth for 1 PRB D2R channel bandwidth.

Proposal 2: To introduce 12 PRBs as the largest CBW and the number of PRBS will be 1,2,3…12.

Proposal 3: It is proposed to define the guard band as 20kHz*(#of PRBs).

Figure 1 A-IOT R2D channel bandwidth

Proposal 4: Define the A-IOT R2D channel bandwidth as figure 1.

Figure 1 A-IOT R2D channel bandwidth

Proposal 5: Define 15kHz as D2R channel bandwidth.

Proposal 6: Wait for RAN1 to define the frequency shift and it can be defined as channel raster for D2R.

Proposal 7: To reuse the NR channel raster for R2D.

4 References

[1] R4-2502858	WF on requirements for A-IoT device, CMCC