3GPP TSG-RAN WG4 Meeting # 114bis						         R4-2504210

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.2.1

Source: 	OPPO

Title: 	UE RF requirements for intra-band non-contiguous CA

Document for:	Approval

- Introduction

During the last RAN4#114 meeting, the A-MPR simulation has been discussed with companies provided initial simulation results. In this meeting, we give further simulation results to finish the requirement.

- Discussion

During the last meeting discussion, the agreement on PC3 A-MPR simulation is as following. Hence some further simulation can be carried out based on the agreement.

Issue 2-1-1: PC3 A-MPR

Agreement:

- More stringent MPR requirement apply when one of the CC is at the edge channel.

- GAP has no influence on the MPR requirement.

Based on the simulation assumption, below configuration has been used to develop the A-MPR requirement.

Table 1 A-MPR simulation configuration

| case | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | CBW1 | CBW2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 0 | 10 | 42 | 15 | 10 | 10 |
| 2 | 10 | 18 | 10 | 22 | 15 | 10 | 10 |
| 3 | 10 | 19 | 10 | 22 | 15 | 10 | 10 |
| 4 | 10 | 27 | 10 | 14 | 15 | 10 | 10 |
| 5 | 10 | 33 | 10 | 7 | 15 | 10 | 10 |
| 6 | 10 | 34 | 10 | 7 | 15 | 10 | 10 |
| 7 | 10 | 41 | 10 | 0 | 15 | 10 | 10 |
| 8 | 25 | 0 | 25 | 27 | 15 | 10 | 10 |
| 9 | 25 | 19 | 25 | 8 | 15 | 10 | 10 |
| 10 | 25 | 20 | 25 | 8 | 15 | 10 | 10 |
| 11 | 25 | 26 | 25 | 1 | 15 | 10 | 10 |
| 12 | 30 | 0 | 30 | 22 | 15 | 10 | 10 |
| 13 | 30 | 19 | 30 | 3 | 15 | 10 | 10 |
| 14 | 30 | 19 | 30 | 2 | 15 | 10 | 10 |
| 15 | 30 | 21 | 30 | 1 | 15 | 10 | 10 |
| 16 | 50 | 1 | 50 | 0 | 15 | 10 | 10 |

As the major requirement for the A-MPR is the ASE requirement which needs -30dBm/MHz at the band edge. This will be quite stringent to the UE. Hence, we have simulated the first CC at 5860 channel and 5870 channel to see the difference. For the gap, we have applied the 10MHz. For the upper edge of the band, it should be quite the same as the band lower edge and hence the simulation is omitted.

The results are shown as below:

Table 2 Simulation result for A-MPR

| case | A-MPR 5860MHz | A-MPR 5870MHz | B |
| --- | --- | --- | --- |
| 1 | 14.3 | 11.3 | 3 |
| 2 | 12.6 | 9.5 | 3 |
| 3 | 13.1 | 10.2 | 3 |
| 4 | 13.2 | 10.4 | 3 |
| 5 | 12.7 | 9.8 | 3 |
| 6 | 12.4 | 9.8 | 3 |
| 7 | 12.3 | 9.8 | 3 |
| 8 | 12.5 | 9.7 | 7.5 |
| 9 | 14.8 | 12.1 | 7.5 |
| 10 | 16.5 | 13.4 | 7.5 |
| 11 | 12.8 | 9.6 | 7.5 |
| 12 | 12.9 | 9.6 | 9 |
| 13 | 11.5 | 8.8 | 9 |
| 14 | 11.5 | 8.8 | 9 |
| 15 | 12.3 | 9.2 | 9 |
| 16 | 13.7 | 10.4 | 15 |

Above table 1 is the simulation result for 10+10MHz as NC CA with the first CC at 5860MHz and 5870MHz respectively. Also, PC3 with 2PA 1LO architecture has been used.

Proposal: To take the above simulation results into consideration of A-MPR requirement for PC3.

3	Conclusions

In this paper, we give further simulation results based on the previous meeting agreement, the observations and proposals are as below:

Proposal: To take the above simulation results into consideration of A-MPR requirement for PC3.

4 References