3GPP TSG-RAN WG4 Meeting #114bis	R4-2504215

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface Phase 2

Agenda item:	7.20.1

Document for:	Discussion

Introduction

In RAN#107, “Revised SID for NR_AIML_air_Phase2” [1] was agreed. In this revised SID, following scope was agreed for RAN4:

- Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In this paper, we share our views on the following topics:

- LCM-related aspects for two-sided model case

Discussion

Life-Cycle Management related aspects

Functionality-based LCM aspects of UE-sided use cases have been discussed in the different WGs for several meetings. In RAN2#127, definitions of supported, applicable, and activated functionalities were agreed upon. These definitions are used in the 5-step LCM procedure defined in RAN2#127 for UE-sided beam management.

Agreements on definitions from RAN2#127: - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refer to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference Agreements on procedures from RAN2#127: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig. - Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: - Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

Among these LCM procedures, model identification has been discussed by RAN1 for several meetings. This was also reflected in the FS_NR_AIML_air_Ph2 SID that was revised in RAN#107 ‎[1], where the following objective was agreed for RAN2/RAN1:

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]

We quote below the agreements related to model identification from several RAN1 meetings.

Agreement​ from RAN1#116: - To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases.​ - To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases ​ - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s)​ - MI-Option 2: Model identification with dataset transfer​ - MI-Option 3: Model identification in model transfer from NW to UE​ - FFS: The boundary of the options​ Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose​ Note: other options are not precluded The other options are proposed for model identification type B by companies during the discussion:​ - MI-Option 4: Model identification via standardization of reference models. (for CSI compression)​ - MI-Option 5: Model identification via model monitoring Agreement​ from RAN1#118bis: Regarding the study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, ID (denoted as ID-X) can be transmitted from network/network-side to UE/UE-side for the dataset. ​ - Note: The notation “ID-X” is used for discussion purpose Agreement​ from RAN1#119: For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, ​ - ID-X can be used for pairing the UE-part and the NW-part of a two-sided model ​ - FFS: other information needed for pairing

Furthermore, model selection for two-sided CSI compression was already discussed in TR 38.843. Model pairing process and information was proposed to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB.

TR 38.843 Section 5.1: In CSI compression using two-sided model use case, in order to select a CSI generation model compatible with the CSI reconstruction model used by the gNB, the following aspect has been proposed: - Pairing information can be established based on model identification TR 38.843 Section 7.1.2: In CSI compression using two-sided model use case, feasibility and procedure to align the information that enables the UE to select a CSI generation model(s) compatible with the CSI reconstruction model(s) used by the gNB is studied. At least the following options have been proposed by companies to define the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB: - Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use. - Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use. - Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID. - Option 4: The pairing information is in the forms of by the dataset ID during type 3 sequential training. - Option 5: The pairing information is in the forms of a training session ID to a prior training session (e.g., API) between NW and UE. - Option 6: The pairing information is up to UE/NW offline co-engineering alignment, transparent to 3GPP specification. - Note: the disclosure of the vendor information during the model pairing procedure and model identification procedure should be considered. - Note: If each UE side model is compatible with all NW side model, the information is not needed for the UE. - Note: Above does not imply there is a need for a central entity for defining/storing/maintaining the IDs.

On the issue of pairing ID, RAN1 reached the following agreement in RAN1#120:

Agreement​ In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then​ - The same ID can be used for UE to collect UE-side target CSI for UE-side training ​ - The same ID can be used for applicability inquiry and reporting​ - The same ID can be used for inference configuration​ - FFS: whether ID/even same ID is needed for monitoring configuration​ - FFS: where the ID is assigned​ - Note: whether the purpose for pair will be specified will be discussed separately.

From RAN4 side, the ongoing discussions about functionality-based LCM in UE-sided use cases rely heavily on the progress made by RAN2/RAN1. We believe that RAN4 should also follow the progress made by RAN2/RAN1 regarding model ID-based LCM in the two-sided CSI compression use case.

RAN4 to wait RAN2/RAN1 progress on LCM definitions and procedures for the two-sided CSI compression use case.

In RAN4#111 meeting, various candidates of LCM procedures were considered for defining RRM requirements based on the specific use cases.

Agreement from RAN4#111: RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidates are: - Identification - Selection - Activation - Deactivation - Switching - Fallback to non-AI operation - Performance monitoring - Others are not precluded

Since RRM requirements may not be necessary for all above listed candidates, RAN4 should prioritize the following candidates with a high impact on RRM requirements:

- Activation

- Deactivation

- Switching (If considered by other WGs)

- Fallback to non-AI operation, and

- Performance monitoring.

For 2-sided model case, RAN4 to define RRM requirements for LCM, prioritizing Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring.

Conclusion

In this paper, we provide our views on LCM-related aspects for the two-sided model case. The following Observations and Proposals were made:

Proposal 1: RAN4 to wait RAN2/RAN1 progress on LCM definitions and procedures for the two-sided CSI compression use case.

Proposal 2: For 2-sided model case, RAN4 to define RRM requirements for LCM, prioritizing Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring.

- References

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.