# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504235

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.2

Source: 	OPPO

Title: 		CSI reporting requirement and testing framework for CSI prediction

Document for:	Approval

- Discussion

For CSI prediction, following agreements have been made in RAN4#113 meeting,

Channel model for CSI prediction tests

Agreement:

- For channel model for CSI prediction tests, TDL channel models will be used.

  - FFS on whether CDL channel model is needed for generalization test.

Reference throughput

Agreement:

- reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook

Testing conditions

Agreement:

- use static conditions as baseline.

  - FFS on whether to have non-static condition

According to our understanding, throughput should be utilized to evaluate the model inference performance and existed RAN4 test examples for “reporting of PMI” can serve as a reference, e.g. as captured in 38.101-4, “The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively.”

In terms of the relationship to legacy requirements, as a baseline test, what we define is the minimum performance requirement, which is not to verify how much performance gain an AI/ML function/model can bring. The assessment of performance gain should be considered when RAN1 introduces and evaluates CSI features, rather than when RAN4 defines test cases. Furthermore, under current test conditions, e.g., by TDL channel, even if we define a higher performance testing requirement, it still has little significance for the practical application of AI/ML solutions. Therefore, we can just reuse the legacy PMI requirement (γ) as a baseline test, and let other options/proposals with higher performance requirements be further studied.

The requirements for AI/ML based CSI prediction(or AI/ML based PMI prediction) could be specified in terms of the ratio:

γAI/ML=tue_AI/MLtrnd

In the definition of γAI/ML,  [e.g., for 32TX PMI requirements, FFS other TX values], tue is 90 % of the maximum throughput obtained at [a given SNR, or a SNR range ] using the precoders configured according to the UE reports, and trnd is the throughput measured at [a given SNR, or a SNR range ] with random precoding.

- Note1: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance, γAI/ML could be calculated for each predicted CSI and then do the average within the CSI prediction window, or tue_AI/ML and trnd could be calculated within the CSI prediction window first and then get the averaged γAI/ML value.

- Note2: 32TX PMI could be the baseline, FFS other values.

- Note3: TDL channel could be the baseline, FFS other channel conditions.

- Note4: whether tests should be conducted at a given SNR or a SNR range should be further considered.

Proposal 1: The requirements for AI/ML based CSI prediction(or AI/ML based PMI prediction) could be specified in terms of the ratio:

γAI/ML=tue_AI/MLtrnd

In the definition of γAI/ML,  [e.g., for 32TX PMI requirements, FFS other TX values], tue is 90 % of the maximum throughput obtained at [a given SNR, or a SNR range ] using the precoders configured according to the UE reports, and trnd is the throughput measured at [a given SNR, or a SNR range ] with random precoding.

Proposal 2: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 3: 32TX PMI could be the baseline, FFS other values.

Proposal 4: TDL channel could be the baseline, FFS other channel conditions.

Proposal 5: Whether tests should be conducted at a given SNR or a SNR range should be further considered.

Figure 1 Reference block diagram for 1-sided model

Figure 1 provides the reference block diagram for 1-sided model as agreed previously. For CSI prediction, following steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

Proposal 6: For CSI prediction, following test steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

In addition, it should be noted that if the test is limited only to the legacy approach (e.g., TDL channel), it is challenging to assert that the performance test outcomes can reflect the effectiveness of the tested model. This is because for a data-driven AI/ML algorithm, the TDL channel model is too simple to mimic. Even if a model achieves good test results on the TDL channel, it is still hard to say that it can work on UMa and CDL channels, let alone in complex field channels. Therefore, besides the TDL channel based tests(TDL could be a baseline test), CSI-related tests may be conducted under CDL channel [or other more practical channel conditions] to check a generalized performance.

Regarding the static/non-static scenarios/conditions for testing, first to consider testing cases in static scenarios and configurations is a good choice. After having some feasible testing cases for static configurations, RAN4 can further consider and evaluate whether to introduce non-static testing scenarios and configurations.

Proposal 7: Besides the TDL channel based tests(TDL could be a baseline test), CSI-related tests may be conducted under CDL channel [or other more practical channel conditions] to check a generalized performance.

Proposal 8: After having testing cases for static configurations, RAN4 can further consider and evaluate whether to introduce non-static testing scenarios and configurations.

Regarding the reference model for CSI prediction, the following agreements were reached during the RAN4#114 meeting:

Reference model for CSI prediction

Agreement:

Assume no reference AI model for CSI prediction as baseline

- If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Besides, the "Way Forward on Simulation Assumptions for CSI Prediction" was agreed in RAN4#114 to proceed with CSI evaluation, including the following steps:

Step 1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE)

Step 2: check SGCS (or other metrics, e.g. TPs) after CSI feedback

Our understanding is RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

    - If RAN4 achieves aligned CSI prediction evaluation results in Step 1 (as described in the agreed simulation assumptions), further evaluation (e.g., Step 2 comparisons) can proceed.

    - If alignment in Step 1 proves difficult, RAN4 should identify the key factors causing discrepancies and initiate a second-round evaluation to reassess Step 1 alignment (e.g., before/during RAN4#115).

    - If alignment remains unachievable after the second round, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (e.g., after RAN4#115).

Proposal 9: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

Proposal 10: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction,

  - If RAN4 achieves aligned CSI prediction evaluation results in Step 1 (as described in the agreed simulation assumptions), further evaluation (e.g., Step 2 comparisons) can proceed.

  - If alignment in Step 1 proves difficult, RAN4 should identify the key factors causing discrepancies and initiate a second-round evaluation to reassess Step 1 alignment (e.g., before/during RAN4#115).

  - If alignment remains unachievable after the second round, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (e.g., after RAN4#115).

For  the training and test data consistency, following agreements have been made in RAN4#114 meeting:

Agreement:

- further discuss the following options

  - Option 1: Consistency between training data and test data should be ensured

    - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test

  - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

According to our understanding, the consistency between training data and test data should be ensured. The only difference between option1 and option2 is: Option 1 aims to determine whether additional information/indication is needed to explicitly guarantee the consistency. Option 2 assumes that the current test environment is sufficiently stable and capable of reproducing test results. Thus, for both of them, ensuring consistency between training and test data is a prerequisite. The unclear parts are 1) whether the test emulated channel is reproducible, 2) whether additional information/specifications are required,.

Proposal 11: Consistency between training data and test data should be ensured

    - FFS: Whether the test emulated channel will be similar to the one used in RAN4 simulations

    - FFS: Whether anything special should be done during the test

Regarding the simulation for AI/ML-based CSI prediction, we conducted the Step 1 evaluation for the CSI prediction using the simulation assumptions(highlighted in yellow in Table 1). The simulation results are shown in Table 2.

Table 1: Baseline Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback overhead | Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Option4: PC7 Note: this part relates to step2, may be down selected later |
| Baseline | Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements) |
| KPI | Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |

Table 2: Simulation results for AI/ML based CSI prediction

| Doppler spread | SGCS | NMSE |
| --- | --- | --- |
| 20Hz | 0.9994 | 0.0001 |
| 50Hz | 0.9032 | 0.0335 |

Observation 1: Regarding the simulation for AI/ML-based CSI prediction, we conducted the Step 1 evaluation for the CSI prediction using the simulation assumptions(highlighted in yellow in Table 1). The simulation results are shown as follows:

| Doppler spread | SGCS | NMSE |
| --- | --- | --- |
| 20Hz | 0.9994 | 0.0001 |
| 50Hz | 0.9032 | 0.0335 |

- Conclusions

In this contribution, we have discussed the CSI reporting requirement and testing framework for CSI prediction, and got the following proposals:

Proposal 1: The requirements for AI/ML based CSI prediction(or AI/ML based PMI prediction) could be specified in terms of the ratio:

γAI/ML=tue_AI/MLtrnd

In the definition of γAI/ML,  [e.g., for 32TX PMI requirements, FFS other TX values], tue is 90 % of the maximum throughput obtained at [a given SNR, or a SNR range ] using the precoders configured according to the UE reports, and trnd is the throughput measured at [a given SNR, or a SNR range ] with random precoding.

Proposal 2: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 3: 32TX PMI could be the baseline, FFS other values.

Proposal 4: TDL channel could be the baseline, FFS other channel conditions.

Proposal 5: whether tests should be conducted at a given SNR or a SNR range should be further considered.

Proposal 6: For CSI prediction, following test steps could be considered:

  - Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

  - Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

  - Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

  - Step4: DUT reports the predicted CSI (CSI reporting)

  - Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

Proposal 7: Besides the TDL channel based tests(TDL could be a baseline test), CSI-related tests may be conducted under CDL channel [or other more practical channel conditions] to check a relatively generalized performance.

Proposal 8: After having testing cases for static configurations, RAN4 can further consider and evaluate whether to introduce non-static testing scenarios and configurations.

Proposal 9: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

Proposal 10: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction,

  - If RAN4 achieves aligned CSI prediction evaluation results in Step 1 (as described in the agreed simulation assumptions), further evaluation (e.g., Step 2 comparisons) can proceed.

  - If alignment in Step 1 proves difficult, RAN4 should identify the key factors causing discrepancies and initiate a second-round evaluation to reassess Step 1 alignment (e.g., before/during RAN4#115).

  - If alignment remains unachievable after the second round, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (e.g., after RAN4#115).

Proposal 11: Consistency between training data and test data should be ensured

    - FFS: Whether the test emulated channel will be similar to the one used in RAN4 simulations

    - FFS: Whether anything special should be done during the test

Observation 1: Regarding the simulation for AI/ML-based CSI prediction, we conducted the Step 1 evaluation for the CSI prediction using the simulation assumptions(highlighted in yellow in Table 1). The simulation results are shown in Table 2:

Table 1: Baseline Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback overhead | Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Option4: PC7 Note: this part relates to step2, may be down selected later |
| Baseline | Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements) |
| KPI | Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |

Table 2: Simulation results for AI/ML based CSI prediction

| Doppler spread | SGCS | NMSE |
| --- | --- | --- |
| 20Hz | 0.9994 | 0.0001 |
| 50Hz | 0.9032 | 0.0335 |

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface