3GPP TSG-RAN WG4 Meeting #114bis	R4-2504243

# Wuhan, China, 7th–11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rx beam factor optimization of R19 RRM enhancements

Agenda Item:	7.17.2.1

Document for:	Discussion

- Introduction

During last RAN4 meeting, around the multi-Rx based Rx beam sweeping factor optimization, the following progress were achieved and summarized in [1] as below:

Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2. Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue. Issue 1-1-3: SSB processing delay/time for processing multiple beams received in a SMTC Agreement in RRM session (Monday): - No need to add additional processing time in the delay requirement for scenario 5. Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement Agreement: - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

In this paper, we provide our further views on the remaining issues of the reduced Rx beam sweeping factor on L3 measurements.

- Discussion

- Conditions to activate/deactivated L3 measurement delay reduction by optimizing Rx BSF

In last meeting, companies have warm discussion on the conditions of activation/deactivation of the fast L3 measurement on top of the approvements achieved in 113 meeting, it was decided not to introduce additional conditions beside the agreements from RAN4#113 meeting, on top of this, still some details need to determine. We provide our further analysis from the perspective of the following two sub-issues.

- Sub-issue 1: Whether to limit the serving cell of the FBS triggering condition to FR2 serving cell

To support the RRM measurement for the sake of mobility, normally the NW would configure multiple frequency layers to perform RRM measurement via the MO configuration, all of these MOs may be cross FR1 and FR2 or only within one FR. Under the CA/DC case, there are multiple serving cells within one FR or cross FR1 and FR2. When NW configures the threshold of the serving cell as the trigerring condition of FBS, it is necessary to indicate the applicable serving cell(s) of such threshold, so that UE only needs to evaluate whether to trigger the FBS by comparing the RSRP/RSRQ/SINR of the applicable serving cell(s) with the threshold. Normally the applicable serving cell(s) is in FR2 as the FBS is to enhance the FR2 Rx beam sweeping. But there is use case that for the UE capable of cross FR CA/DC, when the channel quality of FR1 serving cell get worse, then the handover or load balance to FR2 is urgent, it is necessary to accelerate the RRM measurement in FR2. So we prefer not to restrict the threshold must be applicable to the FR2 serving cell.

Proposal 1: Whether to limit the serving cell of the FBS triggering condition to FR2 serving cell, considering one use case is that the demand of handover or load balance to FR2 cell becomes urgent when then channel quality of FR1 serving cell get worse, prefer not to add such restriction.

Proposal 2: NW indicates the applicable serving cell(s) when configuring the threshold for FBS triggering.

- Sub-issue 2: Whether to add the hysteresis and timeToTrigger parameters

- Regarding how to configure the serving cell’s signal quality threshold, some companies proposed it could be similar as Event A2, so they raised the parameters hysteresis and timeToTrigger to avoid ping-pong. But no consensus achieved on whether to follow similar mechanism of Event A2.

- The IE Hysteresis is a parameter used within the entry and leave condition of an event triggered reporting condition. The actual value is field value * 0.5 dB.

- The IE timeToTrigger is to define the time during which specific criteria for the event needs to be met in order to trigger a measurement report.

- Actually here we just aim to define the exact enter and leaving condition for FBS, instead of any reporting. Anyway UE needs to perform RRM measurement, so we do not believe the necessity of avoidence of the ping-pang is very urgent. Similar as the mechanism through the legacy IE s-MeasureConfig to control whether to perform neighbour cell measurement, only RSRP threshold/range is configured, not any ping-pong avoidance mechanism was introduced. So here we suggest to follow similar logic to simplify the procedure, i.e. neither Hysteresis nor timeToTrigger to define besides the threshold.

Proposal 3: Similar as the mechanism through the legacy IE s-MeasureConfig to control whether to perform neighbour cell measurement, to simplify the procedure, neither Hysteresis nor timeToTrigger to define besides the threshold.

- UE capability of L3 measurement delay reduction by optimizing Rx BSF

During last meeting, it is determined that the UE capability of Rx BFD is needed, but the exact UE capability is FFS. In current spec, the Rx beam sweeping factor is melt into the whole scaling factor of M for the measurement requirements of FR2. So regarding how to reduce the Rx BSF, two solutions were proposed in last meeting:

Alt 1: UE reports the reduced N, such as N = 2, 4, 6

Alt 2: UE reports the reduced M, and M = sample*reduced Rx BFD, such as M = 12, 14, 16, 20

To our understanding, no matter defining reduced N or reduced M, there is no big difference, but considering this enhanced UE capability lies in the fast beam sweeping, it should has no impact on the sample number. Starting from this, the reporting of reduced N is preferred by us, which implies smaller Rx beam sweeping factor is assumed, no impact on the legacy assumption of sample. Considering in current FR2 measurement requirements, instead of N, the whole scaling factor M is captured in, so NW and UE should determine the corresponding reduced M based on the reduced N.

Proposal 4: Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6.

Another relevant issue is how to capture the measurement requirements impacted by the reduced N in spec. To align with our above proposal on the UE capability, in terms of how to capture the delay requirements, just replacing the original assumption of N = 8 with the reported UE capability is the most straightforward solution. For Scenario 1/2/3/4/5/7, update the the highlighted factor in the following table suggested in [3] could be as the baseline.

| Scenarios | Factors |
| --- | --- |
| 1 | Mass/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

Proposal 5: For the delay requirements of Scenario 1/2/3/4/5/7, replace the original beam sweeping factor with the reduced value reported by UE capability.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: Whether to limit the serving cell of the FBS triggering condition to FR2 serving cell, considering one use case is that the demand of handover or load balance to FR2 cell becomes urgent when then channel quality of FR1 serving cell get worse, prefer not to add such restriction.

Proposal 2: NW indicates the applicable serving cell(s) when configuring the threshold for FBS triggering.

Proposal 3: Similar as the mechanism through the legacy IE s-MeasureConfig to control whether to perform neighbour cell measurement, to simplify the procedure, neither Hysteresis nor timeToTrigger to define besides the threshold.

Proposal 4: Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6.

Proposal 5: For the delay requirements of Scenario 1/2/3/4/5/7, replace the original beam sweeping factor with the reduced value reported by UE capability.

- Reference

- R4-2502593 “WF on RRM requirements for NR_RRM_Ph5_Part2”, CATT

- TS 38.331

- R4-2418294 “Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor”, NTT DOCOMO, INC