3GPP TSG-RAN WG4 Meeting #114bis	R4-2504248

Wuhan, China, 7th – 11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of UE-initiated beam management of R19 NR MIMO Phase 5

Agenda Item:	7.22.3.1

Document for:	Discussion

- Introduction

Around the objective of UE-initiated/event-driven beam management, during last RAN4 meeting, the following agreements were achieved in last meeting and summarized in [1], we list them as below:

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)? Agreement: The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1. TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4 - TReport = 0 or be removed from the measurement period - N: Reuse the legacy definition - M: FFS pending RAN1 further agreement - TSSB (taking SSB as example) - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1 - Option A: Max{TSSB_current, TSSB_new} - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new. - Option C: others are not precluded Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM Agreement: - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM Issue 1-13: Whether to enhance beam sweeping in event triggered L1 beam reporting? Agreement: Reuse current beam sweeping factor for UE/event triggered beam reporting. Issue 1-16: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? Agreement: - The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

In this paper, we provide our detailed analysis around the UE-initiated/event-driven beam management.

- Discussion

Ever since Rel-15, NR has supported intra-cell beam management. During Rel-17, inter-cell beam management(ICBM) was studied and specified. So actually until now, both intra-cell and inter-cell beam management are allowed. While all the legacy intra-cell and inter-cell beam management are NW-initiated. In terms of the beam management resource configuration, periodic, semi-persistent and aperiodic measurement resource are candidates.With respect to the reporting, similarly, periodic, semi-persistent and aperiodic reporting are allowed with some principle defined between the mapping of resource and reporting. All types of reporting are NW-initiated, in other words, only NW could trigger the beam measurement results reporting. Consequently, even the UE detects no need to update the current beam based on the measurement results, once the reporting is configured or activated or indicated by NW, the UE has to report the quantities configured in the corresponding report configuration.

Actually the factors which would lead to beam switching including UE movement and rotation, all these factors are not controlled by NW, so only UE is aware of these. So no matter for the sake of beam management latency reduction or to save the reporting overhead, it seems the UE-initiated beam management is more efficient and effective.

2.1 Requirements impact

The requirements of measurement period

In last RAN4 meetings, warm discussion around the measurement period for Event 2 was held and finally some initial progress was approved, but one critical issue of how to define the TSSB, which is still suspending. Regarding the relationship of periodicities between current beam and candidate beams, no further progress was approved in RAN1. But RAN1 excluded the cross-CC beam reporting case as below in latest RAN1#120 meeting.

So the follow observation can be achieved:

Observation 1: The current beam RS and the new beam RS(s) must be in the same CC, i.e. only single-CC beam reporting is allowed by RAN1. For the single-CC beam reporting, only same periodicity case is approved by RAN1.

Proposal 1: Based on RAN1 progress, RAN4 only need to define measurement period requirements for the same periodicity case between current beam and candidate new beam(s). The agreements in last meeting for same periodicity case is enough, no need further discussion.

Furthermore, besides Event-2, additional Event-1 and Event-7 are also approved by RAN1. RAN4 needs to define measurement period requirements for all the three types of event.

- For Event-1, since only current beam involved in, so the measurement period could be derived from simplifying the measurement period for Event-2, that is, ignoring the periodicity of new beam in the evaluation period for Event-2. While as our analysis above, only same periodicity case approved until now by RAN1, so actually the measurement period requirements of Event-1 is same as Event-2.

Proposal 2: For Event-1, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1.

- For Event-7, the basic Case 1 is one-shot measurement based reporting, since UE needs to choose the Q-th best TCI state from the active TCI state list and set this Q-th best TCI state as reference to determine the new beam, so actually the periodicity of all beam within the active TCI state list and the periodicity of new beam are involved in the event detection. For the optional Case 2, we believe some additional clarification from RAN1 is needed, i.e. whether the Q-th best TCI state should be consistent or not during the event detection. If the Q-th best TCI state should be consistent, then as long as the Q-th best TCI state is determined at the 1st shot, then UE only needs to monitor this Q-th best beam and the new beam until the counter condition and timer condition meet, no need to monitor the beam of other TCI states. But if the Q-th best TCI state could be different beam at each measurement shot, then UE always need to monitor all beams of the active TCI state list at each measurement shot and pick the current Q-th beam.

Proposal 3: For Event-7, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1.

For Event 2, RAN1 has defined two types of trigger-event detection for reporting as below in [2][3][4]:

- Case 1: Basic solution(mandatory), once the L1-RSRP of the new beam becomes a threshold value better than the current beam(i.e. Event-2), UE initiated beam report occurs.

- Case 2: Optional solution(subject to UE capability), f within a time window, the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

- For the basic solution, which is one-shot measurement based reporting, while for the optional solution, the sample number M is configured by NW signalling, and a timer X should also be considered. Other scaling factor such as collusion with other measurement or Rx beam sweeping in FR2 should also be considered.

Proposal 4: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. For the optional Case 2, the sample number M and timer X should be considered.

The requirements of event triggered measurement reporting delay

The critical point for how to define the measurement reporting delay is whether to involve the latency of Tfirst UL channel during last meeting discussion. Actually in legacy event triggered L3 report, RAN4 did not define any equation to capture the measurement reporting delay. Instead, only the upper bound is specified as the cell identification delay as below.

9.2.4.3 Event Triggered Reporting ... The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. - The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

So for the event triggered L1 reporting, we suggest to apply similar solution, i.e. only specify the upper bound of reporting delay as TL1_measure_period+Tfirst UL channel. Why need to add the component of Tfirst UL channel, that is because during the event triggered L1 reporting, anyway UE needs to send 1st PUCCH to request UL resource or to inform NW the subsequent reporting, this 1st PUCCH is always needed regardless of Mode A or Mode B.

Proposal 5: Reuse the style of legacy L3 measurement reporting delay to specify the L1 measurement reporting delay, i.e. the event triggered L1 measurement reporting delay shall be less than TL1_measure_period + Tfirst UL channel.

The requirements of TCI state switching

RAN1 held some discussion on the reduction of TCI state switching latency after UEIBR in RAN1#119 meeting and the following progress were achieved:

Even no final conclusion between the 3 alternatives, it can be seen that the common point of all alternatives lies in: activate new beam(s) without additional SSB reception. From RAN4 perspective, which implies the target TCI state is of course known and maintained by UE, so the components of TL1-RSRP and Tfirst-SSB+TSSB-proc in legacy TCI state switching procedure could be ignored after confirmation/acknowledgement from NW or after receiving a TCI state activation command or after sending a UE-initiated beam report.

Proposal 6: Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

2.2 CSI-RS based measurement

In previous meeting, it is approved to define CSI-RS based UEIBM L1-RSRP measurements at least for periodic CSI-RS. This aligns with the RAN1 progress, i.e. periodic CSI-RS is applicable to UEIBM. But it should be noted that different from SSB, the CSI-RS based beam management was not specially discussed for the inter-cell mTRP scenario in legacy R16/17/18. While the scope of this UEIBM has beed determined as including both intra-cell and inter-cell cases. So if applying CSI-RS based measurement into the inter-cell case, from the perspective of beam management, is there any potential issue should be addressed or clarified, still FFS. So it is better to be more cautious on the inter-cell CSI-RS based UEIBM.

After some review on the legacy R17 ICBM especially around the RAN1 background, now we believe the reason of why no discussion developed on CSI-RS based ICBM, is that the intra-cell/inter-cell CSI-RS configurations are actually transparent to UE as no PCI used for RS sequence screambling. So from RAN1 view, they believe no difference between intra-cell/inter-cell CSI-RS based beam management, no need to specially discuss CSI-RS based ICBM, that is why only SSB based ICBM was discussed in R17 ICBM. So it seems applying CSI-RS measurement in intra-cell and inter-cell mTRP are applicable.

However, during the discussion in R17 ICBM, the following points around SSB based L1-RSRP measurement were mainly identified and concrete solutions were approved:

- The requirements are only defined for a NSC who meets the known condition. The exact known condition is defined in [6] as below:

- Agreements: - Known condition for a cell with different PCI on which L1-RSRP measurement is performed. - A cell with different PCI is considered as known provided - SSBs from serving cell and the cell with different PCI for L1-RSRP measurement has the same center frequency, SCS and SFN offset; - During the last [5]s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with different PCI; - Timing offset between serving cell and the cell with different PCI are within CP. - A cell with different PCI is considered as unknown provided the known condition is not meet.

- NW shall configure L3 measurement on a NSC before L1-RSRP measurement is configured for UE on that cell.

- Sharing factor are introduced when any symbol of the SSBs from serving cell and NSC are overlapping or adjacent (in time domain).

- Applicable for both inter-cell BM and inter-cell mTRP scenarios.

- Referring to CSI-RS based L1-RSRP measurement, all these points should be further check. In our view, since unable to distinguish intra-cell/inter-cell CSI-RS configuration, 3) could be ignored. 4) is just the applicable scenario clarification, no impact on CSI-RS based. For 1) and 2), they should be clarified in CSI-RS based UEIBM.

Proposal 7: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary:

-  Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

-  The prerequisite on L3 measurement on NSC.

Besides the above clarification, for CSI-RS based UEIBM, the measurement reporting delay and evaluation period/measurement period are similar as SSB based. It seems no need to additional discuss, just extend the SSB based requirements to CSI-RS based.

Proposal 8: For CSI-RS based UEIBM, RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: The current beam RS and the new beam RS(s) must be in the same CC, i.e. only single-CC beam reporting is allowed by RAN1. For the single-CC beam reporting, only same periodicity case is approved by RAN1.

Proposal 1: Based on RAN1 progress, RAN4 only need to define measurement period requirements for the same periodicity case between current beam and candidate new beam(s). The agreements in last meeting for same periodicity case is enough, no need further discussion.

Proposal 2: For Event-1, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1.

Proposal 3: For Event-7, the measurement period requirements is same as Event-2 as only same periodicity case approved until now by RAN1.

Proposal 4: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. For the optional Case 2, the sample number M and timer X should be considered.

Proposal 5: Reuse the style of legacy L3 measurement reporting delay to specify the L1 measurement reporting delay, i.e. the event triggered L1 measurement reporting delay shall be less than TL1_measure_period + Tfirst UL channel.

Proposal 6: Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

Proposal 7: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary:

-  Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

-  The prerequisite on L3 measurement on NSC.

Proposal 8: For CSI-RS based UEIBM, RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Reference

- R4-2502595, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung, RAN4#114

- Final_Minutes_report_RAN1#116_v100

- Final_Minutes_report_RAN1#116b_v100

- Final_Minutes_report_RAN1#117_v020

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, RAN1#119

- R4-2207108, WF on FeMIMO inter-cell beam management, Samsung, RAN4#102e