3GPP TSG-RAN WG4 Meeting #114bis	R4-2504250

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on UE-to-UE CLI of R19 SBFD

Agenda Item:	7.23.3.1

Document for:	Discussion

- Introduction

In last meeting, good discussion were held and the following progress were achieved in [1] around UE-to-UE CLI as below:

- Sub-topic 1-1: General issues #### Issue 1-1-1: Measurement methods - Agreements (from Mon AH session) - RAN4 to define the CLI measurement requirements for Method #3 (UE measures RSSI within UL subband) Issue 1-1-3: Measurement resources - Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types. Issue 1-1-4: Measurement reporting - Agreements - Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1. - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the aperiodic L1 based CLI measurement reporting. #### Issue 1-1-5: Scheduling restriction - Agreements (from Thu AH session) - For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition. - For L1-CLI-RSSI Methods #1, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2. - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed. - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS Sub-topic 1-2: L1-SRS-RSRP Issue 1-2-1: Side conditions - Agreements (from online session) - For SRS configuration, reuse R16 assumptions on SRS configuration for L1 SRS-RSRP measurements - Agreements - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition. - For SRS Es/Iot, further discuss the following options - Option 1: 1dB - Option 2: > 1dB - For SRS BW, further discuss the following options - Option 1: 24RB - Option 2: 48RB Issue 1-2-2: Measurement period - Agreements (from online session) - RAN4 to define one set of accuracy requirements based 1 sample. - Send LS to RAN1/2 on whether the NW configuration of timeRestrictionForChannelMeasurement or a similar configuration will apply for L1 SRS-RSRP and/or CLI-RSSI measurement. - From RAN4 requirement perspective: if it applies, RAN4 to define two sets of requirements for L1 SRS-RSRP measurement delay. - Agreements - For aperiodic resources, measurement period is based on 1 sample. Sub-topic 1-2: L1-CLI-RSSI Issue 1-3-1: Measurement timing for Method #3 - Agreements (from online session) - Base on DL reference timing

It can be seen that good progress were achieved in last meeting. In this paper, we provide our further views on the remaining issues of UE-to-UE CLI based on RAN1 progress.

- Discussion

- Until now, RAN1 has developed discussion on the measurement resource configuration, UE-to-UE CLI reporting configuration and resource determination. It is obvious that the legacy CSI framework was determined to reuse as much as possible. On top of these RAN1 progress, from RAN4 perspective, we analysis the suspending part of UE-to-UE CLI.

- Rx beam

- In previous RAN4 meetings, it was confirmed that no need to perform Rx beam sweeping for L1 CLI measurement, and the exact assumption of Rx beam has beem determined by RAN1 progress. In RAN1#118bis meeting, the case of a-periodic measurement resource was concluded as to reuse the legacy a-periodic CSI-RS resource beam assumption, i.e. to follow the list of TCI state(s) in CSI-AssociatedReportConfigInfo if any or to follow the unifiedTCI-StateType if configured, otherwise to assume the Rx beam as one of the latest received PDSCH and the latest monitored CORESET. In latest RAN1#119 meeting, for the case of periodic/semi-persistent measurement resource, the following agreements were achieved:

Agreement(119) For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

- So it can be summarized that the Rx beam assumption for all P/SP/AP CLI resource were determined. I.e reuse the typeD QCL solution in CSI framework. Furthermore, the R16 CLI Rx beam determination is deemed as the default solution.

- Regarding how to capture the Rx beam assumption, to avoid missing any detail, it is convenient to directly refer to RAN1 spec, instead of repeating the whole details in RAN4 spec. Furthermore, in last RAN4 meeting, it was proposed by some company that the method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period. We believe it is natural since no Rx beam sweeping touched.

- Proposal 1: Regarding how to capture the Rx beam assumption, it is suggested to directly refer to RAN1 spec instead of repeating all details in RAN spec.

- Measurement resource

- In last meeting, RAN4 approved to define measurement requirements for wideband reporting for both RSRP and RSSI, and all the P/SP/AP types of resources are considered. One suspending issue is whether UE could measure dual DL subbands simultaneously in case of DUD and the potential impact on measurement period and accuracy if any. Take the approved three methods into consideration, only RSSI is allowed to measure in DL subband, so here we only focus on the RSSI measurement. Not any sequence involved in the RSSI measurement. So for the DUD case, UE needs to measurement CLI-RSSI resource simultaneously in the dual DL subbands if it is configured by NW.

- Regarding whether there is potential impact on the accuracy, to our understanding, similar as the condition defined for legacy CLI-RSSI accuracy, as long as the conditions for reference sensitivity are fulfilled, no impact on the accuracy.

- Proposal 2: Within the approved three CLI methods, only CLI-RSSI measurement is allowed in DL subbands. For the case of DUD configuration, UE needs to measurement CLI-RSSI resource simultaneously in the dual DL subbands if it is configured by NW. Since no sequence involved in, we can not see additional challenge for DUD case.

- Proposal 3: Regarding the potential impact on the accuracy, similar as the condition defined in legacy CLI-RSSI accuracy, as long as the conditions for reference sensitivity are fulfilled, no impact on the accuracy.

- Side condition

- For the side condition for the accuracy of LI-SRS-RSRP measurement, the timing offset, SRS Es/Iot and SRS BW are still open.

- It is concluded to further discuss the timing offset during performance part. For the SRS Es/Iot, whether follow legacy 1 dB or some higher value to apply, companies are still diverse during last meeting. For the SRS BW, besides the legacy 48 PRBs, 24 PRBs is also raised to accomodate the small UL subband case especially for DUD configuration.

- To our understanding, both the SRS Es/Iot and the SRS BW could be determined based on the evaluation of accuracy requirements.

- Proposal 4: Decide the side condition of SRS Es/Iot and SRS BW based on the evaluation of accuracy requirements.

- Measurement reporting

- Based on latest RAN1 progress, besides the aperiodic reporting, periodic L1 CLI-RSSI reporting is approved additionally. And RAN1 concluded the whole picture of reporting as below:

- For L1 CLI-RSSI reporting, both periodic and aperiodic reporting are supported;

- For L1 CLI-RSRP reporting, only aperiodic reporting is allowed.

- As a result, RAN4 needs to define measurement reporting requirements for P/AP CLI-RSSI reporting, and define measurement reporting requirements for AP CLI-RSRP reporting.

- Another suspending issue is whether timeRestrictionForChannelMeasurement similar as that in legacy L1 measurement would be allowed in L1 CLI-RSRP measurement and/or L1 CLI-RSSI measurement, we need to wait for RAN1 LS reply.

Proposal 5: Based on RAN1 latest progress, RAN4 needs to define measuremetn reporting requirements for periodic CLI-RSSI reporting.

- Scheduling restriction

- Around scheduling restriction, the strategy for L1-SRS RSRP measurement and L1-CLI RSSI Method#1 have been determined. The case of L1-CLI RSSI Method#3 is still suspending. In last meeting, it is approved that the DL reference timing is assumed for Method#3 measurement, i.e. apply same measurement timing assumption as Method#1. So we believe same scheduling restriction strategy as Method#1 could be applied.

Proposal 6: The scheduling restriction for L1-CLI RSSI Method#3 is:

-  For UL, scheduling restrictions always apply, for both FR1 and FR2.

-  For DL, scheduling restriction do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2.

-  The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed.

- Requirement applicability

- In legacy R16 CLI, the CLI-RSRP/CLI-RSSI resource has the following applicability rule defined in clause 9.7.3 in [2]:

9.7.1 Introduction The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements requirements in this clause apply for TDD mode only. CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP 9.7.2 SRS-RSRP measurements 9.7.2.1 Introduction When configured by the network, the UE shall be able to perform SRS-RSRP measurements of configured srs-ResourceConfigCLI. The requirements apply when the subcarrier spacing for SRS-RSRP measurement resource configuration is the same as the subcarrier spacing of the active DL BWP of serving cell. The UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier. 9.7.3 CLI-RSSI measurements 9.7.3.1 Introduction When configured by the network, the UE shall be able to perform CLI-RSSI measurement of configured rssi-ResourceConfigCLI. The subcarrier spacing for CLI-RSSI measurement resource configuration can be same or different from the subcarrier spacing of active BWP. UE shall perform CLI-RSSI measurement with the SCS of the active BWP.

- Similar as legacy R16 CLI, we believe the follow applicability rules should be followed for the UE-to-UE CLI:

- For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP.

- For CLI-RSSI resource

- For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP.

- For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration.

- No gap is needed for UE-to-UE CLI measurement

- Proposal 7: The following applicability rules are specified for UE-to-UE CLI measurement:

- For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP.

- For CLI-RSSI resource

- For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP.

- For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration.

- No gap is needed for UE-to-UE CLI measurement

- The Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

- Proposal 1: Regarding how to capture the Rx beam assumption, it is suggested to directly refer to RAN1 spec instead of repeating all details in RAN spec.

- Proposal 2: Within the approved three CLI methods, only CLI-RSSI measurement is allowed in DL subbands. For the case of DUD configuration, UE needs to measurement CLI-RSSI resource simultaneously in the dual DL subbands if it is configured by NW. Since no sequence involved in, we can not see additional challenge for DUD case.

- Proposal 3: Regarding the potential impact on the accuracy, similar as the condition defined in legacy CLI-RSSI accuracy, as long as the conditions for reference sensitivity are fulfilled, no impact on the accuracy.

- Proposal 4: Decide the side condition of SRS Es/Iot and SRS BW based on the evaluation of accuracy requirements.

Proposal 5: Based on RAN1 latest progress, RAN4 needs to define measuremetn reporting requirements for periodic CLI-RSSI reporting.

Proposal 6: The scheduling restriction for L1-CLI RSSI Method#3 is:

-  For UL, scheduling restrictions always apply, for both FR1 and FR2.

-  For DL, scheduling restriction do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2.

-  The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed.

- Proposal 7: The following applicability rules are specified for UE-to-UE CLI measurement:

- For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP.

- For CLI-RSSI resource

- For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP.

- For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration.

- No gap is needed for UE-to-UE CLI measurement

- Reference

- R4-2502597, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#114, 2025.

- TS 38.133-v18.8.0